The Windsor Boys School -  D Block Roof Replacement

Full replacement of D Block Roof – Quotes required by 12 noon 19th May 2025. All works to be guaranteed for 30 years.

| Description | Unit Price |
| --- | --- |
| Preliminaries: Health & Safety documentation, Welfare, Site compound, Skips. |  |
| Roof 1 - 800m2 Scope of works: - Strip existing tiles, felt & batten, renew with premium treated BS5534 – compliant batten and install new high-quality breathable membrane. - Renew tiles with Redland Concrete Regent tiles. - Renew ridge tiles with Redland Concrete half-round ridge tiles and dry ridge kit. - Install new eaves trays, fascia vents and comb fillers. - Clear all waste and debris from site. - Scaffolding |  |
| Roof 2 – 920m2 Scope of works: - Strip existing tiles, felt & batten, Renew with premium treated BS5534- compliant batten and install new high-quality breathable membrane. - Renew tiles with Redland Concrete Regent tiles. - Renew ridge/hip tiles with Redland Concrete half-round ridge/hip tiles and dry ridge kit. - Remove existing verge & install new dry verge unit with end caps. - Remove existing valley & install new dry valley trough. - Install new eaves trays, fascia vents & comb fillers. - Replace all lead flashing as required. - Clear all waste & debris from site. - Scaffolding - Remove existing Velux rooflights and install 8no PK08 & 3no FPK08 Velus units with associated flashings. |  |
| Roof 3 – 520m2 Scope of works: - Strip existing tiles, felt & batten, Renew with Premium treated BS5534- compliant batten and install new high-quality breathable membrane. - Renew tiles with Redland Concrete Regent tiles. - Renew ridge/hip tiles with Redland Concrete half round ridge/hip tiles and dry ridge kit. - Remove existing verge & install new dry verge unit with end caps, - Install new eaves trays, fascia vents & comb fillers. - Clear all waste and debris from site. - Scaffolding |  |
| 170 x metal box gutters |  |