Specification for the annual maintenance of all grass pitches

1.Mowing

- April the 1st until October the 31st

- All pitches should be mowed weekly,(weather dependant)

- A grass height of between 25 and 35mm should be achieved.

- Cuttings are to be evenly spread around the pitches.

2. Fertiliser application

- 9:7:7 @ 350kg/ha in April

- 12:6:6 to be supplied in Summer, do not apply in drought conditions or to compliment renovation works

- 14:4:14 (with at least 50% slow release N) @ 350kg/ha in mid-late August

3. Vertidraining and dressing

- The pitches should be vertidrained to about 150mm depth in the spring

- 60 tonnes of top dressing with a 70/30 sand to soil ratio to be applied to each pitch

4. Overseeding

- The pitches are to be overseeded once per year.

5. Goal areas

- These are in a poor condidtion and will need additional work. This will be priced separately by the contractor that wins he contract.