Checklist Principal Designer’s CDM Design Risk Register

This list includes certain key hazards/activities that need to be identified/addressed at the outset of design to ensure the health and safety of those carrying out or affected by construction activities or using a structure designed as a workplace. Applicable items must be identified, and the form completed to set out how hazards are to be eliminated and risks reduced. The list is not exhaustive and must be supplemented by project-specific hazard identification and design risk assessment/s. The list also includes some features that can be incorporated within the design to improve health and safety. This is to be a live document and updated periodically by the Principal Designer AND the Design Team over the duration of the design phase. This document will provide a record against which the developing design can be monitored, and records key health and safety decisions made which affect health and safety.

Hazard Elimination and Risk Reduction: All hazards, including those normal to the type of work, should be eliminated, and/or remaining risks reduced by altering the design so far as reasonably practicable (i.e. unless, when compared to the hazard/risk, it is grossly disproportionate in terms of time, cost and effort to do so) and taking account of other relevant design considerations (e.g. cost, fit for purpose, aesthetics, buildability, maintainability and environmental impact). Reduce overall risk by reducing likelihood of harm (injury or adverse health effect); potential severity of harm, number of people exposed to the harm; and frequency or duration of exposure to harm. Information Needed by Others: Information other designers or contractors are likely to need to identify and manage remaining risks. A remaining risk is ‘significant’ only if it is not likely to be obvious to a competent contractor, unusual or likely to be difficult to manage effectively. Do not include information or generic risks or site controls. Further Guidance: Construction (Design & Management) Regulations 2015, (and HSE guidance document - L153) - http://www.hse.gov.uk/pubns/books/l153.htm

Key of Abbreviations:  Ops – Client Ops; PD – Principal Designer; PM – Project Manager; Arch – Architect; S.Engr – Structural Engineer; M. Eng – Mechanical Engineer; E.Eng – Electrical Engineer; PC – Principal Contractor.

Ctor – Contractor; CPP – Construction Phase Plan; RAMS – Risk Assessment and Method Statements; TMP – Traffic Management Plan; TFFRA – Timber Frame Fire Risk Assessment

| CHECKLIST PRINCIPAL DESIGNER - CDM RISK REGISTER |  |  | Doc Ref/Rev: 1 |
| --- | --- | --- | --- |
|  |  |  | Issue Date: 260219 |
| Project: Isles of Scilly - Belonging and Inclusion hub | Package or Element: - RIBA Design stage 5(Man & Const) |  | Reviewed By: D. Hill |
| Project/Design Phase: | Prepared By: Danielle Hill |  | Authorised By: D. Hill |
| Design Function/Organisation | Address: The Apex, Venaspace, Derriford Business Park, Brest Road, Plymouth, PL6 5FL | Contact: | Email: |
| Principal Designer: Collaton Safety | Danielle Hill | 01752 936344 | dhill@collatonsafety.co.uk |
| Design Team | Significant risk identified following the consultant design freeze. |  |  |
| Architect: | To comment on initial draft and to be completed after completion – significant risks |  |  |
| Structural Engineer: | To comment on initial draft and to be completed after completion – significant risks |  |  |
| M/E Engineers: | To comment on initial draft and to be completed after completion – significant risks |  |  |
| Landscape designers: | To comment on initial draft and to be completed after completion – significant risks |  |  |

Predicted Risk & Residual Risk Level Descriptions - Red / Amber / Green Status

Predicted Risk

Red:	Significant risk/s identified. Action must be taken by the design risk owner to reduce the risk/s to ALARP.

Amber:	Medium risk/s identified. Action must be taken by the design risk owner to reduce the risk/s to ALARP, with information being provided wherever necessary to manage the remaining risk/s.

Green:	Low risk. No significant risks, difficult to manage or unusual risks identified which a competent contractor would not be expected to be aware of.

Residual Risk

Red: 	No red items allowed. Red items must be designed out during the design stage.

Amber:	Medium risk/s identified during future operation and/or maintenance. Identify where information needed by others is to be provided (such as SHE Box on drawings, PCI document, H&S File) and, where applicable, cross reference to relevant document/drawing.

Green:	Low risk. No significant risks, difficult to manage or unusual risks identified which a competent contractor would not be expected to be aware of.

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 1 | Select the position and design of structures to avoid or minimise risks from site hazards. |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Traffic movements and access to and from and within the site, including the need for reversing |  | Establish traffic movements of delivery and refuse vehicles, also consider emergency vehicle access. TMP to be included with CPP. | PD, PM, Arch, PC |  | ASAP | TMP in place around site and between remote compound to include emergency access | PC, PM, PD |  | TMP/A&M strategy expected place ref no in column L |  |
| 1.2 | Inadequate space on site for delivery and/or storage of materials or welfare facilities |  | Acquisition of separate area for contractor`s storage / welfare | PD, C, PC |  | Prestart | Plan at pre-start stage if space or area is allowed for deliveries or laydown area. If no area available look at out of hours vehicle delivery or off-site storage. | PD, C, PC |  | Place plan or veh circuit in CPHSP. Consider off-site parking or site. Otherwise stagger deliveries. Details in column L |  |
| 1.3 | Unauthorised access to hazardous construction site areas. |  | PD and PC to ensure sufficient segregation of site works from public areas. Details to be included in CPP. | PD, PM, Ops, PC |  | ASAP | Erection of hoarding/ heras fencing. Road and pedestrian path closures to prevent access. | PC, PM, PD |  | Place plan and designs for hoarding in column L |  |
| 1.4 | Inadequate separation of site and public areas or concurrent activities e.g. nearby house, offices, schools etc. Review of planning conditions. |  | Works involving noisy operations are to be undertaken out of hours, or at times of least inconvenience. Installation of suitable screens, plant/equipment in good working order. Liaison with Client operators. | PD, PM, Pc, Ctor |  | ASAP | Ongoing review of procedures. | PM, Pc, Ctor |  | Consider systems procedures that do not impact on public, consider working hours. Details to be placed in column L |  |
| 1.5 | Asbestos (if present or unknown, early survey required) |  | For all projects, an R&D Asbestos survey is required. | PC, PD, C, |  | Due Dill | Ensure that no work takes place until an Asbestos survey is supplied. | PC, PD, C |  | Asbestos survey to be completed, and report placed in CPHSP, report ref to be placed in column L |  |
| 1.6 | Presence/storage of hazardous materials e.g. fuel storage (LPG, propane etc) |  | Suitable storage of Hazardous materials required. Consider HSG168 and a suitable FRA for the site. Encapsulate in a locker, cupboard and consider in FRA. | C, PD, PC |  | Pre start | A suitable FRA must be considered and implemented before work starts on site. | C, PD, PC |  | Ensure guidance is followed regarding placement of any gas cages. Consider on fire plan especially as site is in Residential area. Cages, correct methods to be used. Details in CPHSP and refer to doc in column L. |  |
| 1.7 | Buried (or concealed) site services Inc. gas pipelines, electrical apparatus, oil pipelines etc (if present or unknown, early survey required) |  | Establish if services require protection or diversion for the works. Liaise with appropriate statutory authorities. | S. Eng, Arch, PM, PD, PC |  | Due Dill stage | Completed at early site stage checks and surveys. | S. Eng, Arch, PM, PD, PC |  | Consider due diligence, Utility checks must be considered otherwise client may suffer delays, service strikes. Hand dig to uncover known services. PD to request and place in PCP enclosures file. Plan or maps to be referenced in Column L. |  |
| 1.8 | Contaminated ground e.g. by remediation, capper layer, or by minimising disturbance – using shallow excavations |  | Ground report completed | S. Eng, PM, Arch, PD, PC |  | Due Dill stage | Ground investigation will establish any form of contamination on the ground that is to be dug or excavated. | S. Eng, PM, PD, PC. |  | A Ground SI report must be strongly considered as contaminates must be established before site start. PD to request and place report in encloses section of PCP. Ref plan in column L. |  |
| 1.9 | Previous site use risks e.g. mining history, unexploded ordnance risk (consider high level search of area) |  | To complete | S. Eng, PM, PD, PC |  | Due Dill stage | Ground investigation or Topo report will usually pick up historical mining activities. PD to use experience and area knowledge to discuss report and findings with PC/Client. | S. Eng, PM, PD, PC |  | Report to be read and understood by Engr so that the correct ground structure can be established and suggested. Report ref no to be placed and description to be placed in column L. |  |
| 1.10 | Unsafe existing structures (if present or unknown, early survey required) |  | To complete | S. Eng, PM, PD, PC |  | Due Dill Stage | If the project is designed with a form of structural element involved, e.g. demo, load bearing walls removed, retaining wall constructed, excavations etc a structural engineer must comment on plans in a written email or even compile a report. | S. Eng, PM, PD, PC |  | Structural report or comment referenced in column L |  |
| 1.11 | Lack of information re. existing site/structure/ground conditions (early survey required) |  | To complete | PM, S.Eng, PD |  | Due Dill stage | A sound SI report will supply information that will be able to assist the structural engineer. The report should suggest methods of retaining walls and foundations that work in line with the soil type and formation. | PM, S.Engr, PD |  | Ensure that a SI report is attached to the enclosures section of the PCP and ref no attached in column L of this report. |  |
| 1.12 | Confined Spaces |  | To complete | PM, Arch, P, PC |  |  | Every confined space | PM, Arch, P, PC |  | Confined spaces must be identified on a site plan and only trained personnel allowed to enter areas. Ensure PTW used for tasks and held on site, process and RA ref to be identified and placed in column L |  |
| 1.13 | Dismantling/ Demolition Works |  | All water supplies to be isolated before disconnection of pumps where large pumps need to be removed they will be dismantled on site and removed. Structural survey for buildings to be demolished required. 1 Feb no demolition contract issued at this time | S. Eng, PM, PD, PC |  |  | Competent persons carrying out works, all water will be isolated. Dismantling process for pipe works in place for pumps | S. Eng, PM, PD, PC |  | PC to control but follow guidelines - - avoid entry to confined spaces, e.g. by doing the work from the outside, - if entry to a confined space is unavoidable, follow a safe system of work; and put in place adequate emergency arrangements before the work starts. Detail site conf spaces on a site plan and induct staff to PTW and safe systems. Ref plan in column L. |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 2 | Can the project be designed to minimise health hazards? |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Avoid use of organic solvent-based paints/thinners and isocyanates, particularly for use in confined areas. Avoid on-site spraying or other application of harmful substances i.e. as defined by COSHH and current EH40 e.g. paints and retardants etc. |  | Specify use of water-based paints, ensure adequate ventilation provided, review CPP and RAMS. Use of alternate substances less hazardous to health, create a preparation area external to building with natural ventilation. | PD, PC, Ctor |  | By SOS as per CPP | Only use water-based paints | PC |  | Method statement and product procurement to be considered by Arch or PC. Comment on procedure in CPHSP ref in column L |  |
| 2.2 | Avoid processes that create hazardous fumes, vapours, dust, noise or vibration, e.g. disturbing existing asbestos, chasing, scrabbling, blast cleaning, hand-dig tunnelling, flame cutting, sanding of areas of lead paint or cadmium coatings etc. |  | PC to manage sub-contract trades and ensure LEV, dust suppression, forced/natural ventilation is provided – all included within RAMS. | PD, PC, Ctor |  |  | Ensure adequate controls in place for cutting pipe works and cable runs, this includes noise, vibration, and shards. Adequate ventilation for welding is required. | PC, PM |  | Risk and Method statement in place with monitoring of task. Comment on procedure in CPHSP ref in column L |  |
| 2.3 | Avoid use of large/heavy materials, e.g. building blocks (including quoins) & kerbs > 20kg, large glazing panels, heavy lintels, full-size plasterboard panels etc. |  | PC to manage sub-contract trades, continually review material schedules to reduce risk of material handling. Provision of RAMS. | PD, Arch., PC, Ctor |  | RIBA stage 4 | Design completed heavy materials considered. Hierarchy of controls considered. | PC |  | If used ensure that it is mentioned in final design risk register as a significant risk. Comment on procedure in CPHSP ref in column L |  |
| 2.4 | Design block paved areas to enable mechanical handling and laying of blocks |  | Review of TMP in place, ensure off-loading can take place adjacent/near to point of installation | PD, Arch., PC, Ctor |  | RIBA stage 4 | Access & maintenance of vehicles TBC | Arch, PD, Ops. |  | Discussed on CDM workshop ref A&M strategy. Comment on procedure in CPHSP ref in column L |  |
| 2.5 | Specify off-site timber treatments, for timber preservation to inhibit insect infestation. |  | On delivery to site the material shall be classed inert with no safety/health hazard | PD, PC, Ctor |  | Pre start | Ensure that all timber treated is lifted into place with recognised Access equipment. Ensure there is space for equipment and structure can take system suggested. | PD, PC, Ctor |  | Any lifting must be pre-empted with a lift plan and the Ctor must submit RAMS. |  |
| 2.6 | Other project specific |  |  |  |  |  |  |  |  |  |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 3 | Can the project be designed to avoid the following safety hazards? |  |  |  |  |  |  |  |  |  |  |
| 3.2 | Need for work at height from ladders or where a safe means of access/working platform is not provided |  | Believe all areas can be accessed by MEWP | PD, PC, Ctor |  | CPP | Ensure that the PC understands the importance of the correct access equipment for the correct task. | PC, PD |  | Follow directions in CPP which should line with the WAH and CDM regs 2015. Comment on procedure in CPHSP ref in column L |  |
| 3.3 | ‘Trip hazards’ e.g. shallow steps/risers in external paved areas, ‘lip’ details on top of pre-cast stairs |  | Through compliance with Building Control, and the Equality Act. PC to flag potential issues so a solution can be found. | PM, Arch., PD, PC, Ctor |  | CPP | PC to ensure safety measures followed. | PM, PD, PC, Ctor |  | Site safety reports Should identify trip hazards and other site areas. Comment on issues in CPHSP in column L. |  |
| 3.4 | Other project specific |  |  |  |  |  |  |  |  |  |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 4 | Can pre-fabrication be used to minimise hazardous work or allow it to be carried out under more controlled conditions? |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Pre-assembly at ground level, e.g. roof trusses, structural steel, or plant, and subsequent craneage of finished/semi-finished components |  | To complete | PM, PD, S.Engr, Arch., PC, Ctor |  | CPP | Lift required for pre - fab positioning. Appointed person lifts plans. | PM, PD, PC, Ctor |  | Lift plans should be completed by an appointed person and listed in column L of this report. |  |
| 4.2 | Off-site cutting to size of materials and components |  | Create a preparation area, ideally external to the site area/building where natural ventilation is available. Alternately contractor provision of LEV-enabled equipment | PD, PC, Ctor |  | CPP | Ensure env conditions are considered such as, dust, noise, vibration etc. | PC |  | List significant risks in CPHSP and ref in column L of this report. |  |
| 4.3 | Early installation of permanent access, such as stairs/handrails |  | Ensure temporary works design considers for this to prevent falls from height. Engage sub-contractors for design input. | PD, PM S.Engr, PC, Ctor |  | CPP | Design assumption for edge protection to be considered ASAP. Design to be monitored during DTM`s. | PM, PD, PC, Ctor, S Engr |  | The design of the project must incorporate all edge protection and ensure it has been considered. A&M strategy to be considered for project. During construction stage consider slab edges or open edges and ensure there is some form of edge protection. Detail in CPHSP and refer in column L. |  |
| 4.4 | Permanent edge protection or other features that increase safety of access or construction, where there is a foreseeable risk of falling (e.g. scaffold, safety nets) |  | Pre-assembly of safety fittings / collective / passive measures (hierarchy of risk). Engage specialist sub-contractors for design input. | PD, S.Engr, PC, Ctor |  | Early design stage | Design at concept stage covering the A&M plan. To incorporate all edge protection or access routes. | PD, Arch, Client Ops |  | A&M strategy to be established at early stage and monitored throughout the life of the project. During construction stage consider slab edges or open edges and ensure there is some form of edge protection. Detail in CPHSP and refer in column L. |  |
| 4.5 | Specification of elevated steelwork specifically designed to accommodate systems to prevent falls from height, e.g. safety nets |  | Steelwork must ensure that it has retaining hocks or access for net clips. All nets must be erected before work starts. | PD, S.Engr, PC, Ctor |  | Early design stage | Design must incorporate the use of nets and allow for this style of fall arrest equipment. The access equipment during construction must be suitable. | PD, S.Engr, PC, Ctor |  | Struct Engr & PC must plan the use of access equipment and routes in/out for build and safety purposes. Detail plan in column L. |  |
| 4.6 | Specification of curtain walling with provision for tying scaffolds |  | Not to be tied, height can contain independent tied scaffold or mewps | PD, PC, Ctor, Client Ops |  |  | Designed with scaffold or tower for maintenance in mind. | PD, Arch |  | A&M strategy to consider cleaning maintenance and repair. Consider heights and access equipment. A&M strategy to be referred to in column L. |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 5 | Can the following features be designed in to simplify safe construction? |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Make allowance for temporary works required during construction |  | The site design must be considered for the appointment of a TWS or TWC at early concept design stage. | PD, S.Engr, PC, Ctor |  | Start on site | If the project contains temporary works then the PC is to employ a TWS and TWC to deal with all the works on site. | PD, S.Engr, PC, Ctor |  | Details of all TW and TW staff positions in the site directory. Temp wks plan and register to be held on site and reviewed regularly. Details of TWS/C to be placed in column L. |  |
| 5.3 | Design joints in vertical structural steel members for bolting up from a permanent floor and by seating angles to give support while bolts are put in place |  | To complete | PD, S.Engr, PC, Ctor |  | Struct Engr plan | Engr to ensure all aspects of safety are considered with steel works, access equipment, tools, etc. Report to highlight safety concerns and issues. | PD, S.Engr, PC, Ctor |  | All Engr and site staff to follow guidelines in plans Engr to sign off plans that ref struct work in column L. |  |
| 5.4 | Design connections to minimise risk of incorrect assembly |  | Off-site design during fabrication, include for lifting-eyes, marking of components onto a schedule. | PD, S.Engr, PC, Ctor |  | Prestart. | Steels to be identified before arriving at site, plans to be followed. | PD, S.Engr, PC, Ctor |  | To be identified in CPHSP and enforced on site. Detail and ref in column L |  |
| 5.5 | Avoid need for on-site welding, e.g. of steel structures, process plant, etc. |  | Where unavoidable, undertake hot-works in external preparation area designated by PC. Implement hot-works permit procedure accordingly. | PD, S.Engr, PC, Ctor |  | Prestart. | PC to consider method of works ref Hot works. Risk may dictate methods. Engage with Engr for connection methods. | PD, S.Engr, PC, Ctor |  | To be identified in CPHSP and enforced on site. Detail and ref in column L. |  |
| 5.6 | Other project specific |  |  |  |  |  |  |  |  |  |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 6 | Can the project be designed to avoid the following in-use safety and health hazards (operations environment i.e. staff and guests) |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Bad posture and manual handling of components, materials, and equipment. |  | Where building adaptations do not allow improvement, suggest provision of lifting equipment i.e. wheeled-trolley and lift hoist. | PC |  | CPP | PC to ensure correct procedures followed. | Client to ensure safety on site with site safety visits |  | CPHSP to mention methods and safety standards, site safety reports to ensure standards are kept high. |  |
| 6.2 | Slips and trips, e.g. using entrances, floors, ramps, stairs, and escalators, etc. including effect of rainwater and spillages |  | Interior design considerations for floor finishes/products, entry matting requirements. Also consider the cleaning/maintenance requirements | PM, Arch., PD, PC, Ctor |  | CPP | PC to ensure correct procedures followed. | Client to ensure safety on site |  | CPHSP to mention methods and safety standards, site safety reports to ensure standards are kept high. |  |
| 6.3 | Falls from height, e.g. unprotected edges at stairs, free-opening windows and adjacent to fragile surfaces, consider also external soft landscaping, site levels / uneven or soft ground |  | Consider how the building and its features shall be cleaned and maintained. Most sites considered water-fed pole cleaning of window units, but where site constraints dictate, specialist MEWP access (and/or towers) maybe required. | PD, Arch., PC, Ctor |  | CPP | PC to ensure correct procedures followed. | Client to ensure safety on site |  | CPHSP to mention methods and safety standards, site safety reports to ensure standards are kept high. |  |
| 6.4 | Traffic movements and moving plant & equipment, e.g. by segregation or separation |  | Review existing traffic arrangements for the site, if deemed necessary implement works to improve traffic/pedestrian interface. | PM, Ops, PD, Arch., PC, Ctor |  | CPP | PC to ensure correct procedures followed. | Client to ensure safety on site |  | CPHSP to mention methods and safety standards, site safety reports to ensure standards are kept high. |  |
| 6.5 | Hazardous systems/processes, e.g. electricity, hot water and steam, piped gases/liquids, hot/cold surfaces, protruding edges etc. |  | Design review by team | PM, Arch., PD, PC, Ctor |  | Prestart | M&E Engr to design systems that are safe and minimise danger. Plan and check locations of all new service risers and voids. | PM, Arch., PD, PC, Ctor |  | M&E design to be followed on site. SSOW to be followed, and to be detailed inn CPHSP and also in column L. |  |
| 6.6 | Abnormal events, e.g. fire, explosion, falling objects, disproportionate collapse, overturning, drowning and asphyxiation, crowding and malicious human intervention |  | Emergency event planning, to be consulted with Client Ops. | All |  | Prestart | HSG 168 to be followed and a safe fire Risk Ass and plan to be in place before work starts. Ensure the plan is live and grows with the site. | All |  | FRA and fire site plan to be compiled and referred to in column L. |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 7 | Can the project be designed to simplify future maintenance and cleaning work? |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Breakage of loft fire-compartmentation when installing MEP / data cabling |  | Specify loft hatches to each vertical compartment wall. Provide detailed drawing showing the hatch requirements. | PM, PD, Arch., PC, M. Eng, E.Eng, |  | Early design stage | Fire consultant/Arch to ensure correct fire compartmentation is designed and in place. Any access for maintenance must be planned at an early stage so after thought decisions are not made. Ensure fire stopping in place and any stopping listed. | PM, PD, Arch., PC, M. Eng, E.Eng, |  | Any access for maintenance must be planned at an early stage so after thought decisions are not made. Ensure fire stopping in place and any stopping listed and held on file for the O&M manual. Detail register and plans in column L. |  |
| 7.2 | Photovoltaic panels |  | Design and placement of panels must allow area or walkway around the roof area for access and maintenance. Placement of panel boxes must not impede any roof pop ups, plant etc. Allow 2 metres from edge of roof to panel. Arch assist with the placement of inverter and allow for power to ventilate room to avoid overheating. | PM, PD, Arch., PC, M. Eng, E.Eng, |  | Early design stage | The photovoltaic layout must be scrutinised by the PD and the inverter placement must be placed in an area that is ventilated or cooler than normal. Global warming is causing warmer summers and the systems overheat. Consider placement. | PM, PD, Arch., PC, M. Eng, E.Eng, |  | Early design stage compliance for all PV plans and systems is imperative to ensure safety in the A&M strategy. Drone suggested for inspection every 6 months. Detail A&M strategy in Column L. |  |
| 7.3 | Provide for safe/permanent access, e.g. to roof mounted/high level plant and services |  | Roof-hatch, fixed/folding handrails, and/or man safe design, early engagement of specialist contractors. | PM, PD, Arch., PC, M. Eng, E.Eng |  | comp | Client spec to be compiled and registered for the project. If completed it will be followed during the design phase, safety will be classed a priority. | PD, Arch |  | Addressed and designed on CDM workshop at the early stage of the project by PD. Detail A&M strategy in Column L. |  |
| 7.4 | Provide for practical and safe cleaning of windows, e.g. from the inside, and other normal maintenance and cleaning activities (smoke detectors, light fittings, etc) |  | Safe and simple methods must be adhered to and followed as per client spec. Pole to 7th floor and abseil or tilt and turn. No obstruction such as Brea solay to be designed unless specified by the client. | PM, PD, Arch., PC, M. Eng, E.Eng, Ctor |  | comp | Simple practical methods of cleaning will be suggested. | PD, Arch |  | Addressed and designed on CDM workshop at the early stage of the project by PD. Detail A&M strategy in Column L. |  |
| 7.5 | Design plant rooms to give adequate access and headroom to plant and equipment |  | Early engagement with MEP contractors to ascertain sizes and headroom of equipment. | PM, PD, Arch., PC, M. Eng, E.Eng, Ctor |  | comp | Plant room A&M strategy to be completed by M&E designer to include every item over 20kgs in weight. Routes in/out for maintenance and equipment used for maintenance to be itemised in strategy. Strategy to be forwarded to arch for inclusion in main A&M strategy. | PD, Arch |  | Addressed and designed on CDM workshop at the early stage of the project by PD. Detail A&M strategy in Column L. |  |
| 7.6 | Design plant room access to allow practical and safe removal/replacement of large/bulky plant |  | Early engagement with MEP contractors to ascertain sizes and headroom of equipment. | PM, PD, Arch., PC, M. Eng, E.Eng, Ctor |  | comp | Plant room a&M strategy to be completed by M&E designer to include every item over 20kgs in weight. Routes in/out for maintenance and equipment used for maintenance to be itemised in strategy. Strategy to be forwarded to arch for inclusion in main A&M strategy. | PD, Arch |  | Addressed and designed on CDM workshop at the early stage of the project by PD. Detail A&M strategy in Column L. |  |
| 7.7 | Avoid internal manholes in circulation areas and external manholes in heavy-use vehicle areas |  | To be consulted with Client Ops, check infrastructure plans and ensure correct weights for traffic routes | PM, PD, Arch., PC, M. Eng, E.Eng, Ctor |  | comp | Reduce internal inspection covers. | PD, Arch |  | Addressed and designed on CDM workshop at the early stage of the project by PD. Detail A&M strategy in Column L. |  |
| 7.8 | Make provision for safe temporary access to allow for painting/maintenance of facades/roofs etc. e.g. allow for access for MEWP’s or for erection of scaffolding |  | The A&M strategy must be considered at the earliest possible stage of the design, safety for the operational maintenance and access around the building must be considered. | PM, PD, Arch., S.Engr, PC, Ops. |  | comp | Addressed on CDM workshop, a suitable strategy is compiled in line with CDM requirements and clients preferred specification. | PM, PD, Arch., S.Engr, PC, Ops. |  | A&M strategy compiled in CDM workshop with Arch. Monitored throughout the project and handed as completed document to client. Detail A&M strategy in Column L. |  |

| Item No. | Hazard/Activity | Applicable? | Measures Taken by Designer to Eliminate Hazards or Reduce Risks | Design Risk Owner | Predicted Risk Level | By When | Information Needed by Others |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | - Critical design assumptions - Significant remaining risks - Suggested work methods/sequences and other control measures | Needed by Who | Residual Risk Level | Include in: SHE Box/Pre-Construction Info/H&S File/ Project Risk Register | Cross Ref. to Document/ Drawing |
| A | B | C | D | E | F | G | H | I | J | K | L |
| 8 | Can the structure be designed to avoid future dismantling/demolition hazards? |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Unusual stability concepts, e.g. pre-stressed structures, party walls |  | Unstable structures such as party wall on site. | PM, PD, Arch., S.Engr, PC |  | Pre start stage | Party wall is showing signs of leaning or possible failure. Structural engr to comment and then issue addressed to client as a risk to site and person(s). | PM, PD, Arch., S.Engr, PC |  | Issue to be addressed and identified on structural report and forwarded to Client for their appraisal. Refer in column L |  |
| 8.2 | Use modular construction/bolted connections/ identified ‘weak points’ |  | To complete | PM, PD, Arch., S.Engr, PC |  |  |  |  |  |  |  |

Danielle Hill

Principal Designer (CDM)

Collaton Safety Management Ltd