Re-roofing of Units 4 & 5 Highbury Road, Brandon

Re-roofing of 2 no. existing light industrial units

Preliminaries

Contents

A10 Project particulars	1

A11 Tender and contract documents	3

A12 The site/ existing buildings	4

A13 Description of the work	6

A20 JCT Minor Works Building Contract with contractor's design (MWD)	7

A30 Tendering/ subletting/ supply	11

A31 Provision, content and use of documents	13

A32 Management of the works	17

A33 Quality standards/ control	21

A34 Security/ safety/ protection	27

A35 Specific limitations on method/ sequence/ timing	31

A36 Facilities/ temporary work/ services	32

A37 Operation/ maintenance of the finished works	34

A40 Contractor's general cost items: management and staff	35

A41 Contractor's general cost items: site accommodation	36

A42 Contractor's general cost items: services and facilities	37

A43 Contractor's general cost items: mechanical plant	38

A44 Contractor's general cost items: temporary works	39

A50 Work/ products by/ on behalf of the employer	40

A53 Work by statutory authorities/ undertakers	41

A54 Provisional work/ items	42

A55 Dayworks	43

A10
Project particulars

Clauses

110 	The Project

- Name: Re-roofing of Units 4 & 5 Highbury Road

- Nature: Re-roofing and re-cladding

- Location: Units 4 & 5 Highbury Road, Brandon, Suffolk, IP27 0ND

- Timescale for construction work: 12 weeks

120 	Employer (client)

- Name: West Suffolk Council

- Address: West Suffolk House, Western Way, Bury St. Edmunds, Suffolk IP33 3YU

- Contact: West Suffolk Council

- Telephone: 01284 763233

- Email: customer.services@westsuffolk.gov.uk

130 	Principal Contractor (CDM/ Building Regulations)

- Duties: Fulfil all applicable duties that relate to the role of Principal Contractor for the purposes of the CDM Regulations and Building Regulations.

- Name: The Contractor

- Address: TBC

- Telephone: TBC

- Email: TBC

140 	Architect/ contract administrator

- Name: William Davis, West Suffolk Property Services

- Address: West Suffolk House, Western Way, Bury St. Edmunds, Suffolk IP33 3YU

- Telephone: 01284 757451

- Email: william.davis@westsuffolk.gov.uk

150 	Principal Designer (CDM/ Building Regulations)

- Duties: Fulfil all applicable duties that relate to the role of Principal Designer for the purposes of the CDM Regulations and Building Regulations.

- Name: William Davis, West Suffolk Property Services

- Address: West Suffolk House, Western Way, Bury St. Edmunds, Suffolk IP33 3YU

- Telephone: 01284 757451

- Email: william.davis@westsuffolk.gov.uk

200 	Consultants

- Description: Structural Engineer

- Name: Morrish and Partners

- Contact: Jonathan Parker

- Address: 85A Whiting Street, Bury St Edmunds, Suffolk, IP33 1NX

- Telephone: 01284 761444

- Email: jonathan@morrish.co.uk

200 	Consultants Type A

- Description: CDM Advisor

- Name: Ben Mulley

- Address: Unit 11, Brickworks Business Estate, Old Stowmarket Road, Woolpit, Suffolk IP30 9QS

- Telephone: 07706 149591

- Email: ben.mulley@dmasafety.net

Ω End of Section

A11
Tender and contract documents

Clauses

110 	Tender drawings

- The tender drawings are: As shown on the drawing issue sheet.

120 	Contract drawings

- The contract drawings: The same as the tender drawings.

160 	Pre-construction information

- Format: The pre-construction information is described in these Preliminaries in section A34. It refers to information given elsewhere in the Preliminaries, specification, drawings and associated documents.

Ω End of Section

A12
The site/ existing buildings

Clauses

110 	The site

- Description: Units 4 & 5 Highbury Road, Brandon, Suffolk, IP27 0ND

120 	Existing buildings on/ adjacent to the site

- Description: Light industrial units

140 	Existing utilities and services

- Drawings: (Information shown is indicative only): Refer to plans and DigDat/UKPN drawings.

160 	Soils and ground water

- Information: Included in the tender documents.

170 	Site investigation

- Report: Included in the tender documents.

200 	Access to the site

- Description: Off Highways

- Limitations: Access to adjoining units to be maintained at all times

- Access for inspections: Provide access at reasonable times for both on-site and off-site work.

210 	Parking

- Restrictions on parking of the Contractor’s and employees' vehicles: On site parking available

220 	Use of the site

- General: Do not use the site for any purpose other than carrying out the Works.

230 	Surrounding land/ building uses

- General: Adjacent or nearby uses or activities are as follows:

  - Light industrial units along Highbury Road.

240 	Health and safety hazards

- General: The nature and condition of the site/ building cannot be fully and certainly ascertained before it is opened up.

- Information: The accuracy and sufficiency of this information is not guaranteed. Ascertain if any additional information is required to ensure the safety of all persons and the works.

- Site staff: Draw to the attention of all personnel working on the site the nature of any possible contamination and the need to take appropriate precautionary measures.

250 	Site visit

- Assessment: Ascertain the nature of the site, access thereto and all local conditions and restrictions likely to affect the execution of the Works.

- Arrangements for visit: Contact William Davis
Property Services
West Suffolk Council
01284 757451
william.davis@westsuffolk.gov.uk

Ω End of Section

A13
Description of the work

Clauses

120 	The works

- Description: Removal of existing roof panels, roof cladding, associated rooflights and SW drainage (where shown on plans), to be replaced with new roof panels, cladding, rooflights and SW drainage

Ω End of Section

A20
JCT Minor Works Building Contract with contractor's design (MWD)

Clauses

JCT Minor Works Building Contract with contractor's design

- The Contract: JCT Minor Works Building Contract with contractor's design 2024 Edition.

- Requirement: Allow for the obligations, liabilities and services described.

Recitals

First - The Works and the Contract Administrator

- The work comprises: Removal of existing roof panels, roof cladding, associated rooflights and SW drainage (where shown on plans), to be replaced with new roof panels, cladding, rooflights and SW drainage

- Architect/ Contract Administrator: See clause A10/140.

Second - Contractor's designed portion

- The Works include the design and construction of: Electrical, Mechanical Ventilation and Drainage.

Third - Contract documents

- Contract drawings: As listed in clause A11/120.

- Contract documents: The following have been prepared which show and describe the work to be done A Specification.

Fourth - Priced documents

- Documents to be priced or provided by the Contractor: Schedule of Works

Articles

4 - Architect/ Contract Administrator

- Architect/ Contract Administrator: See clause A10/140.

5 - CDM Regulations - Principal Designer and Principal Contractor

- Principal Designer: See clause A10/150.

- Principal Contractor: See clause A10/130.

6 - Building Regulations - Principal Designer and Principal Contractor

- Principal Designer: See clause A10/150.

- Principal Contractor: See clause A10/130.

Contract Particulars

Fifth Recital and the JCT Fluctuations Option - Base Date

- Base Date: 22/09/2025

Fifth Recital and clause 4.2 - Construction industry scheme (CIS)

- Employer at Base Date is a 'contractor' for the purposes of the CIS.

Sixth Recital - CDM Regulations

- The project is notifiable.

Eighth Recital and Schedule 2 - Supplemental Provisions

- Health and safety: Supplemental Provision 1 applies.

- Cost savings and value improvements: Supplemental Provision 2 applies.

- Performance Indicators and monitoring: Supplemental Provision 3 does not apply.

- Sustainable development and environmental considerations: Supplemental Provision 4 applies

- Notification and negotiation of disputes: Supplemental Provision 6 applies. Where Supplemental Provision 6 applies, the respective nominees of the parties are:

- Employer's nominee: Rachael Mann, West Suffolk Council, Chief Operating Officer).

- Contractor's nominee: TO BE COMPLETED BY CONTRACTOR

Or such replacement as each party may notify to the other from time to time.

Article 8 - Arbitration

- Article 8 and Schedule 1 apply.

Clause 2.3 - Commencement and Completion

- Works commencement date: Monday 22nd September 2025

- Date for Completion: Friday 12th December 2025

Clause 2.9 - Liquidated damages

- At the rate of £695.00 per calendar week or pro-rata thereto.

Clause 2.11 - Rectification period

- Period: 12 months from the date of practical completion.

Clause 4.3 and 4.4 - Interim payments

- Interim Valuation Dates

  - The first Interim Valuation Date is: one month from commencement on site

  - Thereafter at intervals of: one month

- Payments due prior to practical completion

  - Percentage of total value of the work etc.: 95 per cent

- Payments becoming due on or after practical completion

  - Percentage of the total amount to be paid: 97½ per cent

Clause 4.9.1 - Supply of documentation for computation of amount to be finally certified

- Period: Three months from the date of practical completion.

Clause 5.3 - Contractor's Public Liability insurance - injury to persons or property

- The required level of cover for any one occurrence or series of occurrences arising out of one event

  - Not less than: £10 million

Clauses 5.4, 5.5 and 5.6 - Insurance of the works, etc. - alternative provisions

- Clause 5.4 applies.

- Where clause 5.4 or 5.5 applies, percentage to cover professional fees: 15 per cent

Clause 7.1 - Notification and negotiation of disputes

- The respective nominees of the Parties are

  - Employer's nominee: Rachael Mann, Chief Operating Officer, West Suffolk Council

  - Contractor's nominee: TO BE COMPLETED BY CONTRACTOR.

- Or such replacement as each Party may notify to the other from time to time

Clause 7.3 - Adjudication

- The Adjudicator is: President

- Nominating body: Royal Institute of British Architects

Schedule 1 paragraph 2.1 - Arbitration

- Appointor of Arbitrator (and of any replacement): President or a Vice-President of the: Royal Institute of British Architects.

Conditions - No Amendments

Section 1: Definitions and Interpretation

1.8 - Applicable law

- Amendments: England and Wales

Section 2: Carrying out the Works - No Amendments

Section 3: Control of the Works - No Amendments

Section 4: Payment - No Amendments

Section 5: Injury, Damage and Insurance - No Amendments

Section 6: Termination - No Amendments

Section 7: Settlement of Disputes - No Amendments

Execution

Execution

- The Contract: Will be executed as a deed.

Contract guarantee bond

Contract guarantee bond

- Contract guarantee bond: to be confirmed.

Ω End of Section

A30
Tendering/ subletting/ supply

Main contract tendering

110 	Scope

- General: These conditions are supplementary to those stated in the invitation to tender and on the form of tender.

145 	Tendering procedure

- General: In accordance with the principles of: NBS Guide to Tendering for Construction Projects.

- Arithmetical errors: Alternative 2 is to apply.

160 	Exclusions

- Inability to tender: Immediately inform if any parts of the work as defined in the tender documents cannot be tendered.

- Relevant parts of the work: Define those parts, stating reasons for the inability to tender.

170 	Acceptance of tender

- Acceptance: No guarantee is offered that any tender will be recommended for acceptance or be accepted, or that reasons for non acceptance will be given.

- Costs: No liability is accepted for any cost incurred in the preparation of any tender.

190 	Period of validity

- Period: After submission or lodgement, keep tender open for consideration (unless previously withdrawn) for not less than the period stated in the invitation to tender.

- Date for possession/ commencement: See section A20.

Pricing/ submission of documents

210 	Preliminaries in the specification

- The Preliminaries/ General conditions sections (A10-A56 inclusive) must not be relied on as complying with SMM7/ NRM2.

250 	Priced documents

- Alterations: Do not alter or qualify the priced documents without written consent. Tenders containing unauthorised alterations or qualifications may be rejected.

- Measurements: Where not stated, ascertain from the drawings.

- Deemed included: Costs relating to items, which are not priced, will be deemed to have been included elsewhere in the tender.

- Submit: With tender

310 	Tender

- General: Tenders must include for all work shown or described in the tender documents as a whole or clearly apparent as being necessary for the complete and proper execution of the Works.

480 	Programme

- Programme of work: Prepare a summary showing the sequence and timing of the principal parts of the Works and periods for planning and design. Itemize any work which is excluded.

- Submit: With tender.

510 	Alternative method tenders

- General: In addition to and at the same time as tendering for the Works as defined in the tender documents, alternative methods of construction/ installation may be submitted for consideration. Alternatives, which would involve significant changes to other work, may not be considered.

- Alternative tenders: Include a complete and precise statement of the effects on cost and programme.

- Safety method statement: Carry out a health and safety risk assessment for each alternative and where appropriate provide a safety method statement suitable for incorporation in the health and safety plan.

- Full technical data: Submit for each alternative together with details of any consequential amendments to the design and/ or construction of other parts of the Works.

- Submit: With tender.

515 	Alternative time tenders

- General: In addition to and at the same time as tendering based upon the date or period specified in section A20, an alternative tender based upon a different date for completion or period may be submitted.

- Date for Completion: If any such tender is accepted the Date for Completion inserted in the Contract will be the date stated in the alternative tender or determined from the period stated in the alternative tender.

530 	Substitute products

- Details: If products of different manufacture to those specified are proposed, submit details with the tender giving, reasons for each proposed substitution. Unless notification is given at tender stage, proposals for substitutions may not be considered.

- Compliance: Substitutions accepted will be subject to the verification requirements of clause A31/200.

Subletting/ supply - No Amendments

Ω End of Section

A31
Provision, content and use of documents

Definitions and interpretations

110 	Definitions

- Meaning: Terms, derived terms and synonyms used in the Preliminaries/ general conditions and specification are as stated here or in the appropriate referenced document.

120 	Communication

- Definition: Includes advise, inform, submit, give notice, instruct, agree, confirm, seek, provide or obtain information, consent or instructions, or make arrangements.

- Format: In writing to the person named in clause A10/140 unless specified otherwise.

- Response: Do not proceed until response has been received.

130 	Products

- Definition: Materials, both manufactured and naturally occurring, and goods, including components, equipment and accessories, intended for the permanent incorporation in the Works.

- Includes: Goods, plant, materials, site materials and things for incorporation into the Works.

135 	Site equipment

- Definition: Apparatus, appliances, machinery, vehicles or things of whatsoever nature required in or about the construction for the execution and completion of the Works but not materials or other things intended to form or forming part of the Permanent Works.

- Includes: Construction appliances, vehicles, consumables, tools, temporary works, scaffolding, cabins and other site facilities.

- Excludes: Products and equipment or anything intended to form or forming part of the permanent works.

140 	Drawings

- Definitions: To BSRIA BG 6 Design framework for building services.

- CAD data: In accordance with BS EN ISO 19650.

145 	Contractor's choice

- Meaning: Selection delegated to the Contractor, but liability to remain with the specifier.

150 	Contractor's Design

- Meaning: Design to be carried out or completed by the Contractor and supported by appropriate contractual arrangements, to correspond with specified requirements.

155 	Submit proposals

- Meaning: Submit information in response to specified requirements.

160 	Terms used in specification

- Remove: Disconnect, dismantle as necessary and take out the designated products or work and associated accessories, fixings, supports, linings and bedding materials. Dispose of unwanted materials. Excludes removal and disposal of associated pipework, wiring, ductwork or other services.

- Remediate: Action or measures taken to lessen, clean up, remove or mitigate the existence of hazardous materials; in accordance with standards, or requirements as may be set out by statutes, rules, regulations or specification.

- Fix: Receive, unload, handle, store, protect, place and fasten in position; dispose of waste and surplus packaging. To include all labour, materials and site equipment for that purpose.

- Supply and fix: As above, but including supply of products, components or systems to be fixed, together with everything necessary for their fixing. All products, components or systems are to be supplied and fixed unless stated otherwise.

- Keep for reuse: Do not damage designated products or work. Clean off bedding and jointing materials. Stack neatly, protect adequately and store until required by the employer/ purchaser, or until required for use in the works as instructed.

- Keep for recycling: As 'keep for reuse', but relates to a naturally occurring material rather than a manufactured product.

- Make good: Execute local remedial work to designated work. Make secure, sound and neat. Excludes redecoration and/ or replacement.

- Replace: Supply and fix new products matching those removed. Execute work to match original new state of that removed.

- Repair: Execute remedial work to restore something to its original working state. Make secure, sound and neat. Excludes redecoration and/ or replacement.

- Refix: Fix removed products.

- Ease: Adjust moving parts of designated products, or work to achieve free movement and good fit in open and closed positions.

- Match existing: Provide products and work of the same appearance and features as the original, excluding ageing and weathering. Make joints between existing and new work as inconspicuous as possible.

- System: Equipment, accessories, controls, supports and ancillary items (including installation) necessary for that section of the work to function.

- Building Manual: A document containing information of use to subsequent building owners, occupiers and users about the requirements and procedures for effective operation, maintenance, decommissioning and demolition of the building.

170 	Manufacturer and product reference

- Definition: When used in this combination:

  - Manufacturer: The person or legal entity under whose name or trademark the particular product, component or system is marketed

  - Product reference: The proprietary brand name and/ or identifier by which the particular product, component or system is described.

- Currency: References are to the particular product as specified in the manufacturer’s technical literature current on the date of the invitation to tender.

200 	Substitution of products

- Products: If an alternative product to that specified is proposed, obtain approval before ordering the product.

- Reasons: Submit reasons for the proposed substitution.

- Information to be submitted:

- Manufacturer and product reference.

- Cost.

- Availability.

- Relevant standards.

- Performance.

- Function.

- Compatibility of accessories.

- Proposed revisions to drawings and specification.

- Compatibility with adjacent work.

- Appearance.

- Copy of warranty or guarantee.

- Alterations to adjacent work: If needed, advise scope, nature and cost.

- Manufacturers’ guarantees: If substitution is accepted, submit before ordering products.

210 	Cross references

- Accuracy: Check remainder of the annotation or item description against the terminology used in the section or clause referred to.

- Related terminology: Where a numerical cross reference is not given, the relevant sections and clauses of the specification will apply.

- Relevant clauses: Clauses in the referred to specification section dealing with general matters, ancillary products and execution also apply.

- Discrepancy or ambiguity: Before proceeding, obtain clarification or instructions.

220 	Referenced documents

- Conflicts: Specification prevails over referenced documents.

230 	Equivalent products

- Inadvertent omission: Wherever products are specified by proprietary name the phrase 'or equivalent' is to be deemed included.

240 	Substitution of standards

- Specification: To British Standard or European Standard.

- Substitution: May be proposed complying with a grade or category within a national standard of another Member State of the European Community or an International Standard recognized in the UK.

- Before ordering: Submit notification of all such substitutions.

- Documentary evidence: Submit for verification when requested as detailed in clause A31/200. Any submitted foreign language documents must be accompanied by certified translations into English.

250 	Currency of documents and information

- Currency: References to published documents are to the editions, including amendments and revisions, current on the date of the invitation to tender.

260 	Sizes

- General dimensions: Products are specified by their co-ordinating sizes.

- Timber: Cross section dimensions shown on drawings are:
Target sizes as defined in BS EN 336 for structural softwood and hardwood sections.
Finished sizes for non-structural softwood or hardwood sawn and further processed sections.

Documents provided on behalf of the employer

410 	Additional copies of drawings/ documents

- Additional copies: Issued free of charge.

440 	Dimensions

- Scaled dimensions: Do not rely on.

460 	The specification

- Coordination: All sections must be read in conjunction with Main Contract Preliminaries/ General conditions.

Documents provided by contractor/ subcontractors/ suppliers

600 	Contractor's Design information

- General: Complete the design and detailing of parts of the Works as specified.

- Provide: Production information based on the drawings, specification and other information.
Liaison to ensure coordination of the work with related building elements and services.

- Master programme: Make reasonable allowance for completing design/ production information, submission (including information relevant to the CDM Regulations), comment, inspection, amendment, resubmission and reinspection.

- Information required: Electrical, Mechanical Ventilation and Drainage

  - Format: Electronic

- Submit: Within one week of request.

640 	Maintenance instructions and guarantees

- Components and equipment: Obtain or retain copies, register with manufacturer and hand over on or before completion of the Works.

- Information location: In Building Manual.

Ω End of Section

A32
Management of the works

Generally

110 	Supervision

- General: Accept responsibility for coordination, supervision and administration of the Works, including subcontracts.

- Coordination: Arrange and monitor a programme with each subcontractor, supplier, local authority and statutory undertaker, and obtain and supply information as necessary for coordination of the work.

118 	Vehicle safety requirements

- Vehicle equipment: Ensure that all vehicles have the following:

- Audible alert to other road users of the planned movement of the vehicle when the vehicle’s indicators are in operation.

- Prominent signage at the rear of the vehicle to warn cyclists of the dangers of passing the vehicle on the inside.

- Properly adjusted Class VI mirror(s) or Fresnel lens to eliminate the near-side blind spot.

- Side underrun guards.

- Drivers:

- Trained on vulnerable road user safety through an approved course.

- Hold a current valid Certificate of Professional Competence.

- Have a valid driving licence and be legally able to drive the vehicle.

- Scheme membership: Submit evidence of registration with and accreditation to the Fleet Operator Recognition Scheme (FORS)

120 	Insurance

- Documentary evidence: Before starting work on site submit details, and/ or policies and receipts for the insurances required by the Conditions of Contract.

125 	Professional Indemnity Insurance

- Provide and maintain Professional Indemnity insurance in respect of Contractor Designed Works

  - Level of cover: Relates to claims or series of claims arising out of one event.

  - Period of insurance for these purposes: One year.

- Amount of indemnity required: £250,000

- Expiry of required period of CDP Professional Indemnity insurance: 12 years.

- Documentary evidence: Details and/ or policies and receipts for the insurances required.

- Submittal date: Before starting work on site.

130 	Insurance claims

- Notice: If an event occurs that may give rise to any claim or proceeding in respect of loss or damage to the Works or injury or damage to persons or property arising out of the Works, immediately give notice to the Employer, the person administering the Contract on their behalf and the insurers.

- Failure to notify: Indemnify the Employer against any loss, which may be caused by failure to give such notice.

150 	Ownership

- Alteration/ clearance work: Materials arising become the property of the Contractor except where otherwise stated. Remove from site as work proceeds.

155 	Environmental Management

- General: 
The Contractor shall put in place and implement a programme approved by the Council's Environmental Management Team aimed at conserving resources and protecting the environment in the execution of the contract by implementing measures to :-
- ensure compliance with the Councils' environmental policies.
- ensure compliance with environmental regulations relating to air, water and land emissions including noise and traffic.
- prevent or otherwise render harmless the release of hazardous substances into the environment.
- minimise waste production throughout the product life cycle and the duration of any works.
- work with the Council to reduce, reuse or recycle waste materials wherever practical.
- reduce energy and water demand, maximise energy and water efficiency and reduce carbon emissions.
- ensure effective protection of fauna and flora especially where works take place in an environmentally sensitive area.

165 	Environmental Compliance

- General: The Contractor shall provide to the Council's Environmental Team evidence of compliance on request to allow monitoring of the Contractor's compliance with Clause 155 specifically and other clauses which may affect the environmental impact of this contract

- Audit: The Council's Environmental Team may undertake an environmental audit of this contract and the Contractor shall fully cooperate as reasonably requested with the audit team and implement any identified improvements according to timetables set by the Council.

Programme/ progress

210 	Programme

- Master programme: When requested, and before starting work on site, submit a master programme for the Works in an approved form.

- Include:

  - Planning: Planning and mobilization by the Contractor, including subcontractor's work.

  - Engineering services: Running in, adjustment, commissioning and testing of engineering services and installations.

  - Instructions: Work resulting from instructions issued in regard to the expenditure of provisional sums.

  - Concurrent work: Work by others and concurrent with the Contract. The nature, scope and relevant limitations of which are suitably defined in the Contract Documents, as is the relationship with preceding and following work.

- Number of copies: Electronic

- Submittal date: Within one weeks request.

240 	Notice of commencement of work

- Notice period (minimum): Two weeks.

250 	Monitoring

- Progress

  - Records: Record on a copy of the programme kept on site.

  - Delays: Minimize. Take appropriate action to recover lost time.

  - Corrective action: Where progress falls below target, submit proposals.

  - Submittal date: As soon as possible.

  - Completion forecast: Submit on the last working day of each week.

260 	Site meetings

- General: Site meetings will be held to review progress and other matters arising from administration of the Contract.

- Frequency: Every month

- Location: Site office.

- Accommodation: Ensure availability at the time of such meetings.

- Attendees: Attend meetings and inform subcontractors and suppliers when their presence is required.

- Chairperson (who will also take and distribute minutes): Contract Administrator

290 	Notice of completion

- Requirement: Give notice of the anticipated Date for Completion of the whole or parts of the Works.

- Associated works: Ensure that necessary access, services and facilities are complete.

- Period of notice (minimum): One month

310 	Extensions of time

- Notice: When a notice of the cause of any delay or likely delay in the progress of the works is given under the contract, written notice must also be given of all other causes which apply concurrently.

- Details: As soon as possible, submit:

- Relevant particulars of the expected effects, if appropriate, related to the concurrent causes.

- An estimate of the extent, if any, of the expected delay in the completion of the Works beyond the Date for Completion.

- All other relevant information required.

Control of cost

420 	Removal/ replacement of existing work

- Extent and location: Agree before commencement.

- Execution: Carry out in ways that minimize the extent of work.

430 	Proposed instructions

- Estimates: If a proposed instruction requests an estimate of cost, submit without delay and in any case within seven days.

440 	Measurement

- Covered work: Give notice before covering work required to be measured.

450 	Daywork vouchers

- Before commencing work: Give reasonable notice to person countersigning daywork vouchers.

- Content: Before delivery, each voucher must:

- Be referenced to the instruction under which the work is authorized.

- Include a full description of the work undertaken and time spent on individual tasks.

- Be signed by the Contractor's person in charge as evidence that the operatives' names, the time spent by each, and the plant and materials shown are correct.

- Submit: By the end of the week in which the work has been executed.

470 	Products not incorporated into the Works

- Ownership: At the time of each valuation, supply details of those products not incorporated into the Works which are subject to reservation of title inconsistent with passing of property as required by the Conditions of Contract, together with their respective values.

- Evidence: When requested, provide evidence of freedom of reservation of title.

Ω End of Section

A33
Quality standards/ control

Standards of products and executions

110 	Incomplete documentation

- General: Where and to the extent that products or work are not fully documented, they are to be:

  - Standard: Of a kind and standard appropriate to the nature and character of that part of the Works where they will be used.

  - Suitability: Suitable for the purposes stated or reasonably to be inferred from the project documents.

- Contract documents: Omissions or errors in description and/ or quantity shall not vitiate the Contract nor release the Contractor from any obligations or liabilities under the Contract.

120 	Workmanship skills

- Operatives: Appropriately skilled and experienced for the type and quality of work.

- Registration: With Construction Skills Certification Scheme.

- Verification: When requested, operatives must produce evidence of skills/ qualifications.

130 	Quality of products

- Generally: New (proposals for recycled products may be considered).

- Supply of each product: From the same source or manufacturer.

- Whole quantity of each product required to complete the Works: Consistent kind, size, quality and overall appearance.

- Tolerances: Where critical, measure a sufficient quantity to determine compliance.

- Deterioration: Prevent. Order in suitable quantities to a programme and use in appropriate sequence.

132 	Timber Procurement

- Timber (including timber for wood based products): Obtained from well managed forests/plantations in accordance with: - The laws governing forest management in the producer country or countries. - International agreements such as the Convention on International Trade in Endangered Species of wild fauna and flora (CITES).

135 	Quality of execution

- Generally: Fix, apply, install or lay products securely, accurately, plumb, neatly and in alignment.

- Colour batching: Do not use different colour batches where they can be seen together.

- Dimensions: Check on-site dimensions.

- Finished work: Without defects, e.g. not damaged, disfigured, dirty, faulty or out of tolerance.

- Location and fixing of products: Adjust joints open to view so that they are even and regular.

140 	Evidence of Compliance

- Proprietary products: Retain on site evidence that the proprietary product specified has been supplied.

- Performance specification: Submit upon request evidence of compliance with performance specifications, including:

- Test reports indicating properties tested.

- Pass or fail criteria.

- Test methods and procedures.

- Test results.

- Identity of testing agency.

- Test dates and times.

- Identities of witnesses.

- Analysis of results.

150 	Inspections

- Products and executions: Inspection, or other action, of products or executions must not be taken as approval, unless confirmed in writing and including:

- Date of inspection.

- Part of the work inspected.

- Respects or characteristics which are approved.

- Extent and purpose of the approval.

- Any associated conditions.

155 	Protection of products

- Details: Protection of products:

  - Prevent over-stressing, distortion and other damage.

  - Keep clean and free from contamination. Prevent staining, chipping, scratching or other disfigurement, particularly of products exposed to view in the finished work.

  - Keep dry to prevent premature setting, moisture movement and similar defects. Where appropriate store off ground and allow free air movement between stored products.

  - Prevent excessively high or low temperatures and rapid changes of temperature in the products.

  - Protect adequately from rain, damp, frost, sun and other elements as appropriate.

  - Ensure that products are at a suitable temperature and moisture content at time use.

  - Ensure that sheds and covers are of ample size, in good weatherproof condition and well secured.

  - Keep different types and grades of products separately and adequately identified.

  - Keep products in their original wrappings, packings or containers until immediately before they are used. Wherever possible retain protective wrappings after fixing and until shortly before practical completion.

  - Ensure that protective measures are fully compatible with and not prejudicial to the products/materials.

160 	Related work

- Details: Provide all trades with necessary details of related types of work. Before starting each new type or section of work ensure previous related work is:

- Appropriately complete.

- In accordance with the project documents.

- To a suitable standard.

- In a suitable condition to receive the new work.

- Preparatory work: Ensure that all necessary preparatory work has been carried out.

170 	Manufacturer’s recommendations/ instructions

- General: Comply with manufacturer's printed recommendations and instructions current on the date of the Invitation to tender.

- Exceptions: Submit details of changes to recommendations or instructions.

- Execution: Use ancillary products and accessories supplied or recommended by main product manufacturer.

- Products: Comply with limitations, recommendations and requirements of relevant valid certificates.

180 	Water for the works

- Mains supply: Clean and uncontaminated.

- Other: Do not use until:

- Evidence of suitability is provided.

- Tested to BS EN 1008 if instructed.

Samples/ approvals

220 	Approval of products

- Submissions, samples, inspections and tests: Undertake or arrange to suit the Works programme.

- Approval: Relates to a sample of the product and not to the product as used in the Works. Do not confirm orders or use the product until approval of the sample has been obtained.

- Complying sample: Retain in good, clean condition on site. Remove when no longer required.

230 	Approval of execution

- Submissions, samples, inspections and tests: Undertake or arrange to suit the Works programme.

- Approval: Relates to the stated characteristics of the sample. (If approval of the finished work as a whole is required this is specified separately). Do not conceal, or proceed with affected work until compliance with requirements is confirmed.

- Complying sample: Retain in good, clean condition on site. Remove when no longer required.

Accuracy/ setting out generally

320 	Setting out

- General: Submit details of methods and equipment to be used in setting out the Works.

- Levels and dimensions: Check and record the results on a copy of drawings. Notify discrepancies and obtain instructions before proceeding.

- Inform: When complete and before commencing construction.

330 	Appearance and fit

- Tolerances and dimensions: If likely to be critical to execution or difficult to achieve, as early as possible either:

- Submit proposals; or

- Arrange for inspection of appearance of relevant aspects of partially finished work.

- General tolerances (maximum): To BS 5606, Tables 1 and 2.

350 	Levels of structural floors

- Maximum tolerances for designed levels to be

  - Floors to be self-finished, and floors to receive sheet or tile finishes directly bedded in adhesive: +/- 10 mm.

  - Floors to receive dry board/ panel construction with little or no tolerance on thickness: +/- 10 mm.

  - Floors to receive mastic asphalt flooring/ underlays directly: +/- 10 mm.

  - Floors to receive mastic asphalt flooring/ underlays laid on mastic asphalt levelling coat(s): +/- 15 mm.

  - Floors to receive fully bonded screeds/ toppings/ beds: +/- 15 mm.

  - Floors to receive unbonded or floating screeds/ beds: +/- 20 mm.

360 	Record drawings

- Site setting out drawing: Record details of all grid lines, setting-out stations, benchmarks and profiles. Retain on site throughout the Contract and hand over on completion.

Services generally

410 	Services regulations

- New or existing services: Comply with the bye-laws or regulations of the relevant statutory authority.

420 	Water regulations/ bye-laws notification

- Requirements: Notify water Statutory Provider of any work carried out to, or which affects, new or existing services. Submit required plans, diagrams and details.

- Consent: Allow adequate time to receive Statutory Provider's consent before starting work. Inform the Employer immediately if consent is withheld or is granted subject to significant conditions.

430 	Water regulations/ bye-laws contractor’s certificate

- On completion of the work: Submit a certificate (copy where also required to the water Statutory Provider), including:

  - Installation: Description of the new installation and/ or the work carried out to an existing installation, including the address.

  - Statement: Confirmation that the installation complies with the relevant water regulations or bye-laws.

  - Inspection: Provide the Contractor's name and address, the name and signature of the individual responsible for checking compliance and the date on which the installation was checked.

435 	Electrical installation certificate

- Submit: When relevant electrical work is completed.

- Original certificate: To be lodged in the Building Manual.

445 	Service runs

- General: Provide adequate space and support for services, including unobstructed routes and fixings.

- Ducts, chases and holes: Form during construction rather than cut in situ.

- Coordination with other works: Submit details of locations, types and methods of fixing of services to fabric and identification of runs and fittings.

450 	Mechanical and electrical services

- Final tests and commissioning: Carry out so that services are in full working order at completion of the Works.

- Building Regulations notice: Copy to be lodged in the Building Manual.

Supervision/ inspection/ defective work

525 	Access

- Extent: Provide access to the Works at reasonable times; also to other places where the Contractor or subcontractors are preparing work for the Contract.

530 	Overtime working

- Notice: Prior to overtime being worked, submit details of times, types and locations of work to be done.

  - Minimum period of notice: One week

- Concealed work: If executed during overtime for which notice has not been given, it may be required to be opened up for inspection and reinstated at the Contractor's expense.

540 	Defects in existing work

- Undocumented defects: When discovered, immediately give notice. Do not proceed with affected related work until response has been received.

- Documented remedial work: Do not execute work which may hinder access to defective products or work, or be rendered abortive by the remedial work.

560 	Tests and inspections

- Timing: Agree and record dates and times of tests and inspections to enable all affected parties to be represented.

- Confirmation: One working day prior to each such test or inspection. If sample or test is not ready, agree a new date and time.

- Records: Submit a copy of test certificates and retain copies on site.

Work at or after completion

710 	Work before completion

- General: Make good all damage consequent upon the Works.

- Temporary markings, coverings and protective wrappings: Remove unless otherwise instructed.

- Cleaning: Clean the Works thoroughly inside and out, including all accessible ducts and voids. Remove all splashes, deposits, efflorescence, rubbish and surplus materials.

- Cleaning materials and methods: As recommended by manufacturers of products being cleaned, and must not damage or disfigure other materials or construction.

- COSHH dated data sheets: Obtain for all materials used for cleaning and ensure they are used only as recommended by their manufacturers.

- Minor faults: Touch up in newly painted work, carefully matching colour and brushing out edges. Repaint badly marked areas back to suitable breaks or junctions.

- Moving parts of new work: Adjust, ease and lubricate as necessary to ensure easy and efficient operation, including doors, windows, drawers, ironmongery, appliances, valves and controls.

720 	Security at completion

- General: Leave the Works secure with, where appropriate, all accesses closed and locked.

- Keys: Account for and adequately label all keys, and hand over together with an itemized schedule, retaining duplicate schedule signed as a receipt.

730 	Making good defects

- Remedial work: Arrange access with Contract Administrator.

- Rectification: Give reasonable notice for access to the various parts of the Works.

- Completion: Notify when remedial works have been completed.

Ω End of Section

A34
Security/ safety/ protection

Security, health and safety

150 	Security

- Protection: Safeguard the site, the Works, products, materials, and any existing buildings affected by the Works from damage and theft.

- Access: Take all reasonable precautions to prevent unauthorized access to the site, the Works and adjoining property.

160 	Stability

- Responsibility: Maintain the stability and structural integrity of the works and adjacent structures during the Contract.

- Design loads: Obtain details, support as necessary and prevent overloading.

210 	Safety provisions for site visits

- Access: Provide at reasonable times.

- Inspections: Agree dates and times several days in advance, to enable affected parties to be present.

- Safety: Submit details in advance of safety provisions and procedures (including those relating to materials, which may be deleterious), which will require their compliance when visiting the site.

- Protective clothing and/ or equipment: Provide and maintain on site for visitors to the-site.

Protect against the following

330 	Noise and vibration

- Standard: In accordance with BS 5228-1.

- Noise levels from the Works: Maximum level: 85 dB(A) when measured from the boundary.

- Equipment: Fit compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles.

- Restrictions: Obtain consent before using percussion tools and other noisy appliances. Do not use radios or other audio equipment, or permit employees to use them in ways or at times that may cause nuisance.

340 	Pollution

- Prevention: Protect the site, the works and the general environment (including the atmosphere, land, streams and waterways) against pollution.

- Contamination: If pollution occurs, report immediately, including to the appropriate authorities, and provide relevant information.

350 	Pesticides

- Use: Not permitted.

360 	Nuisance

- Duty: Prevent nuisance from smoke, dust, rubbish, vermin and other causes.

- Surface water: Prevent hazardous build-up on site, in excavations and to surrounding areas and roads.

370 	Asbestos containing materials

- Duty: Report immediately any suspected materials discovered during execution of the works. Do not disturb, and agree methods for safe removal or encapsulation.

371 	Dangerous or hazardous substances

- Duty: Report immediately suspected materials discovered during execution of the works. Do not disturb and agree methods for safe removal or remediation.

390 	Smoking on-site

- Smoking on-site: Not permitted.

400 	Burning on-site

- Burning on-site: Not permitted.

410 	Moisture

- Wetness or dampness: Prevent, where this may cause damage to the Works.

- Drying out: Control humidity and the application of heat to prevent:

- Blistering and failure of adhesion.

- Damage due to trapped moisture.

- Excessive movement.

420 	Infected timber/ Contaminated materials

- Removal: Where instructed to remove material affected by fungal/ insect attack from the building, minimize the risk of infecting other parts of the building.

- Testing: Carry out and keep records of appropriate tests to demonstrate that hazards presented by concentrations of airborne particles, toxins and other microorganisms are within acceptable levels.

430 	Waste

- Waste: Includes rubbish, debris, spoil, containers and packaging, and surplus material requiring disposal.

- Requirement: Minimize production and prevent accumulation of waste. Keep the site and works clean and tidy. Clean out voids and cavities in the construction before closing.

- Disposal: Collect and store in suitable containers. Remove from site and dispose of in a safe and competent manner, as approved and directed by the waste regulation authority.

- Recyclable material: Sort and dispose of at a materials recycling facility approved by the waste regulation authority.

- Documentation: Retain on-site.

440 	Electromagnetic interference

- Duty: Prevent excessive electromagnetic disturbance to apparatus outside the site.

470 	Invasive species

- General: Prevent the introduction or spread of species (e.g. plants or animals) that may adversely affect the site and/ or the Works economically, environmentally or ecologically.

- Duty: Report immediately any suspected invasive species discovered during execution of the Works. Do not disturb and agree methods for safe eradication or removal.

Protect the following

510 	Existing services

- Confirmation: Notify service authorities, statutory undertakers and/ or adjacent owners of proposed works not less than one week before commencing site operations.

- Identification: Before starting work, check and mark positions of utilities/ services. Where positions are not shown on drawings obtain relevant details from service authorities, statutory undertakers or other owners.

- Work adjacent to services: Comply with service authority's or statutory undertaker's recommendations. Adequately protect, and prevent damage to services. Do not interfere with their operation without consent of service authorities, statutory undertakers or other owners.

- Identifying services

  - Below ground: Use signboards, giving type and depth.

  - Overhead: Use headroom markers.

- Damage to services

  - Action: Immediately give notice and notify appropriate service authority/ statutory undertaker.

  - Repair: Make arrangements for making good without delay to the satisfaction of service authority, statutory undertaker or other owner as appropriate.

- Liability: Measures taken to deal with an emergency will not affect the extent of the Contractor's liability.

- Marker tapes or protective covers: Replace, if disturbed during site operations, to service authority's/ statutory undertakers recommendations.

520 	Roads and footpaths

- Duty: Maintain roads and footpaths within and adjacent to the site and keep clear of mud and debris.

- Damage caused by site traffic or otherwise consequent upon the Works: Make good to the satisfaction of the Employer, local authority or other owner.

530 	Existing topsoil/ subsoil

- Duty: Prevent over compaction of existing topsoil and subsoil in those areas which may be damaged by construction traffic, parking of vehicles, temporary site accommodation or storage of materials and which will require reinstatement prior to completion of the Works.

- Protection: Before starting work submit proposals for protective measures.

540 	Retained trees/ shrubs/ grassed areas

- Protection: Preserve and prevent damage, except those not required.

- Replacement: Mature trees and shrubs if uprooted, destroyed, or damaged beyond reasonable chance of survival in their original shape, as a consequence of the Contractor's negligence, must be replaced with those of a similar type and age at the Contractor's expense.

550 	Retained trees

- Protected area: Unless agreed otherwise, do not dump spoil or rubbish, excavate or disturb topsoil, park vehicles or plant, store materials or place temporary accommodation within the root protection area.

- Roots: Do not sever if exceeding 25 mm in diameter. If unintentionally severed, give notice and seek advice.

- Ground levels: Do not change within the root protection area.

560 	Existing features

- Protection: Prevent damage to existing buildings, fences, gates, walls, roads, paved areas and other site features, which are to remain in position during execution of the Works.

570 	Existing work

- Protection: Prevent damage to existing work, structures or other property during the course of the work.

- Removal: Minimum amount necessary.

- Replacement work: To match existing.

580 	Building interiors

- Protection: Prevent damage from exposure to the environment, including weather, flora, fauna, and other causes of material degradation during the course of the work.

600 	Existing furniture, fittings and equipment

- Protection: Prevent damage or move as necessary to enable the Works to be executed. Reinstate in original positions.

625 	Adjoining property restrictions

- Precautions: Prevent trespass of workpeople and take precautions to prevent damage to adjoining property. Pay charges. Remove temporary protection and make good on completion or when directed.

- Damage: Bear cost of repairing damage arising from execution of the Works.

630 	Existing structures

- Duty: Check proposed methods of work for effects on adjacent structures inside and outside the site boundary.

- Supports

  - Standards: In accordance with BS 5975 and BS EN 12812.

  - Requirements:

- Provide and maintain all incidental shoring, strutting, needling and other supports as may be necessary to preserve stability of existing structures on the site or adjoining, which may be endangered or affected by the Works.

- Do not remove until new work is strong enough to support existing structure.

- Prevent overstressing of completed work when removing supports.

- Adjacent structures: Monitor and immediately report excessive movement.

640 	Materials for recycling/ reuse

- Duty: Sort and prevent damage to stated products or materials, clean off bedding and jointing materials and other contaminants.

- Storage: Stack neatly and protect until required by the Employer or for use in the Works as instructed.

Ω End of Section

A35
Specific limitations on method/ sequence/ timing

Clauses

160 	Use or disposal of materials

- Specific limitations: Skips and materials to be secure and sited within the site boundary.

Ω End of Section

A36
Facilities/ temporary work/ services

Generally

110 	Spoil heaps, temporary works and services

- Location: Give notice and details of intended siting.

- Maintenance: Alter, adapt and move as necessary. Remove when no longer required and make good.

Accommodation

210 	Room for meetings

- Facilities: Provide suitable temporary accommodation for site meetings, adequately heated and lit. The room may be part of the Contractor's own site offices.

- Furniture and Equipment: Provide table and chairs for 8 people.

230 	Temporary accommodation

- Accommodation made available by the Employer: The following may be used for the duration of the Contract without charge provided that:

- It is used solely for the purposes of carrying out the Works.

- The use to which it is put does not involve undue risk of damage.

- Temporary adaptations are approved by or on behalf of the Employer before being carried out.

- It is vacated on completion of the Works or determination of the Contract.

- When vacated, its condition is at least equivalent to its condition at the start of the Contract.

- Description: 1 no. existing toilet and basin to be made available.

- Available services and facilities: Water and power available.

Temporary works

340 	Name boards/ advertisements

- Name boards/ advertisements: Permitted subject to agreement.

Services and facilities

420 	Lighting and power

- Supply: Electricity from the existing mains may be used for the Works as follows:

  - Metering: Free of charge

  - Point of supply: Mains distribution board.

  - Frequency: 50 Hz.

  - Phase: 3

  - Current: Alternating.

- Continuity: No responsibility will be accepted for the consequences of failure or restriction in supply.

430 	Water

- Supply: The existing mains may be used for the Works as follows:

  - Metering: Free of charge

  - Source: Mains

  - Location of supply point: TBC

- Continuity: No responsibility will be accepted for the consequences of failure or restriction in supply.

570 	Personal protective equipment

- General: Provide the equipment described below for the sole use of other members of the project team, in sizes to be specified.

- Safety helmets:

  - Standard: To BS EN 397, neither damaged nor time expired.

  - Number required: 4

- High-visibility waistcoats:

  - Standard: To BS EN ISO 20471.

  - Number required: 4

Ω End of Section

A37
Operation/ maintenance of the finished works

Generally

110 	The Building Manual

- Responsibility: The Contractor.

- Content: Obtain and provide comprehensive information for owners and users of the completed Works. Include an overview of the main design principles. Describe key components and systems within the finished Works to enable proper understanding, efficient and safe operation, and maintenance.

- Specific requirements: Warranties to be in the name of West Suffolk Council.

- Format: Electronic.

- Delivery to: CDM Advisor by (date) Prior to practical completion.

155 	Content of the Building Manual

- General: Details of the property, the parties, fire safety strategy, operational requirements and constraints of a general nature.

- Building fabric: Design criteria, maintenance details, product details, and environmental and trafficking conditions.

- Building services: Description and operation of systems, diagrammatic drawings, record drawings, identification of services, product details, equipment settings, maintenance schedules, consumable items, spares and emergency procedures.

- Documentation: Guarantees, warranties, maintenance agreements, test certificates and reports.

Ω End of Section

A40
Contractor's general cost items: management and staff

Clauses

110 	Management and staff

- Cost-significant items: TBC

Ω End of Section

A41
Contractor's general cost items: site accommodation

Clauses

110 	Site accommodation

- Details: Site accommodation required or made/ not made available by the Employer: See section A36.

- Cost significant items: TBC

Ω End of Section

A42
Contractor's general cost items: services and facilities

Clauses

110 	Services and facilities

- Details: Services or facilities required or made/ not made available by the Employer: See section A36.

- Cost significant items: TBC

Ω End of Section

A43
Contractor's general cost items: mechanical plant

Clauses

110 	Mechanical plant

- Cost significant items: TBC

Ω End of Section

A44
Contractor's general cost items: temporary works

Clauses

110 	Temporary works

- Details: Temporary works required or made/ not made available by the Employer: See section A36.

- Cost significant items: TBC

Ω End of Section

A50
Work/ products by/ on behalf of the employer

Clauses - No Amendments

Ω End of Section

A53
Work by statutory authorities/ undertakers

Clauses - No Amendments

Ω End of Section

A54
Provisional work/ items

Clauses

110 	Provisional sums for defined work

- Item: Works to existing drainage

- Description of work: Allow for adjustments to existing drainage runs

- Provisional Sums: Include £6,000.00.

- Allow for general attendance.

210 	Provisional sums for undefined work

- Item: Fire stopping

- Description of work: Allow for new means of fire stopping as required by Building Regulations

- Provisional Sums: Include £10,000.00.

- Allow for general attendance.

590 	Contingencies

- Provisional sum: Include: £20,000.00.

- A contingency sum will be held by the client.

Ω End of Section

A55
Dayworks

Clauses

150 	Daywork Charges

- General: Where an instruction is issued requiring a variation which is not of a similar character or executed under similar conditions to work included in the Contract and where work cannot properly be measured and valued, the Contractor shall be allowed payment on a daywork basis at the following rates:

- RICS/ Construction Confederation: Prime cost of labour: The sum of £ 1000.

  - Percentage adjustment to cover incidental costs, overheads and profit: TO BE COMPLETED BY CONTRACTOR%.

- Prime cost of materials and goods: The sum of £ 1000.

  - Percentage adjustment to cover incidental costs, overheads and profit: TO BE COMPLETED BY CONTRACTOR%.

- Prime cost of plant: The sum of £ 1000.

  - Percentage adjustment to cover incidental costs, overheads and profit: TO BE COMPLETED BY CONTRACTOR%.

- RICS/ Electrical Contractors' Association: Prime cost of labour: The sum of £ 1000.

  - Percentage adjustment to cover incidental costs, overheads and profit: TO BE COMPLETED BY CONTRACTOR%.

- Prime cost of materials and goods: The sum of £ 1000.

  - Percentage adjustment to cover incidental costs, overheads and profit: TO BE COMPLETED BY CONTRACTOR

Ω End of Section

| General Summary | £ |
| --- | --- |
| A10 Project particulars 1 A11 Tender and contract documents 3 A12 The site/ existing buildings 4 A13 Description of the work 6 A20 JCT Minor Works Building Contract with contractor's design (MWD) 7 A30 Tendering/ subletting/ supply 11 A31 Provision, content and use of documents 13 A32 Management of the works 17 A33 Quality standards/ control 21 A34 Security/ safety/ protection 27 A35 Specific limitations on method/ sequence/ timing 31 A36 Facilities/ temporary work/ services 32 A37 Operation/ maintenance of the finished works 34 A40 Contractor's general cost items: management and staff 35 A41 Contractor's general cost items: site accommodation 36 A42 Contractor's general cost items: services and facilities 37 A43 Contractor's general cost items: mechanical plant 38 A44 Contractor's general cost items: temporary works 39 A50 Work/ products by/ on behalf of the employer 40 A53 Work by statutory authorities/ undertakers 41 A54 Provisional work/ items 42 A55 Dayworks 43 |  |
| Preliminaries Sub-Total |  |