100

Internal refurbishment of an existing industrial unit

20a Hampstead Avenue

SCHEDULE A.1: Schedule of Works

Rev B

File Ref: PRJ000042

January 2025

Tender clarifications higlighted in cyan.

|  | Description of Works Demolition of existing internal rooms and construction of internal rooms to form new accommodation and associated elements. Also refer to attached Documents; Schedule A.2: Tender schedule -Annex 1- Drawings: -Annex 2- Preliminaries -Annex 3- Asbestos Survey -Annex 4- Pre-Construction Information (DMA Health & Safety Ltd) -Annex 5- Safety & Environmental Requirements for Contractors & Subcontractors -Annex 6 – Drain Survey -Annex 7 – AC Leigh Ironmongery Schedule -Annex 8 - Reference Specification Annex 9 – Structural Information A copy of the Council’s Standard Terms and Conditions may be downloaded from: http://www.westsuffolk.gov.uk/Business/Selling_to_the_Council/ index.cfm?aud=business |  |  |
| --- | --- | --- | --- |
| 1.0 | General Conditions |  |  |
| 1.1 | Supply all documentation relating to Construction (Design & Management) Regulations 2015 prior to commencement on site, including site specific Risk Assessments and Method Statements to the Contract Administrator for approval. Attention is drawn to the Council’s Safety and Environmental requirements for Contractors and Sub-contractors (included within tender package). The contractor is to provide all relevant warranty certificates under the name of ‘West Suffolk Council’. The method statement and programme are to ensure that internal elements are not exposed for longer than necessary. |  |  |
| 1.2 | Allow for providing safe system of work i.e., protection for the public and compound area as determined by the risk assessment and for the removal from site on completion. Fencing is to be a minimum of 1.8m high Heras type and is to remain for the duration of the works to the specified areas with appropriate signage securely affixed. Skips are to be secured when not in use. Compound location to be agreed with client and is not to hinder the vehicular through route, access to rear of other tenanted units or tenant parking. Adequate means of escape from the premises are to be maintained during the building work whilst the premises is occupied. |  |  |
| 1.3 | Power and water and toilet facilities will be not be available for use of the contractor on site. Toilet and mess facilities to be provided by contractor. The contractor will be responsible for keeping these facilities clean and tidy and stocked with consumables and for returning those provided by the client in clean condition at the end of the works. ` |  |  |
| 1.4 | It is a mandatory requirement that the contractor attends site prior to the tender clarification deadline, to fully appreciate the extent of works required and to undertake the relevant measurement surveys. See Invitation to Tender document for further information. The contractor will be responsible for taking of all measurements as required prior to ordering materials. |  |  |
| 1.6 | The works are subject to Local Authority Building Control approval and the contractor will be required to make the site available and arrange for inspections with West Suffolk Building Control at the relevant stages. (Building Regulations Application to be submitted by client). Contact details for WSC Building Control will be provided by the client. |  |  |
| 1.7 | Where possible, waste materials are to be separated, crushed and re-used/recycled. |  |  |
| 1.8 | The Contractor will be responsible for identifying all underground services in accordance with the risk assessments, prior to excavation works. |  |  |
| 1.9 | Allow for any other contractor obligations as described in Annex 2- Preliminaries |  |  |
| 1.10 | Allow for any other contractor obligations as described in Annex 4 - Pre-Construction Information. |  |  |
|  | CARRIED TO SUMMARY | £ |  |
| 2.0 | Demolition/ Asbestos Removal and Site Clearance |  |  |
| 2.1 | Referring to the existing Asbestos R/D survey allow for the removal of asbestos containing materials associated (PXB-5). The work below shall be carried out in accordance with HSE guidelines and comprises a combination of Licensed and Non-licensed asbestos work. Licensable work must be notified to the enforcing authority and carried out by a Licensed contractor. Notifiable non-licensed work must be notified to the enforcing authority. |  |  |
| 2.2 | All waste cleared from site must be in licenced skips to authorised waste landfill sites. |  |  |
| 2.3 | Disconnect and cap off all services and adapt where necessary and leave for future use. |  |  |
| 2.4 | Disconnect and remove the redundant external condensing unit and associated services and cart away |  |  |
| 2.5 | Carefully take down the existing internal partitions, sanitaryware, door sets as shown on the drawings and cart away. |  |  |
| 2.6 | Breakout the existing concrete hardstanding adjacent the existing roller shutter doors as shown on the drawings and cart away. |  |  |
|  | CARRIED TO SUMMARY | £ |  |
| 3.0 | Below Ground Drainage |  |  |
| 3.1 | Allow a Provisional Sum of £4,000 for adjusting the existing drainage. |  |  |
| 3.2 | Allow for re-positioning the existing stop cock and adjusting incoming pipework to suit the new layout. |  |  |
|  | CARRIED TO SUMMARY | £ |  |
| 4.0 | Carpentry |  |  |
| 4.1 | Supply and fit 225 x 50mm C16 flat roof joists @ 400mm c/c’s to the new flat roof to the proposed internal rooms to the warehouse. Allow for double joists/ trimmers over openings and over the main warehouse entrance. |  |  |
| 4.2 | Supply and fit 150 x 47mm timber studwork @ 400 c/c’s to the new internal wall lining to locations shown the drawings. |  |  |
| 4.3 | Build new internal studwork partitions as shown on the drawings, to be formed in 100 x 50mm regularised treated timbers at 400 mm crs with matching size noggins at 450 mm staggered vertical crs and 100mm. Form new door openings where indicated. Double timber studs/lintels over the openings. |  |  |
| 4.4 | Supply and fix 18mm external grade WBP ply to the internal fascia to the new internal rooms. |  |  |
| 4.5 | Boxing to the stub stacks to the w.c.’s. Use 12mm moisture resistant MDF on 50 x 50mm s/w battens. Agree final detail on site with the Contract Administrator (diamond matt emulsion paint finish as described elsewhere. |  |  |
| 4.6 | Supply and fix 12mm moisture resistant MDF boxings behind the W.C.’s to the warehouse W.C.’s and to SVP’s. Boxings to consist of 100 x 50mm s/w framing (900mm high) lined with MDF (diamond matt emulsion paint finish as described elsewhere). |  |  |
| 4.7 | Supply and fit 25mm OSB 3 deck to the new internal room flat roof. |  |  |
| 4.8 | Supply and 50 x 50mm s/w battens fixed to the proposed new flat roof joists (parallel to joists). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 5.0 | Insulation |  |  |
| 5.1 | 140mm Knauf Frame Therm 32 friction fitted in between s/w joists to the new warehouse internal walls and office internal walls. |  |  |
| 5.2 | Supply and fit 2 no. layers of 100mm Knauf Omni-fit insulation to the new flat roof to the internal rooms in the warehouse to manufacturer’s details. |  |  |
| 5.3 | 100mm Isover acoustic slab between studs/noggins to new warehouse internal partitions to manufacturer’s detail’s |  |  |
| 5.4 | Supply and fit 15mm Space Therm window reveal board to window jambs and window reveals to the office with 3mm skim coat plaster and paint finishes noted below to manufacturer’s details. |  |  |
| 5.5 | Supply and fit 35mm Knauf PIR laminate to the underside of the flat roof joists to manufacturers details (3mm skim coat plaster and paint finishes noted elsewhere). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 6.0 | Joinery |  |  |
| 6.1 | Supply and fit 30mins and 60mins fire rated door sets, door leaf’s to be ash veneer finish, contractor to submit proposals. Door linings, stops and architraves to be white gloss painted. Conceal all fixings. Vision panels to DG – 05, DG – 07 and DG - 08 (vision panel to BS 8300), no vision panels to DG – 06, DG – 08, DG – 09, DG – 10 and DG – 11. |  |  |
| 6.2 | Supply and fix 44mm (54mm to DG – 08 only) x wall width sw door linings, sw stops, chamfered sw architraves and skirting’s, timber window boards and cills (including overhangs). Conceal all fixings and prepare for decoration. |  |  |
| 6.3 | Allow for Pendock MX Ply Range pipe boxing out to the warehouse w.c.’s to manufacturer’s details. |  |  |
| 6.4 | Supply and fit s/w white painted window cills to the office windows to fit the new wall thickness. |  |  |
| 6.5 | Supply and fit kitchen wall and base units to the kitchen (G04). Base and wall units to be Howdens Allendale Dove Grey. Handles to be brushed stainless steel effect classic D, with soft shut hinges, 616mm x 38mm white granite effect bullnose laminate worktop. Lamona Arno chrome swivel mixer tap with Lamona Ashworth stainless steel single bowl inset kitchen sink. |  |  |
| 6.6 | Supply and fit 94 x 14.5mm bull nosed pre-finished mdf skirting to non-vinyl areas. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 7.0 | Linings |  |  |
| 7.1 | To the new internal rooms to the warehouse supply and fit Eurocell uPVC white fascia board (315mm width confirm on site prior to ordering) and white soffit board. Confirm dimensions on site prior to fabrication. |  |  |
| 7.2 | 15mm plasterboard lining as noted on the drawings with 3mm skim coat plaster to the new internal partitions to the warehouse (Type 1). |  |  |
| 7.3 | 15mm fireline board with 3mm skim coat plaster to the office new internal wall linings (Type 2). |  |  |
| 7.4 | 15mm fireline plasterboard linings as noted above with 3mm skim coat plaster to the internal room walls to the warehouse. |  |  |
| 7.5 | Allow a provisional sum of £4,000 for fireproofing of service penetrations through fire new and existing compartment walls (external and internal), floors is to be undertaken by a CFPS registered specialist. |  |  |
| 7.6 | To all sink locations and kitchenette (up to u/s of wall units) supply and fit splash back wall tiles, which are to be fixed on waterproof mold resistant adhesive and grout. Fix in accordance with manufacturer’s instructions. Allow for 150 x 150mm square white ceramic wall tiles. Sample to be provided for approval in good time prior to ordering. |  |  |
| 7.7 | Supply and fit 15mm fireline board to the underside of the office flat roof with 3mm skim coat plaster and paint finishes (C1 ceiling type). |  |  |
| 7.8 | Supply and fit 300 x 400mm access panels to the toilet as shown on the drawings. |  |  |
| 7.9 | Supply and fit blue grit with 3mm skim coat plaster to existing wall locations as shown on the drawings with finishes as described elsewhere (type 5). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 8.0 | Sanitaryware |  |  |
| 8.1 | Supply and install Ideal Standard sanitaryware (i.e. W.C.’s, urinals, mirrors, hand basins, hand dryers and sealant) and associated elements as noted on the drawings. |  |  |
| 8.2 | Supply and install Ideal Standard doc M pack accessible W.C. to locations shown on the drawings (N13/15). |  |  |
| 8.3 | Supply and install a new cleaner’s sink pack to the proposed cleaner’s cupboard as shown on the drawings (N13/25). |  |  |
| 8.4 | Wall tiles to be fixed on waterproof mold resistant adhesive and grout. Fix in accordance with manufacturer’s instructions. Allow for 150 x 150mm square white ceramic wall tiles. Sample to be provided for approval in good time prior to ordering. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 9.0 | Ironmongery |  |  |
| 9.1 | Supply and fit the proposed ironmongery as shown on the AC Leigh ironmongery schedule |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 10.0 | AVCL/ Breather membrane |  |  |
| 10.1 | To the new internal flat roof supply and fit Glidevale Protect BarriAir AVCL fitted to u/s of the flat roof joists to manufacturer’s details. |  |  |
| 10.2 | Supply and fit Glidevale Protect BarriAir AVCL fixed to the inside face of the new studwork (type 2 & 3 wall types) to the new internal rooms to the warehouse to manufacturer’s details. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 11.0 | Floor Finishes |  |  |
| 11.1 | Supply and lay Altro Xpresslay (Pavement finish) nonslip vinyl flooring to locations as shown on the drawings to manufacturer’s details (M50/20). Supply samples for approval prior to ordering. |  |  |
| 11.2 | Allow for a latex levelling screed to new areas (G01, G02, G03, G04, G05 and G06). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 12.0 | Decoration |  |  |
| 12.1 | Prepare all new plasterwork (to include ceiling and walls), which are to receive one mist coat and minimum two full coats emulsion magnolia finish (M60/110). |  |  |
| 12.2 | New architraves, window boards, to be rubbed down and receive a primer (to surfaces not already primed), undercoat and topcoat of gloss white Dulux paint). Lightly rub down between coats (M60/110). |  |  |
| 12.3 | Clean down the existing warehouse floor and apply 2 No. coats of Johnstone’s Heavy Duty Floor paint in Grey. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 13.0 | General Fixtures |  |  |
| 13.1 | Supply and fit fire signage to the internal doors as noted on the drawings. |  |  |
| 13.2 | Mechanically fix new male and female signs to the toilet entrance doors (47mm diameter satin finish). |  |  |
| 13.3 | Mechanically fix new accessible toilet sign to the toilet entrance doors (47mm diameter satin finish). |  |  |
| 13.4 | Mechanically fix new cleaners cupboard sign to the toilet entrance doors (47mm diameter satin finish). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 14.0 | Electrics – (BY OTHERS) |  |  |
| 14.1 | Electrical installation to be as shown on drawing and to be carried out as in accordance with current NICEIC Regulations. Upon completion of the work an electrical test certificate is to be provided from an approved installer in accordance with BS7671 and IEE Model forms. Unit has been fitted with new DB and contractor’s sockets. All cables or ducts passing through fire compartments to have intumescent fire sleeves or sealant to maintain fire resistance. Emergency lighting to be designed and installed to British Standards BS 5266-1:2016. Illuminated fire exit signage to BS 5499. The main contractor is to allow attendance on the named sub-contractor. |  |  |
| 14.2 | The main contractor is to allow attendance on the named sub-contractor. |  |  |
| 14.3 | Allow a for builders work in connection with the electrical installation (BWIC). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 15.0 | Fire Detection (BY OTHERS) |  |  |
| 15.1 | Alterations and additions to existing system only to suit new layout. Fire alarm installation to be as shown on drawing. Fire Detection and Alarm system to be in accordance with BS5839 part 1:2017. All cables or ducts passing through fire compartments to have intumescent fire sleeves or sealant to maintain fire resistance. |  |  |
| 15.2 | The main contractor is to allow attendance on the named sub-contractor. |  |  |
| 15.3 | Allow a for builders work in connection with the fire alarm system installation (BWIC). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 16.0 | Above Ground Drainage |  |  |
| 16.1 | ABOVE GROUND - Polypipe 110mm drain run from SVP/stub stack to below ground drainage. Link internal fittings as shown on the drawings. 32mm basin waste, 110 mm wc connection, 40 mm sink. Anti - siphon wastes. Provide access points to SVP above the connections. Agree all final waste runs on site with the contract administrator. SVP to open air/with durgo valve where necessary. Test the system for Building Control |  |  |
| 16.2 | Link internal fittings as shown on the drawings. 40 mm sink. Anti - siphon wastes. Provide access points to SVP above the connections. Agree all final waste runs on site with the architect. SVP to open air/with durgo valve where necessary. Test the system for Building Control. |  |  |
| 16.3 | Supply and fit 2 No. Ariston Andris Lux local hot water heating to the new kitchenette and W.C.’s to the warehouse. Hot water heater is to be positioned in the base unit in the kitchenette. |  |  |
| 16.4 | Determine on site the best option for adjusting existing drainage to suit the new works. |  |  |
| 16.5 | Allow for adjusting the existing stopcock and meter location to suit the new works. |  |  |
| 16.6 | Allow a provisional sum of £3000.00 for adjusting the existing internal manhole to suit the new floor finish and replacing with a sealed cover. |  |  |
| 16.7 | Allow a provisional sum of £4000.00 for adjusting the existing above ground drainage. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 17.0 | Electrical Heating – BY OTHERS |  |  |
| 17.1 | The main contractor is to allow attendance on the named sub-contractor. |  |  |
| 17.2 | Allow a for builders work in connection with the fire alarm system installation (BWIC). |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 18.0 | External Works |  |  |
| 18.1 | Supply and lay 225mm well compacted type 1 material sub-base, well compacted in 75mm layers; 225mm reinforced concrete hardstanding top layer with expansion joints (to concrete areas adjacent the existing roller shutter door). |  |  |
| 18.2 | Allow for 25mm Flexcell compressible fibre board with cold applied two part polysulphide mix incorporating resin and curing agent (contractors choice). |  |  |
| 18.3 | Supply and fit 2 No. Broxap Heavy Duty Economy bollards (allow for root fixing option to manufacturers details) as shown on the drawings. Allow for a traffic yellow finish and reflective band. Make good existing tarmac surface affected by the works. |  |  |
|  | CARRIED TO SUMAMRY | £ |  |
| 19.0 | Site Clear Up |  |  |
| 19.1 | The Contractor is to carry out a thorough builders’ clean prior to handover inc. vacuum of all internal floors and sweep-up of external working areas. |  |  |
|  | CARRIED TO SUMMARY | £ |  |
|  | SUMMARY |  |  |
|  | 1.0 General Conditions |  |  |
|  | 2.0 Demolition/ Asbestos Removal and |  |  |
|  | 3.0 Below Ground Drainage (Contractor’s Design) |  |  |
|  | 4.0 Carpentry |  |  |
|  | 5.0 Insulation |  |  |
|  | 6.0 Joinery |  |  |
|  | 7.0 Linings |  |  |
|  | 8.0 Sanitaryware |  |  |
|  | 9.0 Ironmongery |  |  |
|  | 10.0 AVCL/ Breather Membrane |  |  |
|  | 11.0 Floor Finishes |  |  |
|  | 12.0 Decoration |  |  |
|  | 13.0 External Render System |  |  |
|  | 13.0 General Fixtures |  |  |
|  | 14.0 Electrics (BY OTHERS) |  |  |
|  | 15.0 Fire Detection (BY OTHERS) |  |  |
|  | 16.0 Above Ground drainage (Contractor’s Design) |  |  |
|  | 17.0 Electric Heating (BY OTHERS) |  |  |
|  | 18.0 External Works |  |  |
|  | 19.0 Site Clear Up |  |  |
|  | PRELIMINARIES SUB TOTAL | £ |  |
|  | TENDER SUB TOTAL | £ |  |
|  | TENDER TOTAL SUM | £ |  |

Company Name

Company Address

Signed

Print Name

Date