Enhanced Reed management Literature Review

Written by Sam Ozaktanlar, March 2025

Nutrient Removal in Wetlands by Reed Plants

Wetland vegetation, particularly species like Phragmites australis (common reed), plays a pivotal role in nutrient removal and water purification. Nutrients are removed primarily by channelling water through plant beds, where bacterial activity-triggered by reed stalks delivering air to root zones via aerenchyma-facilitates nitrogen and phosphorus transformation. Key microbial processes involved include ammonification, nitrification, and denitrification for nitrogen, alongside chemical adsorption and biological transformation for phosphorus.

Role of Plants and Microorganisms

- Both plants and microbes are crucial for nitrogen removal.

- While direct plant uptake contributes, bacterial nitrification-denitrification accounts for more than 50% of nitrogen elimination in many wetlands.

Impact of Harvest Time on Nutrient Removal

Harvesting wetland biomass influences nutrient removal outcomes, ecological integrity, and management practices.

Summer Harvesting

- Peak nutrient content: Nutrients accumulate in reed stems by July/August.

- Higher removal efficiency: Summer harvesting removes more nitrogen (92 kg ha⁻¹ y⁻¹), phosphate (9 kg ha⁻¹ y⁻¹), and potassium (66 kg ha⁻¹ y⁻¹) from Phragmites reedbeds.

- Ecological drawbacks:

  - Disruption to nutrient resorption processes.

  - Damage to flora and fauna.

  - Increased sediment and pollutant resuspension.

  - Reduced plant vitality in subsequent years.

  - Higher transport and storage costs due to moisture.

Winter Harvesting

- Ecological compatibility: Lower disturbance, especially to breeding birds.

- Reduced nutrient removal: By winter, 45–75% of nutrients are translocated to rhizomes, leaving only 10–20% in above-ground biomass.

- Benefits:

  - Supports denitrification via retained warmth.

  - Preserves reed structure for oxygen supply.

  - Enhances reed stand vitality and reduces pests.

  - Allows practical harvesting on frozen terrain.

- Quantified removal: 20 kg N, 1 kg phosphate, and 8 kg K ha⁻¹ y⁻¹ extracted in a south Sweden study.

Nutrient Allocation and Recovery

Reeds actively resorb nutrients into belowground biomass prior to senescence, affecting post-harvest nutrient dynamics.

- Nutrient storage shifts seasonally: Roots become the primary storage organs in winter.

- Harvesting timing impacts root nutrient levels:

  - High nitrogen input leads to post-harvest nutrient depletion in roots.

  - Low nitrogen input sees increased root nutrient levels after mid-harvest.

- Excessive or ill-timed harvests can reduce rhizome reserves and suppress future growth and nitrogen removal performance.

- Understanding root nutrient regulation is key for winter ecosystem management.

Efficiency and Economic Considerations

Economic and ecological strategies must be balanced in wetland biomass harvesting.

- Annual winter harvesting improves nitrogen removal—especially if litter is managed to prevent nutrient leaching.

- Species productivity: Phragmites australis and Typha latifolia yield 10–30 t dry matter ha⁻¹ y⁻¹.

- Removal rates: Summer harvesting of Phragmites achieves up to 600 kg N and 60 kg P ha⁻¹.

- Market viability:

  - Thatching offers highest profitability.

  - Combustion bales rank second.

  - Summer-harvested biomass for biogas yields the lowest return.

- Payments for ecosystem services can offset low market values, promoting sustainable nutrient management.

Conclusion

Harvesting Phragmites biomass is an effective strategy for nutrient removal from wetlands. Although summer harvesting achieves higher nutrient extraction, winter harvesting offers ecological and practical benefits that support long-term ecosystem health. Optimal management should balance nutrient removal efficiency with environmental impacts and economic feasibility to maintain the functionality of reed-dominated wetlands.

Sources

Eller, F., Ehde, P.M., Oehmke, C., Ren, L., Brix, H., Sorrell, B.K., & Weisner, S.E.B. (2020). Biomethane Yield from Different European Phragmites australis Genotypes, Compared with Other Herbaceous Wetland Species Grown at Different Fertilization Regimes. Resources, 9(5), 57.

Findlay, S., Groffman, P., & Dye, S. (2003). Effects of Phragmites australis removal on marsh nutrient cycling. Wetlands Ecology and Management, 11, 157–165.

Geurts, J.J.M., Oehmke, C., Lambertini, C., Eller, F., Sorrell, B.K., Mandiola, S.R., Grootjans, A.P., Brix, H., Wichtmann, W., Lamers, L.P.M., & Fritz, C. (2020). Nutrient removal potential and biomass production by Phragmites australis and Typha latifolia on European rewetted peat and mineral soils. Science of the Total Environment, 747, 141102.

Hosoi, W., Kido, Y., Miki, M., & Sumida, M. (1998). Field examination on reed growth, harvest and regeneration for nutrient removal. Water Science and Technology, 38(1), 351-359.

Köbbing, J.F., Thevs, N., & Zerbe, S. (2013/14). The utilisation of reed (Phragmites australis): a review. Mires and Peat, 13(01), 1–14.

Kõiv-Vainik, M., Ostonen, I., Kanu-Oji, C.O., & Kasak, K. (2025). Assessment of nutrient storage and translocation in winter harvested Typha latifolia from free-water surface treatment wetland mitigating diffuse agricultural pollution. Science of the Total Environment, 962, 178424.

Sricoth, T., Meeinkuirt, W., Pichtel, J., Taeprayoon, P., & Saengwilai, P. (2018). Synergistic phytoremediation of wastewater by two aquatic plants (Typha angustifolia and Eichhornia crassipes) and potential as biomass fuel. Environmental Science and Pollution Research, 25(6), 5344–5358.

Sun, S., Ding, Y., Ding, S., Zheng, X., Wang, Z., Zhang, M., Yan, P., Gu, X., & He, S. (2024). Unraveling the synergistic promotion mechanism of Fe0 coupling phragmites australis biomass for nitrogen removal in coastal wetland: From low to moderate salinities. Environmental Research, 263, 120005.

Tanaka, T.S.T., Irbis, C., Kumagai, H., Wang, P., Li, K., & Inamura, T. (2017). Effect of Phragmites japonicus harvest frequency and timing on dry matter yield and nutritive value. Journal of Environmental Management, 187, 436–443.

Wang, J., Chen, G., Fu, Z., Qiao, H., & Liu, F. (2021). Assessing wetland nitrogen removal and reed (Phragmites australis) nutrient responses for the selection of optimal harvest time. Journal of Environmental Management, 280, 111783.

Wichmann, S. (2017). Commercial viability of paludiculture: A comparison of harvesting reeds for biogas production, direct combustion, and thatching. Ecological Engineering, 103, 497–505.

Enhanced Reed Management Scoping Report-rough calculations applied to Stodmarsh

Calculating the removal rate of total nitrogen from cutting x tonnes of biomass from the edge of a lake as well as consider what effect there is on nutrient balancing within the lake.

Questions to consider How under water does reed need to be for best chances of N uptake?

- What time window provides best uptake

Assumptions

- average uptake of phragmites species

- abstraction licence and full control of water level in NNR lake

- access to lakeside reedbed not an issue

- Lake will be re-profiled and time lag for recolonisation of reed discounted

To calculate the removal rate of total nitrogen (TN) from cutting biomass and understand its impact on nutrient balancing within the lake, you can follow these steps:

Steps to Calculate Total Nitrogen Removal Rate

- Determine the Dry Weight of Biomass:

  - Obtain the dry weight of the biomass per tonne. This is essential because nutrient content is typically measured in the dry weight of the plant material.= 2360 g/m²

    - (PDF) Temporal variations in the nitrogen content of Phragites australis (Cav.) Trin. ex Steud. from a shallow fertile lake

- Calculate Total Nitrogen Content in Biomass:

  - Find the average TN concentration in the dry biomass (usually given as a percentage or mg/kg).  10,150 mg/kg or 1.015%=median value in range derived from 2 literature sources

    - Silicon promotes biomass accumulation in Phragmites australis under waterlogged conditions in coastal wetland | Plant and Soil

    - (PDF) Temporal variations in the nitrogen content of Phragites australis (Cav.) Trin. ex Steud. from a shallow fertile lake

  - Multiply the dry weight of the biomass by the TN concentration to get the total nitrogen content per tonne of biomass. 23,954 mg/m².

- Estimate Total Biomass Harvested:

  - Calculate the total biomass harvested per hectare per year. This involves knowing the surface area of reeds that can be cut and the biomass yield per unit area.-In other words-in a given area, how much biomass is removed

  - surface area of reeds that can be cut =~3.6 hectares

  - biomass yield per unit area (hectare)=13 tonnes per hectare per year

    - factors needed to determine biomass

      - water level of reedbed

        - lake top water level=  1.15mAbove Ordance Datum-reeds approx. 1.3/1.4 mAOD

          - therefore waterlevel discounted

    - Nutrient removal through autumn harvest of Phragmites australis and Thypha latifolia shoots in relation to nutrient loading in a wetland system used for polishing sewage treatment plant effluent - York Research Database

    - RSPB reedbed management handbook

- Calculate Total Nitrogen Removed:

  - Multiply the total biomass harvested per hectare (13t/ha/yr) by the TN content per tonne ( to get the total nitrogen removed per hectare

Step 1: TN Content per Tonne of Biomass

- TN concentration = 10,150 mg/kg = 10.15 g/kg

- Since 1 tonne = 1,000 kg, the TN content per tonne is: 10.15×1000=10,150 g=10.15 kg10.15 \times 1000 = 10,150 \text{ g} = 10.15 \text{ kg}10.15×1000=10,150 g=10.15 kg (So, each tonne of biomass contains 10.15 kg of nitrogen.)

Step 2: TN Removed for 13 t/ha/y

- Biomass yield = 13 t/ha/y

- Total nitrogen removed per hectare per year: 10.15×13=131.95 kg N/ha/y10.15 \times 13 = 131.95 \text{ kg N/ha/y}10.15×13=131.95 kg N/ha/y

Final Answer:

Approximately 132 kg N/ha/y is removed through biomass harvest for a summer harvest however source where 13t dm/ha/yr was derived from mentions a 30%/50% reduction when done in winter- giving 92.87 kg N/ha/y or 66 kg N/ha/y respectively ("Nutrient removal potential and biomass production by Phragmites australis and Typha latifolia on European rewetted peat and mineral soils”)

FOR TRIAL PLOT OF 3.6 hectares USING MOST PRECAUTIONARY WINTER HARVEST RATE=237.6 kg N/year.

For autumn harvest autumn harvesting of Phragmites australis represents a balance between maximizing biomass yield and nutrient removal, ensuring sustainable regrowth, and obtaining biomass quality suitable for various applications like bioenergy. The specific timing within autumn can influence these factors, and it's often a preferred period for harvest compared to summer (to avoid impacting rhizome reserves) and winter (when biomass and nutrient content are lower).

Will there still be a net loss compared to nitrogen input into lake and lake concentration and legacy N in silt

as collards is same hydrological unit-need to measure N conc and volume of Collards as well?

TN conc (winep spot sample) 1.62mg/l, and APEM samples=mean 3.4mg/l

NNR lake volume 194,000m3 (APEM,2016)

NNR nitrogen content=659.6kg

FINAL RESULT IS ANNUAL N INPUT VS REMOVAL

| Harvest time | Removal rate (kg N/Ha/Yr) | Estimation of Harvestable Lakeside Reed (hectares) | Estimate removal rates for total harvestable reed (kg N/yr) | Average annual input (kg N/Yr) |
| --- | --- | --- | --- | --- |
| Summer | 132 | 19.75 | 2,607 | 253,027 |
| Winter | 92.87 |  | 1,834.2 |  |
| Winter precautionary | 66 |  | 1,303.5 |  |

Calculations for Phosphorous removal:

Concentration of TP (grams of TP per KG of reed biomass) - 1.0 - 1.5g/kg (conservative range based off literature)

1g/kg x 13,000kg = 13kg TP/Ha x 19.75ha =  256kg TP/yr

1.5g/kg x 13,000kg = 19.5kg TP/Ha x 19.75ha = 385kg TP/yr

Considerations for Nutrient Balancing

- Nutrient Cycling:

  - Removing biomass affects the nutrient cycling within the lake. Consider the role of reeds in nutrient uptake and release.

- Impact on Water Quality:

  - Assess how removing nitrogen affects the overall nutrient balance and water quality. This includes potential changes in algal blooms and aquatic plant growth.

- Reed Regrowth:

  - Consider the regrowth rate of reeds and their ability to continue absorbing nutrients in subsequent years.

- External Nutrient Inputs:

  - Account for other sources of nitrogen entering the lake, such as runoff, atmospheric deposition, and inflows from tributaries.

- Nutrient Removal Effectiveness:

  - Reed cutting in winter is an effective method for fighting against water eutrophication.

  - Havesting reed annually in winter can enhance wetland nitrogen removal compared with no harvesting

  - Multiple harvests of reed are more effective than single harvests in removing nitrogen and phosphorus

  - optimal reed harvest time is late wilting for wetland nitrogen removal

  - Harvesting during the peak of the growing season coincides with peak nutrient standing stocks and the highest nutrient concentration in the aboveground plant organs

Assessing wetland nitrogen removal and reed (Phragmites australis) nutrient responses for the selection of optimal harvest time

Junli Wang a,b, Guifa Chen a,b, Zishi Fu a,b, Hongxia Qiao a,b, Fuxing Liu a,b,*

Assessing wetland nitrogen removal and reed (Phragmites australis) nutrient responses for the selection of optimal harvest time

Junli Wang a,b, Guifa Chen a,b, Zishi Fu a,b, Hongxia Qiao a,b, Fuxing Liu a,b,*

Effect of Phragmites japonicus harvest frequency and timing on dry matter yield and nutritive value

Takashi S.T. Tanaka a, b, Chagan Irbis a, *, Hajime Kumagai b, Pengyun Wang c, Kunzhi Li a, Tatsuya Inamura b

Assessing wetland nitrogen removal and reed (Phragmites australis) nutrient responses for the selection of optimal harvest time

Junli Wang a,b, Guifa Chen a,b, Zishi Fu a,b, Hongxia Qiao a,b, Fuxing Liu a,b,*

Assessment of nutrient storage and translocation in winter harvested Typha latifolia from free-water surface treatment wetland mitigating diffuse agricultural pollution