# Contract Specification

Cash Handling Devices

Appendices:

- Appendix 1 – List of assets and their locations

- Appendix 2 – Secure Suite Equipment Layout

- Appendix 3 – Types of Company Administrators

- Appendix 4 – TfL Policies and Standards

- Appendix 5 – Incumbent Draft Handback Plan

- Appendix 6 – Faults and Resolutions

- Appendix 7 – Trend Report Period 4-25

- Appendix 8 – Trend Report Period 5-25

- Appendix 9 – Trend Report Period 6-25

- Appendix 10 – IPR London Underground

Appendix 11 – London Underground CHD movement

- Appendix 12 – Stations and Device Categories

- Appendix 13 – Parts and Consumables

- Appendix 14 – Incumbent TUPE Information

- Appendix 15 – Loss Prevention Standards

# Contents

Contents	2

1	Requirements Introduction	5

2	Device Specification	6

2.1	Cash Handling	6

2.1.1	Cash Counting	6

2.1.2	Secure Storage	7

2.1.3	Coin Storage and Dispensing	9

2.1.4	Banknote Storage	11

2.1.5	Banknotes and Coins	12

2.1.6	Recycling of Money	13

2.1.7	Transaction Receipts	14

2.1.8	Single User Operation	15

2.1.9	Ergonomics	15

2.2	Data / IT Interfaces	16

2.2.1	Data	18

2.2.2	Dispensing Coins & Banknotes	19

2.2.3	Barcodes	21

2.2.4	Triggers & Notifications	22

2.3	Technical Interfaces & Security Provisions	22

3	Generic Business Requirements	25

3.1	Access For Users	25

3.1.1	User Sign On	25

3.1.2	Role Based User Access	25

3.1.3	Access	32

3.1.4	Cash Collections	33

3.1.5	Audited Transactions	34

3.1.6	Capacity of Multi Users	34

3.1.7	Alarms	35

3.1.8	Ergonomics	35

3.2	Reporting	36

3.3	Human Factors	48

3.4	Location of Works	50

3.5	Hours of Work	51

3.6	Skills & Competencies	51

4	Service & Maintenance Schedule	52

4.1	Scope	52

4.2	Planned Preventative Maintenance	55

4.3	Testing	55

4.4	Materials & Consumables	56

4.5	Modifications to Equipment	57

4.6	Reactive Services	58

4.7	Condition Assessment	60

4.8	Site Access	60

4.9	Asset Management	61

4.10	Incident Management	63

4.11	Dashboards	64

4.12	Service Desk	65

4.13	Help Desk	68

4.14	Maintenance & Faults	69

5	Service & Maintenance Non-functional Requirements	73

5.1	Access and Security	73

5.2	Availability	80

5.3	Compliance	82

5.4	Integrity	82

5.5	Maintainability and Support	86

5.5.1	Incident and Problem Management	86

5.5.2	Change and Release Management	92

5.6	Performance	98

5.7	Reliability	98

5.7.1	Monitoring	100

5.8	Robustness	107

5.9	Scalability	111

5.10	Usability	112

5.11	Data / IT Interfaces	112

5.12	Technical Interfaces & Security Provisions	113

5.13	Training / Support	120

5.14	Physical, Environmental & Engineer Constraints	122

5.15	Account Management	128

6	Compliance	128

6.1	Compliance with TfL Policies and Standards	128

6.2	Information Governance	132

6.3	Cyber Security	140

6.3.1	Security Standards	140

6.3.2	Security Principles	143

6.3.3	Security Requirements	145

7	Glossary of Terms	157

# Requirements Introduction

This contract requires the contractor to maintain existing cash handling devices and supply new devices to replace those devices that are life expired. It is important that both existing and new devices[ are supported by a single back-office system for the purposes of reporting and performance monitoring.

Any reference to Cash Handling Devices within this document applies to both existing and newly installed devices unless otherwise stated.

For new devices, the Supplier shall deliver/install Equipment and provide Services compliant with the Contract to meet the cash handling requirements of the London Underground managed stations (for a list of proposed sites refer to ‘Stations and Device Categories’ which is supplied as Appendix 12 to the Specification. Please note, some of the stations have more than one ticket hall and the Supplier will need to maintain more than one cash management machine within the same station (e.g. King’s Cross /St. Pancras).

The Equipment shall be able to quickly, accurately and efficiently count and reissue money to authorised and audited individuals (through a simple sign on process). The Equipment shall store money securely and enable controlled and safe collection by designated Cash in Transit contractors.

The Supplier shall reliably deliver the Services throughout the Term as set out in this Specification and Schedule 8 (Supplier Performance).

For legacy and new equipment, the Supplier shall provide appropriate User Training and User Manuals as set out in the Contract with particular reference to Schedule 11.  Training will be delivered to designated Company trainers and Company Personnel as part of an implementation package over a set period and covering an agreed number of people (to be agreed as part of the Delivery Program). The Supplier shall provide onsite support as each cash management machine is delivered and help desk support for Company Users and Company Administrators as part of business-as-usual Services.

The Supplier will also be required to provide training to designated trainers from the Company’s Cash in Transit (CIT) Contractors.

# Device Specification

The following requirements’ tables contain general requirements, functional requirements, service requirements, technical requirements and constraints that the Supplier shall be compliant with in the Delivery of the Equipment and Services.

## Cash Handling

The cash handling functional requirements comprise the acceptance, rejection, counting, dispensing and storing requirements of the cash handling Services.

### Cash Counting

| Counting Cash |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-443 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices will count cash efficiently and accurate to 100% subject to coins/notes being validated Note: Any validated coin or note should match the amount deposited. Any coin or note that has not been validated should automatically be rejected and not included. |  |  |  |
| Rationale: |  |  |  |  |

| Scan a Container |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-473 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device can: - scan (or register) a container (bag) from which cash is to be deposited into the cash handling device - identify a bag to be filled with cash from the cash handling device , or to be used to top-up the cash management machine with change. |  |  |  |
| Rationale: |  |  |  |  |

### Secure Storage

| Security Storage Rating |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-446 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling devices have a security storage rating of at least Security Level 2 in accordance with the Loss Prevention Standard (LPS) LPS1175 as amended. |  |  |  |
| Rationale: |  |  |  |  |

| Secure storage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-447 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices securely store coins deposited that can not be accommodated within the specified hoppers. |  |  |  |
| Rationale: |  |  |  |  |

| Bank Note Storage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-448 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Bank notes are stored within the cash handling device in a uniquely numbered collectable container (e.g. pouch, transportation sack) for collection and shall be bar coded for reporting purposes |  |  |  |
| Rationale: |  |  |  |  |

| Stored Bank Notes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-449 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the total value of bank notes stored in the collectable container within the cash handling device contains a maximum of £30,000 |  |  |  |
| Rationale: |  |  |  |  |

| Secure storage - CAT A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-450 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Category A Cash Handling Devices have at least two sacks or an alternative method of storing deposited notes so that notes can be deposited even if one note container is full or becomes defective. |  |  |  |
| Rationale: |  |  |  |  |

| Secure storage - CAT B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-451 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Category B devices have at least one sack or an alternative method of storing deposited notes so that notes can be deposited even if one note container is full or becomes defective. |  |  |  |
| Rationale: |  |  |  |  |

### Coin Storage and Dispensing

| Coin storing and dispensing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-444 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices will accept, count, store and re-dispense 5 denominations of counted coins (£2, £1, 50p, 20p, 10p) either in a specified value/composition or as a free float within specified limits. |  |  |  |
| Rationale: |  |  |  |  |

| Coin storing and dispensing (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-445 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices will accept, count and store 1p, 2p, and 5p coins. It is possible that these coins are stored together. |  |  |  |
| Rationale: |  |  |  |  |

| Coin Storage - CAT A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-469 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the capacity for recyclable coin storage at Category A cash handling devices is a minimum of £8700. |  |  |  |
| Rationale: |  |  |  |  |

| Coin Storage - CAT B & D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-468 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the capacity for recyclable coin storage at Category B and Cat D (coin only) cash handling devices is a minimum of £7,700 |  |  |  |
| Rationale: |  |  |  |  |

| Coin Capacity - A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-472 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Category A cash handling devices can store for recycling (dispensing) a minimum capacity of: £2 = 2500 coins £1 = 2500 coins 50p = 1500 coins 20p = 1500 coins 10p = 1500 coins |  |  |  |
| Rationale: |  |  |  |  |

| Coin Capacity - B & D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-471 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Category B and Category D (coin only) cash handling devices can store for recycling (dispensing) a minimum capacity per coin type of: £2 = 2000 coins £1 = 2500 coins 50p = 1500 coins 20p = 1500 coins 10p = 1500 coins Note: The total mixed coin value should be a minimum of £5000 per device |  |  |  |
| Rationale: |  |  |  |  |

### Banknote Storage

| Bank Note Storage - CAT A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-467 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the capacity for bank note storage at Category A cash management machines is a minimum total value of £75000. |  |  |  |
| Rationale: |  |  |  |  |

| Bank Note Storage - CAT B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-466 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the capacity for bank note storage at Category B cash handling devices is a minimum total value of £45000 |  |  |  |
| Rationale: |  |  |  |  |

| Note Recycling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-470 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that where a cash handling device has a note re-cycler the Supplier shall provide a facility for an authorised User to transfer notes into the collectable container prior to a collection taking place. |  |  |  |
| Rationale: |  |  |  |  |

### Banknotes and Coins

| Acceptance and Storage of UK bank notes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-452 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices accept and store the following UK bank notes: Bank of England Bank of Scotland, Royal Bank of Scotland, Clydesdale Bank Bank of Ireland, First Trust Bank, Danske Bank, Ulster Bank with the exception of commemorative issue bank notes and the £100 bill issued by Scottish and Northern Irish banks. |  |  |  |
| Rationale: |  |  |  |  |

| Acceptance and Storage of UK Coinage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-453 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device shall accept and store the following UK coinage: Bank of England States of Jersey and Guernsey Isle of Man Gibraltar Falkland Islands All current coins issued by Scottish and Northern Irish Banks |  |  |  |
| Rationale: |  |  |  |  |

| New Designs for UK Currency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-454 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall adapt the Cash Handling Devices and Services, where necessary, to accept new designs of UK coins or banknotes. Note; The CHD's should accept whatever the Passenger Operated Machines accept (see requirements 10, 11, 29, 30) |  |  |  |
| Rationale: |  |  |  |  |

| Counterfeit Coins and Notes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-455 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure cash handling device reject all non-UK currency and counterfeit notes and coins. |  |  |  |
| Rationale: |  |  |  |  |

| Counterfeit Coins and Notes (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-456 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure cash handling device not count any rejected currency (notes or coins). |  |  |  |
| Rationale: |  |  |  |  |

### Recycling of Money

| Dispenses Coins |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-457 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device dispenses specified quantities / values of specified coins to authorised Users. |  |  |  |
| Rationale: |  |  |  |  |

| Dispenses Coins (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-458 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device dispense a specified value of coins directly into a separate designated tray (cash tray). |  |  |  |
| Rationale: |  |  |  |  |

### Transaction Receipts

| Receipts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-459 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device prints a receipt of total values collected by the security custodian broken down by denomination. |  |  |  |
| Rationale: |  |  |  |  |

| Receipts (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-460 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device prints a receipt for the User specifying the total values of coins and notes deposited into or dispensed out of the cash management machine. |  |  |  |
| Rationale: |  |  |  |  |

| Summary of Transactions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-461 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device prints a summary of transactions made by all users during a traffic day if required by a User. |  |  |  |
| Rationale: |  |  |  |  |

| CHD Contents (Amount and Value) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-462 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device prints and displays the total amount and value of cash in the machine and the amount of coins within each hopper, any in the overflow vault, within the note container and note unit. On the device display, the monetary contents shall be able to be shown in a graph format. |  |  |  |
| Rationale: |  |  |  |  |

### Single User Operation

| Accountability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-463 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device supports the principle of individual accountability and attribute all actions undertaken by a User to them. Note: When a user logs onto a CHD, they are then responsible for any actions that are preformed on the machine. |  |  |  |
| Rationale: |  |  |  |  |

### Ergonomics

| Accessing the CHD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-464 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device is easily accessed, ergonomically sound and able to be carried out within a potentially constrained environment. |  |  |  |
| Rationale: |  |  |  |  |

| Dispensing of Money |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-465 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure dispensing of money is easily accessible, ergonomically sound and not too low or awkward to remove the cash within a potentially constrained environment. |  |  |  |
| Rationale: |  |  |  |  |

## Data / IT Interfaces

| Remote Viewing of Device Status |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-498 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Authorised Company Administrators are able to view remotely (e.g. via the internet) the real time status of the Equipment, including hopper and/or tray levels (e.g. % full), total value of coins by denomination, total value of notes. |  |  |  |
| Rationale: |  |  |  |  |

| View Stored Values and Denominations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-499 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Users are able to check (on screen or via printed receipt) the total value of the various denominations stored in the cash management device (this transaction shall be captured and auditable). |  |  |  |
| Rationale: |  |  |  |  |

| Intuitive Screens |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-501 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment operational screens are intuitive (they do not require additional training, or require minimal training) and easily updateable if changes are required. |  |  |  |
| Rationale: |  |  |  |  |

| Automatic Setting of Clocks |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-513 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure all Equipment can automatically switch between GMT and BST and vice versa on designated dates, without the need for User or Company Administrator intervention. |  |  |  |
| Rationale: |  |  |  |  |

| Equipment Functions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-523 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment functions, controlled by the software, includes: deposit or, dispensing of cash, status / money levels (stored in the Equipment). |  |  |  |
| Rationale: |  |  |  |  |

| Data Connectivity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-588 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be able to both remotely connect (via 4G/5G) to maintain devices and also physically access (such as via a USB) some sites to maintain devices and collect data where remote connection is not possible. |  |  |  |
| Rationale: |  |  |  |  |

### Data

| Real time data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-494 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment provides live (near real time) data for each function of the Equipment (current coin and note levels / values) both at local station and at back office. |  |  |  |
| Rationale: |  |  |  |  |

| Historic Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-496 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment provides historic data records by each cash management machine (location), station, area, network, User, User group, type of transaction, date and time. Note: This is currently stored for 3 years |  |  |  |
| Rationale: |  |  |  |  |

| Sharing Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-497 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment can interface and share Data with major industry known reporting systems e.g. SAP, Boxi as limited examples. |  |  |  |
| Rationale: |  |  |  |  |

| Transaction and Status Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-521 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall develop and maintain a secure and auditable means to manually extract transaction and status Data from the cash handling device and transfer such Data to the back office systems for consolidated reporting. Such activities would only be undertaken during periods of extended communication Faults at stations and upon agreement with the Company. |  |  |  |
| Rationale: |  |  |  |  |

| Software versions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-520 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment cash transaction Data is available in the latest version of the Microsoft Office products in use, as stated in clause 342. |  |  |  |
| Rationale: |  |  |  |  |

### Dispensing Coins & Banknotes

| Presetting Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-502 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device can be 'preset' for quick dispensing of specific value of coins by denomination (for regular floating of ticket machines or filling bulk coin bags). Note: Ticket machines refer to the Passenger Operated Machine (POMs) |  |  |  |
| Rationale: |  |  |  |  |

| Preset to Dispense Notes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-503 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device can be preset to dispense specific value of notes by denomination if note recycling is provided. |  |  |  |
| Rationale: |  |  |  |  |

| Preset Maximum Dispense values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-504 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device can be preset to establish a maximum dispense and daily dispense limits per user / user type. |  |  |  |
| Rationale: |  |  |  |  |

| Set Up and Add new Dispensing Values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-505 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that all devices are able to add and set up new dispensing values remotely to each cash management machine as defined in Appendix 1. |  |  |  |
| Rationale: |  |  |  |  |

| Amend Dispensing Values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-506 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that all devices are able to amend dispensing values remotely per individual cash management machine as defined in Appendix 1. |  |  |  |
| Rationale: |  |  |  |  |

| Amend Dispensing Limits |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-507 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be able to amend dispensing limits remotely by cash management machine or across all cash management machines as defined in Appendix 1. |  |  |  |
| Rationale: |  |  |  |  |

| Amend Cash Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-508 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be able to amend cash management machines dispensing limits remotely by User type as defined in Appendix 1. |  |  |  |
| Rationale: |  |  |  |  |

### Barcodes

| Barcode Scanner |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-526 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the each cash handling device has an interleaved 2/5 barcode scanner either interfaced to the cash management machine or part of the cash management machine. |  |  |  |
| Rationale: |  |  |  |  |

| Read and Record Coin Bags |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-527 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device can read and record bar coded coin bags with the ability of logging amounts against the scanned and recorded bags. |  |  |  |
| Rationale: |  |  |  |  |

| Recoginse Coded Bages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-528 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment can recognise previously record bar coded bags (and contents identified). |  |  |  |
| Rationale: |  |  |  |  |

### Triggers & Notifications

| Trigger Notifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-495 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment sets cash levels (coins & notes) to trigger notifications to facilitate cash collection. |  |  |  |
| Rationale: |  |  |  |  |

| Automatic Notifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-525 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the automatic notifications are able to be configured to provide an alert when set capacity thresholds are reached on an individual cash handling device to potentially prompt a cash collection, or User action. |  |  |  |
| Rationale: |  |  |  |  |

## Technical Interfaces & Security Provisions

The following business requirements set out the technical and security provisions that the Supplier shall comply with for the Equipment and Services

| Least Privilege Access Principle |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-631 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Services operate on a Least Privilege Access principle. |  |  |  |
| Rationale: |  |  |  |  |

| Networking Modes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-636 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment shall support 802.3 Ethernet and 802.11 Wi-Fi based IP networking in DHCP (static and dynamic) and non-DHCP modes. |  |  |  |
| Rationale: |  |  |  |  |

| Unauthorised Connections |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-637 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier Equipment shall not accept unauthorised connections to its technical interfaces. Attempts of this type of access shall be recorded as a security event with a corresponding alarm condition reported. |  |  |  |
| Rationale: |  |  |  |  |

| End to End Connectivity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-638 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be responsible for providing the end to end network connectivity for the Services. |  |  |  |
| Rationale: |  |  |  |  |

| Near Real Time Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-642 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure transactional data captured by the Equipment is centrally reported in near real time. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting Network Health Statistics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-644 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment shall periodically report network health statistics relating to its integrity and general performance in the delivery of Services to a central location for Supplier monitoring and Company assurance of the Services. |  |  |  |
| Rationale: |  |  |  |  |

| Configuration of CHD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-649 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash management machine is able to be configured to operate standalone or with connection to its centralised server environment. |  |  |  |
| Rationale: |  |  |  |  |

| Central Time Service |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-650 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment shall use a central time service to synchronise all communications between Equipment |  |  |  |
| Rationale: |  |  |  |  |

| PC Component - Latest patching |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-660 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that the PCs component within the CHDs are maintained with the latest software patching |  |  |  |
| Rationale: |  |  |  |  |

# Generic Business Requirements

## Access For Users

The users’ access functional requirements comprise the security arrangements, logging on / off requirements, permission types and capacity range of the users of the cash management Equipment and Services

### User Sign On

| Magnetic Ticketing System Identity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-474 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device can read and accept a current London Underground issued magnetic Ticketing System Identity (TSID) swipe card and 4 digit PIN. |  |  |  |
| Rationale: |  |  |  |  |

### Role Based User Access

| Role Based Permissions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-485 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment and Services facilitate different User permission levels (e.g. User and Company Administrators) for carrying out specific functions. The adding or removal of such Users and Company Administrators will be undertaken by the Supplier once authorised by the Company. |  |  |  |
| Rationale: |  |  |  |  |

| Partioned Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-486 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that access to data can be partitioned so that certain users only have access to the relevant / appropriate data. |  |  |  |
| Rationale: |  |  |  |  |

| Additional Users |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-487 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that TfL Administrators can add additional users as required |  |  |  |
| Rationale: |  |  |  |  |

| Super User Account |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-665 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that an Authorised user (Super User) can reactivate and disabled back office account |  |  |  |
| Rationale: |  |  |  |  |

| Adding and Removing Users |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-488 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure Authorised Company Administrators can add/remove Users to all cash handling devices remotely and such changes shall take effect prior to the start of the next Operational Day. |  |  |  |
| Rationale: |  |  |  |  |

| Grouping Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-489 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling devices are configurable into groups or areas for reporting purposes only. |  |  |  |
| Rationale: |  |  |  |  |

| Setting Permissions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-490 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the TfL Administrators are able to set or change the operational permission levels of authorised Users on an individual cash handling device remotely. Such changes when applied by authorised TfL Personnel will take effect prior to the start of the next Operational Day. |  |  |  |
| Rationale: |  |  |  |  |

| Configure Authorised User Roles |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-187 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall enable an Authorised User to Configure Authorised User Roles. This shall include but is not limited to: - Authorised User roles (LUL Ticketing and Revenue Team) |  |  |  |
| Rationale: | N/A |  |  |  |

| Manually Control User Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-188 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall provide the capability for an Authorised User with sufficient User Permissions to manually control Authorised User Access. This includes but is not limited to: - Disabling an Authorised User's login and access rights when they are no longer classed as an Authorised User - Re-enabling disabled logins as and when required |  |  |  |
| Rationale: | N/A |  |  |  |

| Control User Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-186 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall provide the capability to control the User Access of Authorised Users based on role based permissions (pre-defined by TfL). This includes but is not limited to: - Disabling an Authorised User's login and access rights when they are no longer classed as an Authorised User - Re-enabling disabled logins if required |  |  |  |
| Rationale: | N/A |  |  |  |

| User Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-189 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall have a number of different user types with specific read/write, edit and read only functionality. This must be configurable and the Authority (TfL) reserves the right to change this at any time. |  |  |  |
| Rationale: |  |  |  |  |

| User Access Configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-190 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall provide the capability to Configure back office Authorised User Roles and User Permissions according to but not limited to: - Viewing Records, associated Documentation and other Data types (according to data security restrictions) - Permission templates / security profiles - Download Authorised User reports |  |  |  |
| Rationale: | N/A |  |  |  |

| User Access Permissions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-191 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Authority is able to request new user types at any time and the Supplier will update the permissions template / matrix accordingly Note: the Supplier will have an Access Control Process agreed with the Authority |  |  |  |
| Rationale: |  |  |  |  |

| User Access Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-192 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the LUL Ticketing and Revenue Team can create and run user access reports within their allocated security permissions |  |  |  |
| Rationale: |  |  |  |  |

| User Access Management & Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-193 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure authorised LUL Ticketing and Revenue Team have access to lists of users, their job roles and permissions and have the ability to create amend and disable access (user access management) within the system, including third party users, |  |  |  |
| Rationale: |  |  |  |  |

| User log on |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-194 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the user is able to log on to the system subject to their security profile settings |  |  |  |
| Rationale: |  |  |  |  |

| User credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-195 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the user is given credentials to log onto the system. |  |  |  |
| Rationale: |  |  |  |  |

| User Permissions - Access Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-196 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the user can access all stored product information in accordance with their user security permissions |  |  |  |
| Rationale: |  |  |  |  |

| Reflect Changes to User(s) Profile |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-197 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure that any security profile updates and changes will be reflected to the Authorised User and group affected immediately |  |  |  |
| Rationale: | N/A |  |  |  |

| Reset Credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-206 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall enable an Authorised User to reset a users credentials using one or more verified details including but not limited to their email address. Reset of User credentials shall include but not be limited to: - reset password - forgotten username |  |  |  |
| Rationale: | N/A |  |  |  |

| Add / Remove Company Administrators |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-543 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall add/remove Company Administrators and their access provisions to the Services within 24 hours once such changes have been instructed by the Contract Manager or delegate. |  |  |  |
| Rationale: |  |  |  |  |

| Access to Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-545 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Authorised Company Administrators are able to access the Services throughout the Operating Day (starts 04:30 and ends at 04:29 – every day excluding Christmas day). |  |  |  |
| Rationale: |  |  |  |  |

### Access

| Preventing User Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-475 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure user access can be prevented from areas of the cash handling device where coins and notes are stored. |  |  |  |
| Rationale: |  |  |  |  |

| Restricting Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-476 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the access to the note containers awaiting collection are restricted to the Cash in Transit(CIT) Contractor and cash handling device Supplier Personnel |  |  |  |
| Rationale: |  |  |  |  |

| Engineering Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-477 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Engineering access shall be gained via a uniquely recognisable and logged ID. |  |  |  |
| Rationale: |  |  |  |  |

| Unauthorised Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-478 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is secured with a locking mechanism to prevent unauthorised access to all parts of the Equipment where cash is stored. |  |  |  |
| Rationale: |  |  |  |  |

### Cash Collections

| Cash in Transit (CIT) Contractor |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-479 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device will be operated and cash collections made by the Cash in Transit (CIT) Contractor whenever required and without station staff intervention. |  |  |  |
| Rationale: |  |  |  |  |

| CIT - Access to Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-480 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Cash in Transit (CIT) Contractor(s) can log onto the cash handling device using one or multiple PINs to collect notes and with an auditable transaction record (the CIT Contractor shall not need to use keys). |  |  |  |
| Rationale: |  |  |  |  |

| Individual PIN for CIT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-481 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Cash in Transit Contractor can log on to the cash handling device using individual PINs provided by the Supplier to the Company's authorised Cash in Transit (CIT)Contractor. |  |  |  |
| Rationale: |  |  |  |  |

### Audited Transactions

| Auditable Records |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-482 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling devices capture auditable records of all individual users and transactions undertaken on the specific cash management machine. |  |  |  |
| Rationale: |  |  |  |  |

### Capacity of Multi Users

| Performance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-483 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure each individual cash handling device shall be able to handle the number of potential different London Underground User ID's and PINs assigned to each cash management machine (up to 9000 authorised users) without affecting the performance of the Equipment or Services. . |  |  |  |
| Rationale: |  |  |  |  |

| Performance (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-484 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Across the network , estate of devices, the Supplier shall ensure that the Equipment and Services operate without Fault or degradation during periods when multiple concurrent Users are using the Equipment and Services concurrently. |  |  |  |
| Rationale: |  |  |  |  |

### Alarms

| Reporting - Attempts to Access Device |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-492 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device reports unauthorised attempts to access it showing when the event occurred, and which part of the cash management machine was being tampered with. Such events will be logged by the Supplier in near real time and escalated to the Company. |  |  |  |
| Rationale: |  |  |  |  |

| Alarms |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-493 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash handling device has a duress alarm facility fitted, which shall provide an alarm to the Company in near real time. |  |  |  |
| Rationale: |  |  |  |  |

### Ergonomics

| Ergonomics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-491 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure logging onto the cash handling device is ergonomically easy (clear and user-friendly), particularly given the constraints of the working environment in which the Equipment is located. |  |  |  |
| Rationale: |  |  |  |  |

## Reporting

| Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-555 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall be responsible for the delivery of reports and reporting Data to the Company as defined in the Contract. |  |  |  |
| Rationale: |  |  |  |  |

| Recording & Reporting Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-515 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the all Equipment transaction and status Data is recorded and able to be reported by type of activity (deposit, dispense, cash collection and Fault type) |  |  |  |
| Rationale: |  |  |  |  |

| Recording & Reporting Permissions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-516 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the all Equipment permissions and User/ setting changes shall be recorded and able to be reported upon |  |  |  |
| Rationale: |  |  |  |  |

| Recording & Reporting Transactions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-517 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the all Equipment transactions are recorded and reported by each user. |  |  |  |
| Rationale: |  |  |  |  |

| Recording & Reporting Transactions (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-518 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the all Equipment transactions are recorded and reported by each cash handling device , station, area or accounting unit. |  |  |  |
| Rationale: |  |  |  |  |

| Recording & Reporting Transactions (3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-519 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Equipment and Services accurately capture and are able to report upon as a minimum the following details: - Station ID - Location - User - Transaction type - Deposit – coins - Deposit – notes - Withdrawal – coins - Withdrawal – notes - Cash in Transit (CIT) Contractor collection - User finger tip interventions - Engineer Interventions and Faults reported - Cash movement / transfer from recycler to sack - Transaction date - Value of the transaction - Bag/seal ID for Cash in Transit(CIT) Contractor collections - By each accounting Unit, traffic day opening balance, sum of transactions IN, Sum or transactions OUT, traffic day closing balance. - For each accounting unit, reconciliation of opening to closing balance, detailing all transactions. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting against Data Fields |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-664 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that the solution can report against any data field. |  |  |  |
| Rationale: |  |  |  |  |

| Locally Print Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-522 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment is able to locally print reports of events by day or individual User. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting Company Administrator Requests |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-544 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall report to the Company each Accounting Period all Company Administrator requests submitted and when changes have been affected. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting Transactions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-524 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment reports all transactions by cash management machine (or location reference), station, individual Users, User group, date, time. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting Non Triggered Events |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-531 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall report a list of stations/devices which did not trigger the event to send daily transaction histories for the previous traffic day to the central system at 08.30 every day, to include the auto-generated Daily Transaction Histories Report. This is for all transactions between 04:30 and 04:29 for the previous traffic day. |  |  |  |
| Rationale: |  |  |  |  |

| Download Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-532 |  | Priority: Should have | Status: Final | Version: v4 |
| Description: | The Supplier shall provide the capability to download 'Service Calls' Histories for individual devices in Excel format. |  |  |  |
| Rationale: |  |  |  |  |

| Report Open Daily Jobs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-533 |  | Priority: Could have | Status: Final | Version: v4 |
| Description: | The Supplier shall report a daily list of open jobs, generated at the same time as the Daily Transactions Report. The report will include the date and time of the job reported and the fault reported. |  |  |  |
| Rationale: |  |  |  |  |

| Sack Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-534 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | On the back office portal, the Supplier shall show the sack number of where the cash is moved to. |  |  |  |
| Rationale: |  |  |  |  |

| Removal of Cash |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-535 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | On the back office portal, The Supplier shall show when the cash has been removed by CIT or the Engineer and a record of the sack number. Note: If the CIT Collection reports show the Bag ID (Sack no), this would be a MUST HAVE REQUIREMENT. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting by Station |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-536 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide a daily report that indicates devices by station that have been offline for more than 24 hours, in order to make a preliminary check with the station that the modem is plugged in/switched on etc. before sending an engineer to site. |  |  |  |
| Rationale: |  |  |  |  |

| Manual Report Replacement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-537 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | If a data feed fails, The Supplier shall provide a manual report replacement within 48 hours of being notified of the data feed failure. |  |  |  |
| Rationale: |  |  |  |  |

| Configurable Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-538 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the devices are configurable to ensure that the addition/amendment/deletion of accounting units (reporting field) can be identified and reported on. |  |  |  |
| Rationale: |  |  |  |  |

| Operation Day |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-509 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Operational Day for the Services at the Start Date is: Start of day 04:30am End of day times 04:29am. every day excluding Christmas day |  |  |  |
| Rationale: |  |  |  |  |

| Configurable Times |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-510 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that the specified start / end of day times across all Equipment, is configurable by the Supplier when such changes are authorised by the Company. |  |  |  |
| Rationale: |  |  |  |  |

| End of Day Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-511 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure each cash handling device shall generate an end of day report detailing total cash held within each cash management machine and within each coin hopper, note unit and any vaults for miscellaneous coins. |  |  |  |
| Rationale: |  |  |  |  |

| Printing Local Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-512 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure each cash handling device shall be able to print a local copy of the machine contents at the end of day. |  |  |  |
| Rationale: |  |  |  |  |

| Monthly Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-500 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Company Administrators are able to view remotely a report of the current day's transactions (e.g. money in, money out, Users). |  |  |  |
| Rationale: |  |  |  |  |

| Reporting (Date / Time) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-514 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment can accurately report the correct date and time to within one (1) minute throughout the Operational Day. The time clocks in each cash handling device shall operate with the Network Time Protocol (NTP) or equivalent to ensure that each cash management machine time is synchronised with the Back Office for accurate date and time reporting |  |  |  |
| Rationale: |  |  |  |  |

| Complete & Accurate Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-554 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure all status and transactional Data is transferred to the back office and processed as quickly as possible for the provision of complete and accurate reporting, the Data completeness targets are defined in Schedule 8 (Supplier Performance). |  |  |  |
| Rationale: |  |  |  |  |

| Availability Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-580 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide availability reports of Services and/or Equipment each Accounting Period: By each contracted Service each Accounting Period By each item of Equipment By each LU Station The Supply |  |  |  |
| Rationale: |  |  |  |  |

| Service Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-581 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall report upon all contracted outputs, KPIs and SDI Performance Criteria as a minimum each Accounting Period unless specified to the contrary by the Company. Please note this excludes the Supplier provided dashboards which will present information in near real time. |  |  |  |
| Rationale: |  |  |  |  |

| Report Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-584 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall produce all reports in compatibility with TfL's latest version of Microsoft Office Suite and PDF. The Supplier must align compatibility requirements with TfL as necessary for the duration of the contract. |  |  |  |
| Rationale: |  |  |  |  |

| Maintenance Service Progress Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-578 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall prepare and deliver to the Company a Maintenance Services progress report every Accounting Period as part of their payment application. Such report will provide all necessary information for the Company to assure the contracted outputs have been delivered and process the Suppliers payment application. Such report will be made available to the Company within 5 business days of the end of the Accounting Period. |  |  |  |
| Rationale: |  |  |  |  |

| Develop & Maintain Management Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-582 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop and maintain suitable management reports that detail all contracted outputs and management escalations. Such reports shall be provided to the Company as requested, but as a minimum once each Accounting Period to support the Contract progress meeting and the processing of Supplier Application for Payment. |  |  |  |
| Rationale: |  |  |  |  |

| Service Trend Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-583 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall maintain and deliver to the Company each Accounting Period service trend reports for agreement and discussion with the Supplier at the Contract progress meeting. Limited examples of the type of information likely to be provided include: - Number of Faults - By cash management machine - By Equipment type - By service - By component part - Repeat Faults for each item of Equipment - Fault repair activities - Average repair times - Component parts replaced - Renewed or returned to workshop for repair - MCBF for each Equipment type averaged across the Supply - Identify any Equipment falling below an agreed operational base line MCBF threshold - Refused access to LU Stations and reasons - Equipment usage Data |  |  |  |
| Rationale: |  |  |  |  |

| Extract report data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-209 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | Depending on User access Profile, the Supplier shall allow Users to run reports and extract the report output Note: Any data extracted must be carefully managed, held securely, access controlled and kept only in line with defined retention rules. |  |  |  |
| Rationale: |  |  |  |  |

| Reporting features |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-211 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | The Supplier shall ensure the system has the following reporting functionality, which must be capable of using an appropriate tool (i.e. PowerBI) to be agreed with TfL This functionality is to include as a minimum: - The creation of reports - Reusing of reports - The sharing of reports - The editing of reports - The saving of reports - The copying of reports Note: The Solution will provide an interface which enables enhanced reporting to be developed over time |  |  |  |
| Rationale: |  |  |  |  |

| Reports export |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-212 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall export data in a suitable format to be agreed with TfL |  |  |  |
| Rationale: |  |  |  |  |

| Report Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-213 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | The Supplier shall ensure that all reports (existing and future) can be exported by the user in PDF, CSV and Excel formats |  |  |  |
| Rationale: |  |  |  |  |

| Adhoc reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-214 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | The Supplier shall be capable of providing reports on an adhoc basis, as requested by the user |  |  |  |
| Rationale: |  |  |  |  |

| Scheduled Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-215 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall be capable of scheduling reports as required by the User, these reports could be scheduled, but not limited to: - Daily - Weekly - 4 Weekly |  |  |  |
| Rationale: |  |  |  |  |

## Human Factors

| Access to Device |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-600 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure depositing money into the cash management machine is easily accessed, and able to be carried out within a potentially constrained environment. |  |  |  |
| Rationale: |  |  |  |  |

| Depositing Money into CHD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-601 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure depositing money into the cash management machine should minimise the need for uncomfortable postures |  |  |  |
| Rationale: |  |  |  |  |

| Handling of Heavy Items |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-602 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | The Supplier shall ensure depositing money into the cash management machine does not require manual handling of heavy items which are non compliant with Health and Safety guidance relating to manual handling |  |  |  |
| Rationale: |  |  |  |  |

| Dispensing Money from CHD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-603 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure dispensing of money from the cash management machine needs to be easily accessible within a potentially constrained environment |  |  |  |
| Rationale: |  |  |  |  |

| Dispensing Money from CHD (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-604 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure dispensing money from the cash management machine should minimise the need for uncomfortable postures |  |  |  |
| Rationale: |  |  |  |  |

| Dispensing Money from CHD (3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-605 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure dispensing money from the cash management machine should not require manual handling of heavy items which are non compliant with Health and Safety guidance relating to manual handling. |  |  |  |
| Rationale: |  |  |  |  |

| Intuitive Interfaces |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-606 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all Graphical User Interfaces (GUI) of cash management machines (such as touch screens) or keyboard and mouse operated screens, are intuitive and not unnecessarily complex to use |  |  |  |
| Rationale: |  |  |  |  |

| Accessible Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-607 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all touch screen and keyboard and mouse driven GUIs are easily accessible for the 5th percentile female to 95th percentile male in height |  |  |  |
| Rationale: |  |  |  |  |

## Location of Works

| Locations of Works |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-686 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Company may order and request resources detailed above to work at locations across the London Underground network |  |  |  |
| Rationale: |  |  |  |  |

## Hours of Work

| Access to site (hours) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-687 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Access will be obtained in accordance with the arrangements set out in Clause 13 of the Terms and Conditions of Contract and compliance with these arrangements is mandatory. |  |  |  |
| Rationale: |  |  |  |  |

## Skills & Competencies

| Demonstrate Experience |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-695 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure Supervisors and Service Engineers can demonstrate experience working with cash management equipment and/or satisfactory completion of a recognised training course. A competency certificate will then be issued by the Supplier. |  |  |  |
| Rationale: |  |  |  |  |

| Mandatory Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-696 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Specific training requirements for working on the Company’s premises are set out in Clause 18 of the Terms and Conditions of Contract and Schedule 11, Training. The Supplier shall ensure compliance with these arrangements |  |  |  |
| Rationale: |  |  |  |  |

# Service & Maintenance Schedule

The equipment to be maintained will be the Cash Management Equipment supplied and installed in accordance with Appendix 1 to this Specification, and any added by means of Variation.

An Asset List shall be agreed with the Supplier which will detail the final type, numbers and installation locations of the equipment. The Asset List will be included as Appendix 1 to this Specification.  An updated Asset List shall be provided annually on each anniversary of the Contract Start Date, and on other dates if requested by the Company.

The Supplier is responsible for maintaining, updating and repairing the Equipment as described in the asset list.

## Scope

| Scope |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-666 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall: - maintain existing cash handling devices and associated PCs as defined in Appendix 1 - supply new devices to replace those devices that are life expired. It is important that both existing and new devices are supported by a single back-office system for the purposes of reporting and performance monitoring. Any reference to Cash Handling Devices within this document applies to both existing and newly installed devices unless otherwise stated. |  |  |  |
| Rationale: |  |  |  |  |

| Design and Operating Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-667 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall maintain the equipment such that the equipment performs in accordance with all design and operating parameters defined by the Supplier in their supply and installation proposal. |  |  |  |
| Rationale: |  |  |  |  |

| Fully Inclusive Service |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-668 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure maintenance of the cash management equipment is provided on a fully inclusive basis including related no fault found, 3rd party damage, operator error or misuse |  |  |  |
| Rationale: |  |  |  |  |

| Supplier Responsibilities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-669 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be responsible for all such requirements, including but not limited to the supply of all labour (including related no fault found visits), overheads, consumables, materials, access equipment and plant required to undertake the required tasks. |  |  |  |
| Rationale: |  |  |  |  |

| Stock Levels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-670 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall hold adequate stock to meet Reactive Services needs (and any fault reduction and preventative maintenance initiatives), in order to meet the required performance and minimise unavailability of the systems. |  |  |  |
| Rationale: |  |  |  |  |

| New Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-733 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | For new devices, the Supplier shall deliver/install Equipment and provide Services compliant with the Contract to meet the cash handling requirements of the London Underground managed stations (for a list of proposed sites refer to ‘Stations and Device categories’ which is supplied as Appendix 12 to this Specification). Note: some of the stations have more than one ticket hall and the Supplier will need to maintain more than one cash management machine within the same station (e.g. King’s Cross /St. Pancras) |  |  |  |
| Rationale: |  |  |  |  |

## Planned Preventative Maintenance

| Planned Preventative Maintenance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-671 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide proposals for undertaking planned preventative maintenance (PPM) activities such that the equipment performs in accordance with the scope defined in requirement CHD-REQ-666 A PPM activity plan will be provided by the Supplier in accordance with the requirements in Schedule 2, Contract Programme. |  |  |  |
| Rationale: |  |  |  |  |

## Testing

| Testing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-672 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall carry out routine testing of the equipment, and testing on completion of maintenance work, including portable electrical appliance testing and any other testing required in accordance with the Company’s Standards. |  |  |  |
| Rationale: |  |  |  |  |

## Materials & Consumables

| Materials and Consumables |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-673 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall hold sufficient stocks of materials, spare parts replacement, components, tools, specialist equipment, temporary plant and consumables necessary to ensure the systems are maintained at the specified design and operating levels. This will be a minimum of 3 months of these stocks at a UK location. |  |  |  |
| Rationale: |  |  |  |  |

| Stock / Spares Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-674 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall maintain a stock/spares management system including but not limited to: proposed minimum stock/spares levels (both central store and van stock); stock/spares replenishment procedure; service level arrangements with sub-Suppliers; lead times for non stock items. |  |  |  |
| Rationale: |  |  |  |  |

| Consumable Items |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-675 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide all consumable items as part of the services which shall include cash pouches, transportation sacks for the collection of bank notes, receipt rolls for printing transaction receipts and any other consumable items required for the day to day operation of the machines as required by the manufacturer’s specification within 7 days of an order being placed |  |  |  |
| Rationale: |  |  |  |  |

| Stock Levels (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-676 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide sufficient stocks of materials, spare parts, and replacement components to maintain the systems at the specified design and operating levels for a period of one year after the end of the Contract. The level of such stocks shall be proposed as part of the Contract Handback Plan and agreed with the Company |  |  |  |
| Rationale: |  |  |  |  |

## Modifications to Equipment

| Proposed Changes to Software |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-677 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Where the Supplier is proposing changes to Software on Equipment on Company premises, this shall be notified via the Company’s Change and Release process. Any software deployment should follow the requirements set out. |  |  |  |
| Rationale: |  |  |  |  |

| Remedial Works |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-678 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Following any remedial works or minor work which may be required as part of the Services, the Supplier shall make good with minimum disturbance and consideration to any Railway Operating Equipment and the architectural decoration of the site. |  |  |  |
| Rationale: |  |  |  |  |

| Equipment Stored off Site |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-679 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Company may require Equipment to be removed from site and then stored away from their premises for a period of time, to be notified by Variation. Where this occurs, the Supplier shall ensure such storage will be in suitable premises such that Equipment is not damaged. Should damage take place, the Supplier must make good such damage at its own expense. |  |  |  |
| Rationale: |  |  |  |  |

## Reactive Services

| Reactive Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-680 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide Reactive Services 24 hours a day, 7 days a week, every day of the year, excluding Christmas Day |  |  |  |
| Rationale: |  |  |  |  |

| End to End Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-681 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Wherever possible, end to end monitoring by the Supplier shall take place such that Faults are identified and logged for rectification. Prior to Service Commencement, the Supplier and Company shall agree where this applies. |  |  |  |
| Rationale: |  |  |  |  |

| Service Desk |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-682 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | All Faults shall be notified by the Company to the Supplier. For this purpose the Supplier shall provide a Service Desk, with suitably qualified personnel and all communication shall be in English. Note: See CHD-FLD-108 for specific Service Desk requirements |  |  |  |
| Rationale: |  |  |  |  |

| Fault Rectification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-683 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | All Faults shall be rectified. The Supplier shall respond to and rectify Faults in accordance with Schedule 8, Supplier Performance. |  |  |  |
| Rationale: |  |  |  |  |

| Initial Visit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-684 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that where Faults that cannot be rectified on the initial visit must be left by the Supplier in a safe condition. |  |  |  |
| Rationale: |  |  |  |  |

## Condition Assessment

| Condition Assessment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-685 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall carry out a full condition assessment on each asset at least once a year and provide the results of this assessment to the Company |  |  |  |
| Rationale: |  |  |  |  |

## Site Access

| Permits to work |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-539 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure their Personnel hold the requisite access permits necessary to work on Company Premises; as a minimum each person shall hold a valid Station Access, Fire Awareness Permits, Sentinel Cards and comply with Company Standards |  |  |  |
| Rationale: |  |  |  |  |

| Approriate Documentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-540 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure Personnel attending the Companies premises carry with them appropriate method statements, risk assessment and COSHH sheets appropriate for the work planned in compliance with QUENSH |  |  |  |
| Rationale: |  |  |  |  |

| Planned Preventative Maintenance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-541 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all Planned and preventative maintenance access requests are sent to the Company as a minimum 4 weeks in advance of such access requirements. |  |  |  |
| Rationale: |  |  |  |  |

| Appropriate Credentials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-542 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the new Authorised Personnel have the appropriate credentials and training in order to carry out activities on TfL premises |  |  |  |
| Rationale: |  |  |  |  |

## Asset Management

| Company Asset Management Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-546 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall provide Company Asset management reports to the Company every three (3) Accounting Periods (unless otherwise requested by the Company) detailing all Company Assets and Equipment and indicating where a change has occurred. The type of information requested include but is not limited to: - Equipment installed - Changes to said Equipment - Serialised components or parts changed - Asset life cycle predictions of the Equipment and serialised component parts and rationale - Spare Parts held as stock - Consumable parts stock holding - Back office components - Usage or capacity concerns - Any devices held in storage |  |  |  |
| Rationale: |  |  |  |  |

| Company Asset List |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-547 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall maintain an accurate Company Asset list of all Company Assets and Equipment, including serialised components and consumable parts therein throughout the Term of the Contract. |  |  |  |
| Rationale: |  |  |  |  |

| Maintaining Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-548 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall be responsible for maintaining the Equipment to ensure that the Company discharges its statutory responsibilities for H&S and Mandatory or Category 1 and 2 Standards and provide suitable evidence to the Company at least annually for assurance purposes. |  |  |  |
| Rationale: |  |  |  |  |

| Company Asset Plans |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-549 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide to the Company as a minimum every three (3) Accounting Periods Company Asset plans which detail the reliability of the Equipment and Company Assets and where in the Asset life cycle each Company Asset and components are with the supporting rationale. Information will include but not be limited to remaining useful life and obsolescence issues for each Company Asset with the Suppliers proposals to remedy such issues. |  |  |  |
| Rationale: |  |  |  |  |

## Incident Management

| Incident Management Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-573 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The incident management data is owned by the Company and The Supplier shall ensure such information available to the Company when requested. |  |  |  |
| Rationale: |  |  |  |  |

| Major Incident List |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-574 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall agree a full major incident list with TfL which will be maintained |  |  |  |
| Rationale: |  |  |  |  |

## Dashboards

| Performance Dashboards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-552 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop and maintain suitable performance dashboards on the status of all open Faults and compliance with contracted outputs, such dashboards shall be visible to the Company. Note: The Supplier will work with TfL to agree Dashboard content and user access post contract award and before service commencement. |  |  |  |
| Rationale: |  |  |  |  |

| Performance Dashboards (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-553 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure development of suitable dashboards that present information by exception to Supplier Personnel and the Company is considered essential for the reliable delivery of Services and assurance to the Company. If such dashboards are not available when the Equipment is installed they shall be delivered within 6 Accounting Periods of the first Equipment being Delivered and an Acceptance Certificate issued by the Company. |  |  |  |
| Rationale: |  |  |  |  |

| Secure and Adutiable Records |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-550 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier and Cash in Transit (CIT) Contractor shall work together to enable a secure and auditable means to access the Equipment to deposit/remove monies from stations. |  |  |  |
| Rationale: |  |  |  |  |

## Service Desk

The service desk is a dedicated Oriented activity to TfL, which is currently used for Fault reporting, Level 1 support (issue resolution over the telephone) and Fault Management & Resolution

| Dedicated telephone Number |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-562 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall establish a dedicated telephone number to accept Fault calls from the Company. |  |  |  |
| Rationale: |  |  |  |  |

| Dedicated Email Address |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-563 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall establish a dedicated email inbox to accept reporting via email. |  |  |  |
| Rationale: |  |  |  |  |

| Answering Phone Calls |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-564 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure 90% of all such phone calls shall be answered by suitably qualified Supplier Personnel within 30 seconds |  |  |  |
| Rationale: |  |  |  |  |

| Faults & Threshold exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-565 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure that all Faults and threshold exceptions shall be identified, diagnosed and logged on an incident management system within 15 minutes of the Fault or threshold exception self reporting or the Fault being reported by the Company to the Supplier. |  |  |  |
| Rationale: |  |  |  |  |

| Reported Faults |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-566 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all reported faults shall be identified and tracked by a unique fault reference number. Note: The Unique Reference number must be linked to the original Fault Number |  |  |  |
| Rationale: |  |  |  |  |

| First Line Telephone Support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-567 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide first Line telephone support to Company Users and Administrators to try and resolve faults without having to dispatch Supplier Personnel. Where a resolution by Company staff is not viable or has not proven effective then Supplier Personnel shall be assigned to effect repairs |  |  |  |
| Rationale: |  |  |  |  |

| Faults - Access to Company Premises |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-568 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier's service desk is responsible for arranging access to Company Premises where Faults have occurred and comply with the Company's access processes, adopting the Refused Access Process where applicable. |  |  |  |
| Rationale: |  |  |  |  |

| Single Point of Contact |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-569 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier's service desk shall be the single point of contact and escalation for Maintenance Services and Faults and shall keep the Company informed by pro-active updates where Faults have occurred. Where escalation to Level 2 or 3 support is required this will be managed by the Supplier service desk. |  |  |  |
| Rationale: |  |  |  |  |

| Exceeding Rectification Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-570 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that where a Rectification Time has been exceeded as detailed in Schedule 8 (Supplier Performance) this is escalated immediately to the Company service desk, with a plan to address such failure and the time scales. |  |  |  |
| Rationale: |  |  |  |  |

| Faults Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-590 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier's service desk shall have the availability to report/log faults 24 x 7 and to provide support to Company Personnel as specified in the Specification and Schedule 8 (Supplier Performance), excluding Christmas Day. For Primary Service Desk Hours requirement see requirement CHD-REQ-36 |  |  |  |
| Rationale: |  |  |  |  |

| Telephone Guidance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-591 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where such Faults may be resolved by local Company Personnel the Supplier will provide telephone guidance to these Company Personnel to help repair faults more quickly and reduce the Suppliers operational costs. |  |  |  |
| Rationale: |  |  |  |  |

## Help Desk

The helpdesk is user-centric activity not only dedicated to TfL but extends to the likes of CIT operator, this is currently used for:

- Account queries

- System access requests (password queries)

| Help Desk Support |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-592 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure Help desk support is available to Users (station staff) and/or Company Administrators between the hours of 07:00 – 17:00, Monday to Friday |  |  |  |
| Rationale: |  |  |  |  |

| Out of Hours Faults |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-593 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure Out of hours Faults can be raised via the Help desk (email) by Users (station staff) and/or Company |  |  |  |
| Rationale: |  |  |  |  |

| Help desk availability for CIT contractor, |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-594 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure their Help desk availability for the CIT Contractor, cash collection support (issue resolution) shall be available between 7:00 – 17:00, Monday to Saturday. |  |  |  |
| Rationale: |  |  |  |  |

## Maintenance & Faults

| Delivery & Maintenance of Communications systems |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-556 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall be responsible for the Delivery and maintenance of the communications systems that connects each cash handling device to the back office systems provided by the Supplier to deliver the Services. The Supplier is in all cases responsible for the extract of data from all cash handling devices under the contract. At some locations, the Company provides fixed line services, and in these cases the Supplier remains responsible for the extract of data. |  |  |  |
| Rationale: |  |  |  |  |

| Delivery & Maintenance of Back Office Systems and Components |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-557 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall be responsible for the Delivery and maintenance of suitable back office systems and components to operate the Equipment and reliably deliver the Services, including all Data, financial and service reports throughout the Term of this Contract. |  |  |  |
| Rationale: |  |  |  |  |

| End to End Maintenance Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-558 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall be responsible for the reliable delivery of the end to end Maintenance Services. |  |  |  |
| Rationale: |  |  |  |  |

| Preventitive Maintenance Activities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-571 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all reactive and planned and preventative maintenance activities will be recorded by the Supplier on a suitable service management system. |  |  |  |
| Rationale: |  |  |  |  |

| Preventitive Maintenance Activities (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-572 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure All reactive and planned preventative maintenance tasks and activities will be recorded by the Supplier within the incident record; all activities shall be time and date stamped. Note: All Personnel working on the Companies Premises shall be recorded within the incident record. |  |  |  |
| Rationale: |  |  |  |  |

| Equipment Fault |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-551 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that where an Equipment Fault exists that will prevent a scheduled CIT Contractor delivery or pick up, this Fault will be prioritised and repairs completed to prevent abortive CIT Contractor activities where practicable. |  |  |  |
| Rationale: |  |  |  |  |

| Monitor Equipment and Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-575 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop an ability to monitor the Equipment and Services such that Faults can be identified and repairs affected without reliance upon Company Personnel to report such Faults. |  |  |  |
| Rationale: |  |  |  |  |

| Fixing of Faults |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-560 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall fix all Faults without delay regardless of cause. Any commercial discussions about interfaces and/or responsibilities will be held at the Contract progress meeting |  |  |  |
| Rationale: |  |  |  |  |

| Third Parties fault resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-561 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where ambiguity exists regarding a Fault across an interface, the Supplier shall work with the other party to effect a resolution at no additional cost to the Company |  |  |  |
| Rationale: |  |  |  |  |

| Real Time Visibility of Faults |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-576 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop and maintain suitable tools that allow the Company to have visibility in near real time of all Faults, their current status and activities being undertaken by the Supplier to affect repairs. |  |  |  |
| Rationale: |  |  |  |  |

| Daily Cumulative Fault Reports |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-579 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall send to the Company a Daily Cumulative Faults Report (in Excel and PDF format) each morning by 08:00 hours that details all open and closed Faults that have occurred in the previous Operating Day, as a minimum such report will contain: - Location – machine - Fault type and service affected - Date and Time fault occurred - All Activities undertaken (time and date) - Symptom, Cause and Actions - Parts used - Elapsed failure time - Abatements if applicable - Repeat Faults on the same item of Equipment within the last 12 rolling weeks |  |  |  |
| Rationale: |  |  |  |  |

# Service & Maintenance Non-functional Requirements

## Access and Security

| Network Security: Secured Connections |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-263 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall implement all incoming and outgoing connections using secured networks in accordance with the TfL S1736 Network Information Security standard |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Network Security: Access Logging |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-264 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall log all successful and unsuccessful access attempts to the Solution in an audit trail in accordance with the TfL S1791 Operational Information Security standard |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Network Security: Activity Logging |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-265 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall automatically log all activities performed within the Solution in an audit trail in accordance with the TfL S1791 Operational Information Security standard |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Network Security: Access Log Retention |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-266 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall retain all audit trail log files, as mentioned in requirement Network Security: Activity Logging, for a period of time in accordance with the TfL Corporate Information and Records Disposal Schedules |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Endpoint Data Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-267 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall comply with the TfL S1793 IT System Acquisition, Development and Maintenance standard for the purposes of endpoint and data protection |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Certificate Storage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-268 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Where there are digital/security certificates applicable to the Solution, these shall be stored in TfL's Tech and Data IT Service Management (ITSM) framework with expiry dates at least 9 months in the future. |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Access Control: Approval |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-270 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall comply with the TfL S1735 Information Technology Access Control standard for approving user access to the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Access Control: User Accounts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-271 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall use role-based access control with roles based on TfL provided and maintained directory services or roles otherwise defined by TfL |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Access Control: User Activity Log File |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-272 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Solution shall record the following information in an audit log file (see requirement Network Security: Activity Logging) for all activity performed by users within the Solution, that includes but is not limited to: - Unique identifier for the user activity - Date/Timestamp of the activity - User account identifier - Type of activity (e.g. view record, creation, update, deletion) - Before and After values of attributes changed on data records |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Remote Access Technologies for TfL Users |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-274 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall allow remote access for authorised users through approved TfL remote access services |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Data Types |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-275 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall comply with TfL's approach for the security classification of data as described in TfL's Information Security Classification Standard – Schedule 20 of Volume 2 - for all data, personal and non-personal, that is processed and stored by the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Third Party Access: Approval |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-15 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall comply with the TfL S1735 Information Technology Access Control standard for granting access to the Solution for users employed by the Supplier or those employed by a Partner organisation. This shall require approval to first be obtained from either the: - TfL Business Owner for the Solution OR - TfL Service Owner (or Service Owner delegate) for the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Third Party Access: Technologies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-279 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall allow remote access for authorised users employed by the Supplier or it's Partner organisations through a TfL approved service |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Third Party Access: Information About Third Party |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-281 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier of the Solution shall provide TfL with information about their own organisation and any partner organisations. This information includes, but is not limited to: - Supplier Name - Primary Contact Details - Contacts (Roles and Communications methods) - Scope of service in relation to the Solution - Duration / Cost - TfL Budget Code - Link to Commercial Register entry |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Back Office - Device Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-282 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The back office Solution shall be accessible to authorised users using devices that are supported by TfL or devices that follow an access route supported by TfL. These include, but are not limited to: - Personal Computer connected to the TfL OneLondon network (for example Thick/Thin Desktop Client, Laptop) - Corporate Mobile Device (for example iPhone, iPad) - Bring Your Own Device (BYOD) Note: The front end (the CHD) will not be accessible remotely |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Data Centre Hosting: Location |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-283 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | Where the Solution is hosted in the cloud, it shall adhere to TfL's Hosting Policy and Infrastructure Hosting Architectural Principles |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Data Centre Hosting: Physical Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-284 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall comply with the TfL S1747 Physical and Environmental Security of TfL's Information Technology standard with respect to authorised and unauthorised personnel requiring physical access to the hardware on which the Solution is hosted |  |  |  |
| Rationale: | This is a standard cyber security requirement for all IT solutions which are delivered to TfL. This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Availability

| Maintenance Windows: Frequency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-22 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier will inform TfL four (4) weeks in advance of any planned maintenance activities that are scheduled to take place |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Maintenance Windows: Information Required |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-286 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide TfL with the following information in relation to each maintenance window for the Solution: - Calendar date(s) - Time (24hr clock) window - Additional approval steps (if applicable) - Custom communications approach (if non-standard) - Service impacting (Yes / No) |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Solution Availability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-287 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall be at least 99.995% available on an annual basis, excluding agreed maintenance windows, where annual basis is defined as twenty four (24) hours a day, seven (7) days a week, three hundred and sixty five (365) days a year. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Compliance

## Integrity

| Auditing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-289 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall automatically record an audit trail of all user activities that take place within the Solution in an audit log file that can be viewed by users with the appropriate permissions. The activities to be recorded for the audit trail include, but are not limited to: - Creation of data records - Updates to data records - Deletion of data records - Changes to the system configuration - Changes to user permissions - Creation of user accounts - Deletion of user accounts |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Profile |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-290 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution, whether it is hosted by TfL or a third party, shall run online backups without degrading the performance or limiting the operation of the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Retention Period |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-28 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Solution shall retain all data backups for a maximum of 3 years |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Encryption |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-292 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall encrypt all data backups in accordance with the TfL S1740 Cryptography standard |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Storage Location |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-293 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall, with the agreement of TfL, store backups of the Solution in an alternative location to the datacentre which hosts the primary instance of the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Ad hoc Backups |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-294 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | the Solution shall allow an authorised user to perform an on-demand backup without degrading the performance or limiting the operation of the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Notifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-295 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When a back-up has completed, the Solution shall automatically send a notification, either from within the Solution itself or by other means (for example email), to one or more designated users about the status of the back-up. The information to be included in the notification shall include, but not be limited to: - Date and Time of the back-up - Status of the back-up: Success, Partial Success, Failure |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Backup Log File Retention Period |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-296 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall retain audit log files as live data in the Production environment for a minimum of thirty (30) days before these log files are archived in accordance with TfL's Corporate Information and Records Disposal Schedules |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Maintainability and Support

### Incident and Problem Management

| Third Party Compliance with Incident and Request Management Process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-297 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall comply with TfL's Tech and Data Incident and Request Management Process for the resolution of Incidents raised against the Solution. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Primary Service Desk Hours |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-36 |  | Priority: Must have | Status: Final | Version: v9 |
| Description: | The Supplier of the Solution shall collaborate with TfL in resolving Incidents raised against the Solution on a 24x7x365 (24 hours a day, 7 days a week, 365 days a year, excluding Christmas Day) basis |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Incident Response Times |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-300 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When an Incident is raised against the Solution, the Supplier shall comply with the response times stipulated by TfL's Tech and Data Incident and Request Management process as follows: - Severity 1: 15 minutes - Severity 2: 30 minutes - Severity 3: 120 minutes - Severity 4: 120 minutes The response time is defined as the maximum length of time between the Incident being raised and an initial response by the Supplier being logged. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Incident Resolution Times |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-301 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When an Incident is raised against the Solution, the Supplier shall comply with the resolution times stipulated by TfL's Tech and Data Incident and Request Management process as follows: - Severity 1: 2 hours - Severity 2: 4 hours - Severity 3: 8 hours - Severity 4: N/A The resolution time is defined as the maximum length of time taken to apply a resolution to the Incident. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Incident Update Frequency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-302 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When an Incident is raised against the Solution, the Supplier shall provide TfL with progress updates about the Incident in accordance with the update frequency stipulated by TfL's Tech and Data Incident and Request Management process as follows: - Severity 1: Every 30 minutes - Severity 2: Every 60 minutes - Severity 3: Daily - Severity 4: N/A The update frequency is how often the Supplier is expected to provide updates to TfL and affected users of the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Component Coverage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-303 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide Service Incident support for all components of the Solution on a 24x7x365 (24 hours a day, 7 days a week, 365 days a year) basis. These components include, but are not limited to: - Infrastructure - Application - Middleware - Endpoints |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Create Problem Record |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-305 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | When the root cause(s) of an Incident has been identified, the Supplier shall collaborate with TfL's Problem Manager in creating a Problem record in TfL's Tech and Data IT Service Management (ITSM) framework |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Problem Record Prioritisation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-306 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | When a Problem record is raised against the Solution, the Supplier shall collaborate with TfL's Problem Manager in prioritising the Problem based on: - Impact (the net effect the Problem is expected to have on business activities) - Urgency (how soon the impact of the Problem will be felt by the business) |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Response to Severity 1 Incidents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-307 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Where a Severity One Incident against the Solution has an agreed work-around, the Supplier shall immediately raise a Problem record in order to investigate and identify the underlying cause(s) of the issue within three (3) days of the service being restored for the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Trend Analysis |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-308 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When a Problem is raised against the Solution, the Supplier shall conduct trend analysis on the Problem in order to identify common issues and underlying causes. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-309 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier of the Solution shall produce a Problem Management report at a minimum every week or by exception if no problems previously existed before the new problem occurred. The report shall include information about: - All new Problems registered in the last seven (7) days - All unresolved Problems - All Problems resolved in the last seven (7) days |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Problem Management: Report Details |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-310 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall include the following information for each unresolved Problem identified in the Problem Management Report: - Unique Problem Identifier - Problem Name - Problem Description - Plan for Problem Resolution (unresolved Problems only) - Resources required to resolve the Problem (unresolved Problems only) - Estimated Date to resolve the Problem (unresolved Problems only) - Any identified Problem trends |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

### Change and Release Management

| Third Party Compliance with Change and Release Management Process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-311 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall comply with TfL's Tech and Data Change and Release Management Process to ensure that all changes to the Solution are implemented in a way that minimises disruption to the users of the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Release Plan Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-312 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When an update is to be made to the Solution, the Supplier shall provide TfL with a Release Plan containing information about the update that includes, but is not limited to: - Software required for the Release - Services dependent on the Release - Parties that are impacted by the Release and who need to be engaged (technical, business and third parties) - High-level step-by-step deployment guide for the Release - Roll-back guide for returning the Solution to a previous version - Additional costs incurred by TfL for the Release |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Model |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-314 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall collaborate with TfL to produce a Service Model defining agreed service level measures and support that include, but are not limited to: - Availability - Capacity - Performance - Incident Management - Escalation - Reporting |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Reviews |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-52 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall attend periodic service reviews with TfL regarding the ongoing provision of the Solution. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Reports: Intervals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-316 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier of the Solution shall provide TfL with a Service Report every four (4) weeks. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Reports: Metrics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-317 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide TfL with a Service Report - see requirement Service Report: Intervals - for the Solution that measures the following criteria: - Agreed Service Levels - Number of Incidents - Number of Problems - Number of Changes - Licensing Status - Capacity Reporting |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Reports: Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-318 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide TfL with a Service Report - see requirement Service Report: Intervals - for the Solution containing details about the report that include: - Name of the report - The purpose of the report - How often the report is generated - Who generates the report - Recipients of the report |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Service Requests |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-319 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide TfL's Service Request Management team in a form provided by TfL |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Configuration Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-320 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide TfL with the following information about every component (Configuration Item) of the Solution - Unique Identifier - Name - Description - Type - Status - Location - Relationship to other Configuration Items (CIs) - Owner |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| License Management: Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-321 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide TfL with information about licensing for the Solution, including the following details: - Name of license provider - License Type - Duration / Cost - Quantity - Deployment Method - Known Limits |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| License Management: Renewals |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-322 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure Licenses applicable to the Solution are automatically renewed at least 9 months before the with expiry dates of the licenses. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Hardware (Spare Stock) Availability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-323 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall store adequate levels of spare hardware stock either at or near the site at which the primary instance of the Solution is hosted in order to maintain the Solution's operational service levels |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Performance

| Concurrent Viewing of Data Records (TO BE APPROVED BY OO) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-61 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | The Solution shall allow a minimum of two (2) and a maximum of fifteen (15) authorised users to view the same data record concurrently without degrading the performance of the Solution. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Reliability

| Single Points of Failure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-343 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall contain no single points of failure, so that a failure in any individual component of the Solution does not result in a total loss of service. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Solution Automation: Restarts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-344 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | If there is a failure of the Solution, then it shall be capable of automatically restarting the following: - Entire Solution - Individual functions - Scheduled batch jobs |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Solution Automation: Restart Alerts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-345 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | When the Solution restarts following a failure, as described in requirement Solution Automation: Restarts, the Solution shall automatically generate an alert to authorised users to indicate that there was a failure in the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

### Monitoring

| Capacity Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-325 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall monitor the Solution's capacity for 100% of the time for the agreed availability, as defined in requirement Solution Availability and excluding agreed maintenance windows as defined in requirement Maintenance Windows: Frequency. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| System Capacity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-326 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall ensure that the Solution does not exceed 75% of the available capacity for the components defined in requirement Capacity Monitoring |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| End-to-End Monitoring: Solution Layers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-328 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that there is end-to-end monitoring of the Solution on a 24x7x365 (24 hours a day, 7 days a week, 365 days a year) basis for the characteristics defined in requirement Monitoring Solution Characteristics at each of the following layers of the Solution: - Availability - Application - Application Performance Monitoring (APM) - Infrastructure including hardware, networks and databases - User - Security |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Monitoring Solution Characteristics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-329 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that the end-to-end monitoring capability, as described in requirement End-to-End Monitoring: Solution Layers, monitors the following characteristics of the Solution on a 24x7x365 (24 hours a day, 7 days a week, 365 days a year) basis: - Availability - Capacity - Data Completeness - Performance - Security |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Availability Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-330 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall ensure that the Availability of the Solution is monitored, in accordance with requirements Solution Availability, End-to-End Monitoring: Solution Layers and Monitoring Solution Characteristics, for the following components that include, but are not limited to: - Hardware - Network - Servers - Databases - Application Functions |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Data Completeness Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-331 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall have the capability to monitor, in accordance with requirements End-to-End Monitoring: Solution Layers and Monitoring Solution Characteristics, the integrity of the data that the Solution stores and processes against a number of metrics that include, but are not limited to: - Frequency of data backups - Frequency of testing data restore procedures - Percentage of data that is accurately restored by data restore procedures - Data authenticity (data precision, accuracy of information portrayed, data relationships, accuracy with respect to source data, data encryption methods) |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Performance Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-69 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall have the capability to monitor, in accordance with requirements End-to-End Monitoring: Solution Layers and Monitoring Solution Characteristics, the Performance of the Solution against a number of metrics that include, but are not limited to: - Response times for Solution functions (Minimum, Maximum, Average) - Number of database records accessed in a given time period - Server Load - Network Latency |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Security Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-333 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | Where the Solution is not hosted by TfL, the Supplier shall have the capability to monitor, in accordance with requirements End-to-End Monitoring: Solution Layers and Monitoring Solution Characteristics, the Security of the Solution against a number of metrics that include, but are not limited to: - Status of the Solution's digital certificates - Inactive user accounts - Successful / failed login attempts - Session usage aggregates (start/stop date and time) - Updates to role profiles - Numbers of users for each role permission / privilege level |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Generating Alerts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-334 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall generate an alert to authorised users if a predefined monitoring threshold is breached for the thresholds defined in requirement Capacity Threshold Breach Notification and for the characteristics defined in requirement Monitoring Solution Characteristics. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Monitoring Dashboards and Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-336 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution characteristics that are being monitored, as defined in requirement Monitoring Solution Characteristics, and their associated alerts shall be accessible to authorised users through dashboards and reports (scheduled and ad hoc) |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Contents of Monitoring Alerts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-337 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Alerts generated by the Solution in response to an event shall contain information that is an accurate reflection of that event which includes, but is not limited to: - What happened - Date and time when the event occurred - How the event was handled - Who the event was escalated to - Details of communications with users and systems to support any actions taken - Breaches of any Operational Level Agreements (OLAs) - Breaches of any Service Level Agreements (SLAs) |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Reviewing Monitoring Alerts and Events |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-75 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Prior to service commencement, The Supplier shall agree with TfL a schedule of reviews, at set intervals, to investigate the events and alerts that are generated by the Solution for applicability, accuracy and gap analysis. This will enable the Supplier to understand and define what the events and alerts mean, how they are generated, where and when they are generated and put in place suitable measures to respond to them. |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Create Monitoring Thresholds |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-340 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall enable authorised users to create monitoring thresholds against which events can be logged. The attributes to be defined for these thresholds shall include, but not be limited to: - Name of threshold - Description of threshold - Solution characteristic to be monitored (e.g. Availability, Capacity, Data Completeness, Performance, Security) - Lower-level Solution characteristics to be monitored (e.g. for Capacity these may be storage or number of licenses) - Threshold value (e.g. for Capacity it may be that the number of licenses used exceeds 80% of the total available number of licenses for the Solution) - Which Users are to be notified in the event that Solution thresholds are breached - Raise an Incident Ticket if the Threshold is breached (Yes / No)? - Severity level to be assigned to the Incident ticket |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Robustness

| Infrastructure Resilience |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-346 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall, as a minimum, meet the following defined levels of infrastructure resilience: - Primary Solution instance hosted in a single data centre with high availability - see requirement Solution Availability - Replication of the primary Solution instance to the back-up Solution instance hosted in a separate data centre - Synchronous data replication from the primary Solution instance to the back-up Solution instance |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Failover Connection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-347 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | If there is a failure in the Solution, users shall be able to automatically log into a failover instance of the Solution without having to manually enter their login credentials |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Recovery Point Objective |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-348 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | If there is an unplanned interruption to normal operations, then the Solution shall not permit any loss of data that is input and stored in the Solution |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Recovery Time Objective |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-349 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | If there is an unplanned interruption to normal operations, then the Solution shall be fully recoverable within one (1) minute of service disruption notification |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Disaster Recovery Plan |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-350 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide a Disaster Recovery Plan for the Solution, as agreed with TfL, which shall be in the format of a document called "Disaster Recovery On a Page". |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Disaster Recovery Plan Contents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-351 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall include the following information in the Disaster Recovery Plan for the Solution: - Minimum failure scenarios which trigger the Disaster Recovery process (For example, loss of one or both Datacentres, or the loss of a TfL Hub building) - Common service restoration steps - Resolver matrix describing who is responsible for resolving issues for the components of the Solution - Business contacts who need to be engaged - Service Impact descriptions - Dependent and depending services for the Solution - Links to design and build documents See requirement Disaster Recovery Plan |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| Business Continuity Testing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-352 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall, in collaboration with TfL, test the Solution's Continuity and Disaster Recovery capabilities every year |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Scalability

| System Growth: Users |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-90 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall be scalable at any given time to accommodate its use by the projected number of concurrent users for the next twelve (12) months |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

| System Growth: Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-91 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Solution shall be scalable at any given time to accommodate projected data volumes for the next twelve (12) months |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Usability

| Accessibility |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-355 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Solution shall conform to TfL's Digital Accessibility Standard |  |  |  |
| Rationale: | This non-functional requirement has been stipulated by the Service Strategy and Design (SS&D) team as part of their Standard Service Requirements (SSRs) and is based on the criticality of the Service / Solution which is classed as Mission Critical. |  |  |  |

## Data / IT Interfaces

| Non Proprietary Software |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-529 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall supply and support non-proprietary software. |  |  |  |
| Rationale: |  |  |  |  |

| Non Proprietary Hardware |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-530 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall supply and support non-proprietary hardware. |  |  |  |
| Rationale: |  |  |  |  |

| Intergated Processes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-577 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall work with the Company to develop integrated processes and procedures for service monitoring, incident and change management for the Term of the Contract. All such processes shall be agreed and documented by the Supplier during the Delivery of the Contract. |  |  |  |
| Rationale: |  |  |  |  |

## Technical Interfaces & Security Provisions

The following business requirements set out the technical and security provisions that the Supplier shall comply with for the Equipment and Services

| Authorised Admistrators |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-699 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Company Administrators are able to access authorised Services by suitable user interfaces available via web browser (HTTPS). The user interface(s) shall not require any third party library to be installed on the Company Administrator's web browser to enable the Delivery of this requirement. |  |  |  |
| Rationale: |  |  |  |  |

| Latest Version Software |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-700 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the web browsers and software used / supported by the cash handling solution are the latest version used by TfL and maintained in line with subsequent upgrades to TfL's systems. |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Policy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-701 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure develop Services in compliance with the Company's Information Security Policy. |  |  |  |
| Rationale: |  |  |  |  |

| Generic User Accounts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-702 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure no generic user accounts are allowed or supported once any cash management machine has been installed and in service. All privileged and non-privileged account holders will be issued with their own, unique, logon credentials. |  |  |  |
| Rationale: |  |  |  |  |

| System Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-704 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure all system Documentation relating to security features, user accounts, passwords and Equipment configuration shall be marked and treated as confidential information. System Documentation shall not be sent in unencrypted form. A minimum of 256-bit AES encryption shall be used if Documentation is sent in electronic form; passphrases must be exchanged out of band. |  |  |  |
| Rationale: |  |  |  |  |

| Security Vulnerability Testing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-705 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide to the Company independent certification guaranteeing that the Supplier's Services have passed security vulnerability testing. |  |  |  |
| Rationale: |  |  |  |  |

| Security Passwords |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-706 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment is not configured with default security passwords at go live. |  |  |  |
| Rationale: |  |  |  |  |

| Access via PINs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-707 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall prevent access via insecure PINs (Personal Identification Numbers) (e.g. 0000 or 1111). |  |  |  |
| Rationale: |  |  |  |  |

| Reporting Security Events |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-711 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure attempts to tamper with the technical interfaces between the Equipment and other systems delivering the Services will be reported as a security event with a corresponding alarm condition raised. |  |  |  |
| Rationale: |  |  |  |  |

| Removable Media |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-712 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment shall protect against unauthorised access to removable media ports using a physical access measures. |  |  |  |
| Rationale: |  |  |  |  |

| Network Bridge |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-713 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cash management machine does not act as a network bridge between data networks. |  |  |  |
| Rationale: |  |  |  |  |

| Proposed Technical Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-715 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall inform the Company of proposed technical changes of the Equipment and its associated components for approval by TfL prior to implementation |  |  |  |
| Rationale: |  |  |  |  |

| Cellular Radio Connectivity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-717 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall provide an integrated domestic cellular radio (4G/5G) communication network option for the Equipment. |  |  |  |
| Rationale: |  |  |  |  |

| Cellular Radio Connectivity (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-718 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the cellular radio communication network option will include the capability of a fixed IP SIM with private APN and fixed for security. SIM cards will be configured such that they cannot roam and will only allow Data. |  |  |  |
| Rationale: |  |  |  |  |

| Signal Strength |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-719 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure a network operator with suitable signal strength at each location is used. Note: The Supplier will provide a solution at locations to boost signal strength to the required levels where signal strength is a particular issue. |  |  |  |
| Rationale: |  |  |  |  |

| Confidential & Authentic Data flows |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-720 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment and associated architecture shall use TLS or SSL for securing system to system communication channels and data flows. All transactions and data flows between Equipment is guaranteed confidential and authentic. |  |  |  |
| Rationale: |  |  |  |  |

| Single APN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-723 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure any deployed cellular communication option will utilise a single APN for all communications with the Equipment. The APN will be configured in such a way that a dedicated data pipe will be created between the cellular network operator's network and the Equipment. The APN naming of the APN shall follow an agreed convention between the Supplier and the Company. |  |  |  |
| Rationale: |  |  |  |  |

| SIM/Cellular Network Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-724 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall where practicable ensure that the Equipment with specific reference to the SIM/cellular network complies with the requirements set out below: |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Management Framework |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-725 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall operate and be certified for ISO/IEC27001 information security management framework. |  |  |  |
| Rationale: |  |  |  |  |

| End to End Services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-726 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier Equipment and end to end Services shall support both remote and on site software patching by the Supplier. |  |  |  |
| Rationale: |  |  |  |  |

| Software Patching |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-727 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall be responsible for Software patching of the Equipment and its associated components/modules. |  |  |  |
| Rationale: |  |  |  |  |

| System Communication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-728 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | In relation to system communication, the Supplier Equipment and related modules/components shall utilise either: XML and JSON for definition of Data structures, REST or SOAP, file transfer as XML or flat file, or message queue styles for system to system communication with due consideration to factors such as volume, synchronous/asynchronous, batch/individual. |  |  |  |
| Rationale: |  |  |  |  |

| Documentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-729 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure that the Equipment and end to end solution's technical interfaces are documented in an interface register with corresponding specifications. |  |  |  |
| Rationale: |  |  |  |  |

| Status Updates - Polling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-730 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall ensure the Equipment supports SNMP V3 and is capable of being polled for status updates and the management of network Equipment The Company desire is to integrate the cash management machines with the Company's existing monitoring Equipment. Such integration will be instructed by the Company by means of a Variation. |  |  |  |
| Rationale: |  |  |  |  |

| Monitoring of Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-731 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall monitor all Equipment and Services. Where Equipment goes off line the Supplier shall report such disconnections and raise an alarm condition. |  |  |  |
| Rationale: |  |  |  |  |

## Training / Support

The Training / Support functional requirements comprise the training required prior to and during Delivery of the Equipment and Services across the Company Premises and the further training and support requirements for Users and trainers throughout the Term of the Contract.

| Training - changes to system / device |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-589 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide suitable training/briefings to administrators and designated company trainers, in the event of major system or device changes. |  |  |  |
| Rationale: |  |  |  |  |

| Training - changes to system / device |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-596 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop and maintain suitable training/briefing material to the company in the event of any major system or device changes. |  |  |  |
| Rationale: |  |  |  |  |

| Documentation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-597 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall develop and maintain as a minimum suitable Company Premises access courses, method statements, risk and COSHH assessments and ensure that all Supplier Personnel are suitably trained prior to attending any Company Premises. The first method statements, risk and COSHH assessments for Delivery of the Equipment are required two 2 weeks after the Start Date Accurate training records for all Supplier Personnel shall be maintained by the Supplier and made available to the Company upon request. |  |  |  |
| Rationale: |  |  |  |  |

| Training Materials |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-598 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide training materials to support an e-learning approach for new Users to understand how to use the Equipment. |  |  |  |
| Rationale: |  |  |  |  |

| Training & Help Screens |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-599 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide training and help screens and prompts to assist Users in rectifying problems on the Equipment. |  |  |  |
| Rationale: |  |  |  |  |

## Physical, Environmental & Engineer Constraints

There are many constraints within the London Underground station environment that the Supplier needs to be aware of and consider when planning, installing and maintaining Equipment in these Premises.  The physical size of the secure rooms (POM suite) present challenges to the installation, operation and maintenance of the Equipment that must be assessed and solutions implemented, such that all activities can be undertaken safely.

| Accessing CHD - Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-608 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is accessed and operated from minimum angles (i.e. the front and top) and does not need full 360° access. |  |  |  |
| Rationale: |  |  |  |  |

| Accessing CHD - Maintained |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-609 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is accessed and maintained from minimum angles (i.e. front and top) and does not need full 360° access. |  |  |  |
| Rationale: |  |  |  |  |

| Security Rating Level 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-610 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device has a minimum security rating of Level 2 in accordance with the Loss Prevention Standard (LPS) LPS1175 as amended. |  |  |  |
| Rationale: |  |  |  |  |

| Total Weight - CAT A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-611 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the total weight of the cash handling device when empty of cash for a Category A cash management machine shall be less than 500kg |  |  |  |
| Rationale: |  |  |  |  |

| Total Weight - CAT B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-612 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the total weight of the cash handling device when empty of cash for a Category B cash management machine shall be less than 350kg |  |  |  |
| Rationale: |  |  |  |  |

| Total Weight - CAT D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-613 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the total weight of the cash handling device when empty of cash for a Category D cash management machine shall be less than 250kg |  |  |  |
| Rationale: |  |  |  |  |

| Exceeding Maximum Load Weights |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-614 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that If the above maximum loaded weights cannot be met then the cash management machine may require to be mounted on a load bearing base and / or fixed on the ground beneath. The Supplier is to provide a proposed solution of spreading the load without the need for a concrete base to be installed). Note to bidders: The Contract or is to price such proposals as requested in the ITT guidance documentation. |  |  |  |
| Rationale: |  |  |  |  |

| Device Dimensions - CAT A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-615 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the maximum device dimension to replace an existing Category A cash handling device is less than 160cm x 150cm x 80cm (h x w x d) |  |  |  |
| Rationale: |  |  |  |  |

| Device Dimensions - CAT B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-616 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the maximum device dimension to replace an existing Category B cash handling device is less than 160cm x 80cm x 80cm (h x w x d) |  |  |  |
| Rationale: |  |  |  |  |

| Device Dimensions - CAT D |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-617 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the maximum device dimension to replace an existing Category D cash handling device is less than 160cm x 80cm x 80cm (h x w x d) |  |  |  |
| Rationale: |  |  |  |  |

| Spatial Dimensions of Station Sites |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-618 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure each category of cash handling device is installed, set up, fully maintained and operated effectively and efficiently within the spatial dimensions of the station sites (which may be larger than the maximum dimensions provided in above) |  |  |  |
| Rationale: |  |  |  |  |

| Delivery & Installations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-619 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is split into modular components for Delivery and installation at various sites. |  |  |  |
| Rationale: |  |  |  |  |

| Securing Devices to Wall or Floor |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-620 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where necessary, the Supplier shall ensure Cash handling devices can be secured to the floor or wall to prevent movement or tipping when servicing is undertaken. Note: where future devices weigh more than the floor tolerance level, the Supplier will provide adequate spreader plates, these must be approved by TfL prior to their installation |  |  |  |
| Rationale: |  |  |  |  |

| Load Bearing Base |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-621 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure that if the cash handling device requires a load bearing base it is delivered and installed at the same time. |  |  |  |
| Rationale: |  |  |  |  |

| Electricity Supply |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-622 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is able to function on standard 240v single phase electricity supply |  |  |  |
| Rationale: |  |  |  |  |

| Reporting and Remote Functionality |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-623 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the Equipment provider's full software reporting and remote functionality through a suitable communication system to the Suppliers back office equipment |  |  |  |
| Rationale: |  |  |  |  |

| Secure Suite |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-624 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure the cash handling device is delivered to, installed and set up in Secure Suite, below ground with narrow access passages / corridors. |  |  |  |
| Rationale: |  |  |  |  |

| Movement of Cabinets and other Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-625 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | As part of the Delivery, The Supplier shall take responsibility for the movement of cabinets and other ticket office furniture (including the removal of worktops and coin counting equipment) to make way for the installation of the cash handling device at the time of the installation. Note: Any installations and or changes need to be carried out in consultation with TfL |  |  |  |
| Rationale: |  |  |  |  |

| Engineering Hours |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-626 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure installation takes place during engineering hours (non-train service hours or as agreed with the Company at individual stations), unless otherwise agreed by TFL. |  |  |  |
| Rationale: |  |  |  |  |

## Account Management

| Named Account Manager |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-585 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall provide a named Account Manager and notify the Company of this named individual. |  |  |  |
| Rationale: |  |  |  |  |

| Change of Named Account Manager |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-586 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall notify the Company if there is a change of Account Manager and notify them of the name of the new individual. |  |  |  |
| Rationale: |  |  |  |  |

| Organisational Chart |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-587 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall supply the Company with an organisational chart detailing those who are managing and operating the contract. |  |  |  |
| Rationale: |  |  |  |  |

# 6. Compliance

## Compliance with TfL Policies and Standards

| TfL & LUL Category 1 & 2 Standards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-661 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall comply with TfL and LUL category 1 and 2 standards and draft Category 1 and 2 Standards and such European, British and International Standards and associated Codes of Practice as required by the Company to perform the services in accordance with Good Industry Practice. The following specific TfL and LUL category 1 and 2 standards have been identified as applicable: - 1-085 - Fire Safety Performance of Materials - GU-085 - Code of Practice –Fire Safety of Materials and Fire Safety of Specific Items and Materials used in the Underground - S1371 - Station Capacity Planning - S1375 - Planning for Ticket Issuing Facilities – See S1375 for all standards containing relevant information. - G1008 - LUCAS (London Underground Combined Access System) – Not sure this is a SHE document - S1069 - Low voltage electrical installations - G-073 - Electrical Guidance Document - PR0572 - Work Instructions Template - S1956 A1 - Earthing and Bonding of LUL Electrical Networks - S1902 A1 - Safe Systems of work on or near Electrical Equipment - PR1336 A2 - Electricity at Work Information for Direct Labour Organisation (DLO) - G1339 A4 - Asbestos information and control - F6304 - Waste Electrical & Electronic Equipment (WEEE) Regulations 2013 |  |  |  |
| Rationale: |  |  |  |  |

| Internet Access to Standards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-689 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | A full set of current standards is available for the Supplier’s use on-line. The Supplier shall be responsible for completing and submitting to the Contract Manager the application for internet access to the Standards |  |  |  |
| Rationale: |  |  |  |  |

| Written Derogation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-692 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Compliance to Standards will not be required where there is written derogation from the Standard in place. In such cases Compliance is required to meet the derogation. The Supplier will not be required to be compliant where such requirement is a consequence of a physical design (of the Asset to which the Category 1 or 2 Standard or Draft Category 1 or 2 Standards apply). |  |  |  |
| Rationale: |  |  |  |  |

| Compliance with the Principle of Safety Risks |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-693 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | Not withstanding the Written Derogation requirement, the Supplier shall ensure that in relation to the risks to which the Standards are applicable there is compliance with the principle of safety risks being managed ALARP (as low as reasonable practicable). |  |  |  |
| Rationale: |  |  |  |  |

| Ownership of Information or Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-559 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all Information or Data shall be owned by the Company and all such Information and Data shall be maintained and available to the Company in accordance with the Contract. |  |  |  |
| Rationale: |  |  |  |  |

| Work Related Road Risk Standards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-691 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall also ensure that all Standards required for any deliveries by the Supplier or its sub-contractors meet the Work Related Road Risk standards, and that vehicles comply with the Ultra Low Emission ZON (ULEZ). |  |  |  |
| Rationale: |  |  |  |  |

| Work Related Incidents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-662 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall report all Work Related Incidents related to TfL Infrastructure to TfL immediately or a soon as possible after the incident has taken place Note: The Service Provide will notify the Station Manager at the time of the incident |  |  |  |
| Rationale: |  |  |  |  |

| Safe Systems of Work |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-663 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure all personnel adhere to TfL safe systems of work, this can include method statements, risk assessments and where applicable demonstration of compliance of CDM regs |  |  |  |
| Rationale: |  |  |  |  |

| Standard Anomalies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-694 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall, should it become aware of any Standard anomalies, bring them to the Company’s attention in writing within ten (10) working days of becoming aware of such anomalies. Such notices will contain details of the applicable standards and an impact assessment on cost and programme |  |  |  |
| Rationale: |  |  |  |  |

## Information Governance

| Data Retention |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-356 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the system can retain and dispose data in accordance with TfL's Corporate Information and Records Disposal Schedules and/or any local disposal policies defined by TfL. All personal data processed through the solution must have an assigned deletion trigger. |  |  |  |
| Rationale: | Ability to apply retention in line with TfL Retention Policies |  |  |  |

| Access control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-357 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The solution shall have the ability to set account permissions at differing levels – i.e. implement role-based access controls. The system shall have assigned users roles with unique user ID and password / authentication to log in. The system shall have a process to suspend and restore account permissions. The System shall automatically lock a user account after five failed sign in attempts. |  |  |  |
| Rationale: | Implementation of access controls |  |  |  |

| System auditing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-358 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system will monitor and audit user activities – e.g., log in times, data access and changes made. |  |  |  |
| Rationale: |  |  |  |  |

| Ability to search and manage records |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-359 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The solution shall have the ability to search within it. |  |  |  |
| Rationale: |  |  |  |  |

| Cookies or similar technologies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-360 |  | Priority: Must have | Status: Final | Version: v6 |
| Description: | Where the Solution uses cookies or similar technologies that track information about people accessing a website or other electronic/digital service, including essential and non-essential cookies, the Solution must comply with the Privacy and Electronic Communications (EC Directive) Regulations 2003 (or any relevant subsequent legislation). |  |  |  |
| Rationale: |  |  |  |  |

| Transparency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-361 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The system shall provide TfL privacy statement for customer facing products |  |  |  |
| Rationale: | Ability to provide privacy statements and as required the consent button |  |  |  |

| Data Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-362 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the system is compliant with any legislation in force from time to time in the United Kingdom relating to privacy and/or processing of personal data, including but not limited to: - Data Protection Act 2018 - UK General Data Protection Regulation (UK GDPR) Including statutory codes of practice issued by the Information Commissioner in relation to such legislation. |  |  |  |
| Rationale: | Compliance with privacy and data protection legislation. |  |  |  |

| Data Subject Rights |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-363 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the system is compliant with data subject rights, including but not limited to: - Access to data (including the ability to search the system for information relating to a specific individual and extract this in a machine readable format) - Enable deletion of personal data relating to an individual in accordance with retention rules and, on a case by case basis, allow deletion of individual records outside those retention rules - Enable TfL to rectify / update inaccurate or out of date data - Allow processing of personal data to be placed on hold for an individual record. |  |  |  |
| Rationale: | Compliance with data subject rights |  |  |  |

| TfL Information Governance Policies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-364 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the solution is compliant with TfL's Information Governance policies - Records Management policy - Info Sec policies https://transportforlondon.sharepoint.com/sites/Instructions-and-guidance-information-governance/SitePages/TfL's-Information-Governance-policies.aspx |  |  |  |
| Rationale: | Compliance with TfL's Information Governance policies |  |  |  |

| Security Classification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-365 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The solution shall ensure that all information stored within the system must be given a "security classification" in line with TfL's Information Security Classification Standard. |  |  |  |
| Rationale: |  |  |  |  |

| Using Personal Data for Test Purposes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-366 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the use of personal data for test purposes is prohibited without prior consultation and authorisation from the TfL Privacy and Data Protection team. |  |  |  |
| Rationale: |  |  |  |  |

| Freedom of Information compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-369 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure the system supports compliance with Freedom of Information (FOI) legislation and Environmental Information Regulations (EIR). It must enable the searching and extracting of relevant data across the system if an FOI or EIR request is received – e.g. data searches relating to a specific event, data type or date range. |  |  |  |
| Rationale: |  |  |  |  |

| Secure Migration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-370 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure any migration of data from current systems to a new system is done in accordance with TfL security standards and other relevant policies and standards. |  |  |  |
| Rationale: |  |  |  |  |

| Legal Hold |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-372 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure preservation or extraction of evidence beyond the scheduled deletion point where they are subject to legal hold is supported. |  |  |  |
| Rationale: |  |  |  |  |

| Legal Admissibility |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-373 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Solution and any external scanning services employed by the Solution shall adhere to the mandatory procedures outlined in the TfL Standard S1783 to ensure the legal admissibility of scanned documentation. |  |  |  |
| Rationale: |  |  |  |  |

| Data Protection Training |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-375 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure all users complete data protection training on an annual basis |  |  |  |
| Rationale: |  |  |  |  |

| International Data Transfer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-376 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall ensure personal data must not be hosted, stored, maintained, archived, transferred or remotely accessed from, or otherwise processed, outside the UK or European Economic Area without authorisation from TfL Privacy and Data Protection team. • If hosted and / or provided and / or maintained by a third party, there should be a written contract to govern the processing of personal data by the third-party supplier, including standard data protection clauses. • If hosted by third party, the contract should address handover of information at cessation. If you are intending to process personal data outside of the UK or EEA, please describe the proposed processing; specify the country or countries in which the processing will take place; and provide sufficient evidence that you can meet the relevant requirements of current privacy and data protection legislation to include transfer risk assessments |  |  |  |
| Rationale: | Processing personal data, relating to TfL customers or employees, outside the UK or European Economic Area (EEA), either directly or via a sub contractor. This includes any cloud based IT services that may be used to process personal data outside of the UK or EEA at any time. NB: The EEA consists of all 27 EU member states, plus Norway, Iceland and Liechtenstein). |  |  |  |

| Generative Artificial Intelligence |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-380 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall advise where there solution does or intends to make use of Generative Artificial Intelligence; Processor (and any sub processors) must comply with TfL's policy on generative AI |  |  |  |
| Rationale: |  |  |  |  |

| Remote/Hybrid working |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-381 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The Supplier shall confirm whether their service or solution will make use of remote/home/hybrid working and provide evidence of policies and procedures to ensure the security and integrity of personal data processed where this is the case |  |  |  |
| Rationale: |  |  |  |  |

| Legal Admissibility (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-697 |  | Priority: Must have | Status: Final | Version: v3 |
| Description: | The Supplier shall ensure and demonstrate the evidential integrity of electronic information or data so that it can be admissible as evidence in a Court of Law. This includes but is not limited to ensuring: • That evidential information has been correctly and lawfully generated and has not undergone unauthorised amendment or been otherwise tampered with since its creation • Auditing of evidential information to track any User, system process or data amendments attached to the information, including who/what system made the amendments and when (date and time) Audit logs can be provided to a Court of Law as proof of evidential integrity (where applicable) and chain of custody |  |  |  |
| Rationale: |  |  |  |  |

| Data Protection (2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-698 |  | Priority: Must have | Status: Final | Version: v2 |
| Description: | The Supplier shall put in place appropriate technical and organisational measures to implement the data protection principles effectively and safeguard individual rights. This is 'data protection by design and by default'. |  |  |  |
| Rationale: |  |  |  |  |

## Cyber Security

### Security Standards

| Data Storage |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-382 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | Where the Supplier is connecting to, processing or storing TfL's Systems or Data on a regular basis, the Supplier shall be Independently certified to ISO/IEC 27001:2013 or latest version, with a scope which covers where TfL Data is to be stored. |  |  |  |
| Rationale: |  |  |  |  |

| Cyber Security Risk Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-431 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Policy P123 Cyber Security Risk Management |  |  |  |
| Rationale: |  |  |  |  |

| Information Technology Security |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-432 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Policy P116 Information Technology Security |  |  |  |
| Rationale: |  |  |  |  |

| System Access Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-433 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to S1735 System Access Control |  |  |  |
| Rationale: |  |  |  |  |

| Network Cyber Security |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-434 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1736 Network Cyber Security |  |  |  |
| Rationale: |  |  |  |  |

| Cryptography |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-435 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1740 Cryptography |  |  |  |
| Rationale: |  |  |  |  |

| Physical Security of IT Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-436 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1747 Physical Security of IT Equipment |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Classification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-437 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1782 Information Security Classification |  |  |  |
| Rationale: |  |  |  |  |

| Organisation of Information Security |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-438 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1788 Organisation of Information Security |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Asset Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-439 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1790 Information Security Asset Management |  |  |  |
| Rationale: |  |  |  |  |

| IT System Acquisition, Devlopment and Maintenance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-440 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1793 IT System Acquisition, Development and Maintenance |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Supplier Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-441 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1794 Information Security Supplier Management |  |  |  |
| Rationale: |  |  |  |  |

| Information Technology Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-442 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the system is compliant to Standard S1797 Information Technology Compliance |  |  |  |
| Rationale: |  |  |  |  |

| Evidence of Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-383 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall provide evidence of compliance with the above requirements before entering the contract. |  |  |  |
| Rationale: |  |  |  |  |

### Security Principles

| Security Risk Register |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-384 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure a Security Risk Register is produced and maintained throughout the Term and that all Security Risks are documented in an appropriate manner and are included in any contract risk register for this Agreement if one is in place. The Security Risk Register must be available for audit when reasonably required by TfL. |  |  |  |
| Rationale: |  |  |  |  |

| Service Asset Register |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-385 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure a Service Asset Register is produced and maintained throughout the Term and that all Service Assets are documented in an appropriate manner in the service asset register and shall identify the criticality of the relevant supplier assets in the delivery of this Agreement. The Supplier service asset register must be available for audit when reasonably required by TfL. |  |  |  |
| Rationale: |  |  |  |  |

| Supporting Policies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-386 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure supporting policies are implemented (where relevant) and communicated with Supplier personnel. |  |  |  |
| Rationale: |  |  |  |  |

| Security Threats |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-387 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier and its sub-contractors shall always ensure that security threats to TfL Data, the Supplier's IT environment, and the Services are minimised and mitigated. |  |  |  |
| Rationale: |  |  |  |  |

| Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-388 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure the Services fully comply at all times with the security requirements set out in this Specification, Good Industry Practice and Cyber Essential Scheme |  |  |  |
| Rationale: |  |  |  |  |

### Security Requirements

#### Risk Management

| Risk Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-389 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall implement a Risk Management process for the purpose of identifying, assessing, mitigating, monitoring, and reporting upon cyber security risks. |  |  |  |
| Rationale: |  |  |  |  |

#### Engagement & Training

| Authorising Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-390 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall screen all personnel prior to the supplier authorising access to the Service the Supplier is managing on TfL's behalf. (Security Reporting – Joiners, Movers and Leavers Notification) |  |  |  |
| Rationale: |  |  |  |  |

| Criminal Checks |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-391 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall conduct criminal record checks on all Supplier personnel who have access to any Systems the Supplier operating on behalf of or TfL. The level of checks should be consistent with British Standard 7858 (BS7858 - https://assets.publishing.service.gov.uk/government/uploads/system/uploads/attachment_data/file/417085/BS7858.pdf |  |  |  |
| Rationale: |  |  |  |  |

| Vetting Personnel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-392 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure all Supplier personnel have been vetted in accordance with HMG Guidance for overseas individuals - https://www.gov.uk/government/publications/criminal-records-checks-for-overseas-applicants |  |  |  |
| Rationale: |  |  |  |  |

| Security Risk |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-393 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall make a reasonable determination of whether the individual constitutes an unreasonable security risk taking into consideration the duties of the individual, the type and sensitivity of information to which the individual may be exposed, and all applicable laws. |  |  |  |
| Rationale: |  |  |  |  |

| Crimnial Offenses |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-394 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure all Supplier personnel proactively disclose criminal offences to the supplier unless prohibited by applicable law. |  |  |  |
| Rationale: |  |  |  |  |

| Training |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-395 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure that all persons employed or retained to perform the Services receive security awareness training, annually and supervision at a level and in substance that is appropriate to that person's position and the Supplier's obligations under this Agreement. |  |  |  |
| Rationale: |  |  |  |  |

| Protected Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-396 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall not permit any person the supplier hires or uses to access or obtain any Protected Information unless that person is contractually bound to the supplier in writing to keep Protected Information confidential on terms no less protective than the terms applicable to the supplier under this Agreement. |  |  |  |
| Rationale: |  |  |  |  |

#### Architecture and Configuration

| Asset Hardening and Secure Configuration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-401 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall adhere to industry standards for Asset Hardening and Secure Configuration. |  |  |  |
| Rationale: |  |  |  |  |

| Uneccessary Utilities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-403 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall remove or disable unnecessary utilities from operating systems, configurations and restrict access rights to least privilege. |  |  |  |
| Rationale: |  |  |  |  |

| Passwords |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-404 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure that any preconfigured passwords delivered with any Service Assets are changed prior to their implementation for use in the Service environment. |  |  |  |
| Rationale: |  |  |  |  |

| Password Complexity |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-405 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | Where the Supplier manages User Authentication controls for the personnel to access the Service, the Supplier shall enforce minimum password complexity, such as requiring passwords to be case sensitive, or requiring passwords to contain a minimum of twelve characters and a combination of upper-case letters, lower-case letters, numbers, and/or special characters. |  |  |  |
| Rationale: |  |  |  |  |

| Change of Password |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-406 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | Where the Supplier manages User Authentication controls for the personnel to access the Service, the Supplier shall require regular change of passwords at predetermined intervals, and which limit reuse. |  |  |  |
| Rationale: |  |  |  |  |

| Multi-Factor Authentication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-407 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | Where the Supplier manages User Authentication controls for the personnel to access the Service, the Supplier shall require multi-factor authentication for privileged access. |  |  |  |
| Rationale: |  |  |  |  |

| Applications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-408 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure that Web-based and Mobile applications used to store, receive, send, control or access TfL Data are monitored, controlled and secure. |  |  |  |
| Rationale: |  |  |  |  |

| Change Managent Process |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-409 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall implement and manage a Change Management process to manage changes that occur within the Service environment. |  |  |  |
| Rationale: |  |  |  |  |

#### Asset Management

| Inventory of Assets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-397 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall maintain an inventory of Assets (TfL Information, System and Service Assets). |  |  |  |
| Rationale: |  |  |  |  |

| Disposing of Assets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-398 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall use secure methods when disposing of Assets. |  |  |  |
| Rationale: |  |  |  |  |

| Record Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-399 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall maintain records of the disposal of Assets. |  |  |  |
| Rationale: |  |  |  |  |

| Change Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-400 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall manage changes to any Asset through the operation of the supplier's Change Management process, |  |  |  |
| Rationale: |  |  |  |  |

| Asset Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-402 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure that Assets used to access or manage TfL Data and Services are under the management authority of the supplier or TfL and have a standard set of security policy configuration deployed and enforced. These assets must be placed into a 'known good' state prior to being provisioned into the environment of the supplier. Unless otherwise agreed with the TfL in writing, all supplier assets are expected to meet the set of security requirements. |  |  |  |
| Rationale: |  |  |  |  |

#### Vulnerability Management

| Vulnerability Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-410 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall implement a Vulnerability Management program, which will include the following elements: |  |  |  |
| Rationale: |  |  |  |  |

| Security Patches |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-411 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall ensure security patches will be applied to Service Assets as soon as possible in line with Vendor recommendations in accordance with the Change Management process. |  |  |  |
| Rationale: |  |  |  |  |

| Anti-malware/anti-virus |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-412 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall maintain industry standard processes for defending against Malware / Trojans / Virus infections. |  |  |  |
| Rationale: |  |  |  |  |

| Anti-malware/anti-virus updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-413 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall maintain a program of anti-malware/anti-virus updates to keep provisioned Assets free of infection. |  |  |  |
| Rationale: |  |  |  |  |

#### Supply Chain Security

| Supplier Inventory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-414 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall maintain an inventory of all suppliers / Sub-contractors it utilises. This should be in the form of a supplier matrix with roles and responsibilities defined. |  |  |  |
| Rationale: |  |  |  |  |

| Compliance with Standards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-415 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall ensure that its suppliers and subcontractors involved in the provision of Services meet or exceed the standards set forth in this Specification. |  |  |  |
| Rationale: |  |  |  |  |

| Security Assessments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-416 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The Supplier shall conduct security assessments upon its supply chain to ensure all suppliers of Services to its present a Low or no Risk to the Service it is providing to TfL. |  |  |  |
| Rationale: |  |  |  |  |

#### Incident Management

| Incident Management Plan |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-417 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The supplier will implement and maintain an Incident Management Plan which will be used to respond on breaches to the System. The Incident Management plan will include the following Definition of roles, responsibilities and communication and contact strategies in the event of a compromise, - including notification of customers; - Specific incident response procedures; - Analysis of Legal requirements for reporting compromises; - Coverage of all critical system components; - Regular review and testing of the plan; - Incident management Support personnel that is available to support the Service; - Training of supplier personnel; - Inclusion of alerts from all security monitoring systems; - Modification and evolution of the plan according to lessons learned and to incorporate industry developments. |  |  |  |
| Rationale: |  |  |  |  |

#### Logging and Monitoring

| Audit Trails |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-418 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall ensure that audit trails for critical systems are kept for a period in-line with industry standards. |  |  |  |
| Rationale: |  |  |  |  |

#### Data Security

| Data at Rest |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-419 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall implement and maintain encryption of TfL Data while it is at Rest |  |  |  |
| Rationale: |  |  |  |  |

| Data in Transit |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-420 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall implement and maintain encryption of TfL Data while it is in Transit. |  |  |  |
| Rationale: |  |  |  |  |

#### Compliance

| Compliance Plan |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-426 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier shall have a documented compliance plan and conduct regular reviews (at least annually) to ensure that the security of TfL Information cannot be compromised. |  |  |  |
| Rationale: |  |  |  |  |

| Information Security Questionnaire |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-427 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | TfL may require the supplier to complete an Information Security Questionnaire as part of our supplier review process, which may be subject to a full physical and logical information security review at all relevant supplier locations in accordance with the Right to Audit section of the Agreement |  |  |  |
| Rationale: |  |  |  |  |

| Requests for Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-428 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Unless otherwise stated, the supplier shall respond to any requests for information or data to be provided to TfL in relation to the supplier services within 30 days of notice. |  |  |  |
| Rationale: |  |  |  |  |

#### Identity and Access Management

| Access Control Policies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-421 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where the supplier's personnel access any part of the suppliers Systems that may contain TFL Data, the supplier shall implement access control policies and procedures that address onboarding, off- boarding, transition between roles, regular access reviews, limitations and usage control of administrator privileges, and inactivity timeouts. |  |  |  |
| Rationale: |  |  |  |  |

| Inventory of Computer Accounts |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-422 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where the supplier's personnel access any part of the suppliers Systems that may contain TFL Data, the supplier shall review the inventory of computer accounts on a regular basis to identify dormant, fictitious, or unused accounts. |  |  |  |
| Rationale: |  |  |  |  |

| Principles of Least Privilege |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-423 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where the supplier's personnel access any part of the suppliers Systems that may contain TFL Data, the supplier shall enforce principles of "least privilege" and "need to know". |  |  |  |
| Rationale: |  |  |  |  |

| User Access Rights |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-424 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where the supplier's personnel access any part of the suppliers Systems that may contain TFL Data, the supplier shall review user access rights on a regular basis to identify excessive privileges. |  |  |  |
| Rationale: |  |  |  |  |

| Acceptable Access |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-425 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | Where the supplier's personnel access any part of the suppliers Systems that may contain TFL Data, the supplier ensure that supplier personnel are informed of what constitutes acceptable access of Operational or IT technology, Data and Networks and the consequences of non-compliance. |  |  |  |
| Rationale: |  |  |  |  |

#### Business Continuity

| Business Continuity Policy |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-429 |  | Priority: Must have | Status: Final | Version: v5 |
| Description: | The supplier shall provide a copy of their Business Continuity Policy and a Business Continuity Plan that demonstrates how they will maintain the contracted service level in the event of an emergency. Business Continuity Policy and Planning must align with the best practice detailed in the standard ISO/IEC 22301 Business Continuity Management. |  |  |  |
| Rationale: |  |  |  |  |

| Policy and Plan |  |  |  |  |
| --- | --- | --- | --- | --- |
| Requirement ID: CHD-REQ-430 |  | Priority: Must have | Status: Final | Version: v4 |
| Description: | The supplier's Policy and Plan shall be subject to an annual review by the supplier and the updated documents will be shared with TfL no more than 13 months following the previous submission. TfL, acting reasonably, reserve the right to request further information relating to the suppliers Business Continuity arrangements, including but not limited to exercise schedules and reports, and the suppliers will make all efforts to respond promptly to such information requests |  |  |  |
| Rationale: |  |  |  |  |

# Glossary of Terms

| CAT A, B, D Devices Matrix |  |  |
| --- | --- | --- |
| Term ID: CHD-TERM-19 | Status: Final | Version: v3 |

| Description: | Feature; CAT A; CAT B; CAT D (was based upon CAT C without note unit). Coin Storage; Yes, 5,500 Coins + recycling capacity; Yes, 5,500 Coins + recycling capacity; Yes, 5,500 Coins + recycling capacity. Minimum Coin; TfL requirement:; RCS800:; TfL requirement:; RCS400:; TfL requirement:; RCS400:. £2 = 2500 coins; 2,500; £5,000; £2 = 2000 coins; 2,000; £4,000; £2 = 1500 coins; 2,000; £4,000. £1 = 2500 coins; 2,500; £2,500; £1 = 2500 coins; 2,500; £2,500; £1 = 1500 coins; 2,500; £2,500. 50p = 1500 coins; 1,900; £950; 50p = 1500 coins; 1,500; £750; 50p = 1000 coins; 1,500; £750. 20p = 1500 coins; 2,900; £580; 20p = 1500 coins; 1,500; £300; 20p = 1000 coins; 1,500; £300. 10p = 1500 coins; 2,600; £260; 10p = 1500 coins; 1,500; £150; 10p = 1000 coins; 1,500; £150. Minimum Note; TfL requirement at tender £60,000; 8,000 note capacity; £101,040; TfL requirement at tender £50,000; 5,000 Note capacity; £63,150; Not applicable. Value based on TfL average note value of £12.63 (email 01/05/25); Value based on TfL average note value of £12.63 (email 01/05/25). Recycling Coin; Yes; Yes; Yes. Recycling Capacity (Coins); TfL requirement: At least £8,700; Actual:; TfL requirement: At least £7,700; Actual:; TfL requirement: At least £5,300; Actual:. £9,290; £7,700; £7,700. Recycling Note; Yes; Yes; No. Recycling Capacity (Notes); 2,000 notes; 2,000 notes; Not applicable. Storage Bags (Notes); At least 2; At least 1; Not applicable. |
| --- | --- |
| Abbreviation: |  |

| Change Types |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-4 |  | Status: Final | Version: v2 |
| Description: | There are 3 types of Changes: - Standard Changes - Requests that are frequently performed, regular and routine in nature - Non-standard Change - Encompasses the majority of Changes and must follow the complete Change Management process, including review, planning and approval - Emergency Change - Change in response to a Severity 1 or Severity 2 Production Incident to resolve/restore service |  |  |
| Abbreviation: |  |  |  |

| Company |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-18 |  | Status: Final | Version: v4 |
| Description: | For the purpose of this document, where Company is used, it is referring to Transport for London (TfL) |  |  |
| Abbreviation: |  |  |  |

| Failover |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-5 |  | Status: Final | Version: v2 |
| Description: | Switching to a redundant or standby system, server, application, network or service in the event of failure or abnormal termination of the previously active system, server, application, network or service. |  |  |
| Abbreviation: |  |  |  |

| Incident |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-6 |  | Status: Final | Version: v4 |
| Description: | An Incident is viewed as a disruption to the availability or performance of the solution. In addition, failure of a component that is yet to impact either the availability or the performance of the solution is also considered an Incident. First line support -The Supplier will provide first line support to users of the solution. The exact scope of this shall be determined by the capabilities of the solution. Second line support - The Supplier will provide second line support to users of the solution. The exact scope of this shall be determined by capabilities of the solution. Third line support - The Supplier will provide third line support to users of the solution. The exact scope of this shall be determined by capabilities of the solution.”. Service desk hours - The Service desk hours shall be 24/7. |  |  |
| Abbreviation: |  |  |  |

| On-Premise |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-7 |  | Status: Final | Version: v2 |
| Description: | A Solution hosted in-house and usually supported by a third party |  |  |
| Abbreviation: |  |  |  |

| Primary Service Desk Hours |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-17 |  | Status: Final | Version: v3 |
| Description: | The agreed hours when the Primary Service Desk is to be available to support the business (if different from operating hours). |  |  |
| Abbreviation: |  |  |  |

| Service Criticality: Administrative Services |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-8 |  | Status: Final | Version: v2 |
| Description: | - The system / application is important to the business or a small group - Infrastructure should be available with disruption causing minimal impact - The company can withstand temporary data unavailability - Limited financial or revenue loss from downtime |  |  |
| Abbreviation: | Tier 4 |  |  |

| Service Criticality: Business Critical |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-9 |  | Status: Final | Version: v2 |
| Description: | - The system/application is critical to the business of a department or large group - Business unit custom or packaged applications required to manage back-office processes and collect, protect and enhance revenue - Data can be unavailable for very short periods with minimal loss - Infrastructure should be highly available with automatic recovery and minimal disruption - Upstream infrastructure / utilities should conform to same hosting characteristics |  |  |
| Abbreviation: | Tier 2 |  |  |

| Service Criticality: Business Operational |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-10 |  | Status: Final | Version: v2 |
| Description: | - The system/application is essential to the business of a department or large group - Business unit custom or packaged applications required to manage back-office processes and collect, protect and enhance revenue - Data can be unavailable for very short periods - Infrastructure should be available with manual recovery and minimal disruption - Upstream infrastructure / utilities should conform to same hosting characteristics |  |  |
| Abbreviation: | Tier 3 |  |  |

| Service Criticality: Mission Critical |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-11 |  | Status: Final | Version: v2 |
| Description: | - The system/application is mission critical to all or most departments of the company - The system/application is safety critical to all or most departments of the company Corporate / Business Unit custom or packaged application - Mandatory to support customers; deliver products, services, generate revenues - Data should always be correct and accessible - Infrastructure should be continuously available and accessible at all times - Upstream infrastructure / utilities should conform to same hosting characteristics |  |  |
| Abbreviation: | Tier 1 |  |  |

| Severity 1 (Major Incident) |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-12 |  | Status: Final | Version: v2 |
| Description: | Where one or more of the following apply: - The entirety of a service is unavailable - Service performance is degraded to such a point that the service is deemed unusable. - The overall security of the TfL estate is significantly compromised. - There is a significant safety risk to TfL staff, third parties and/or members of the public. - There is a significant risk of revenue loss - There is a significant risk of reputational loss |  |  |
| Abbreviation: | SEV 1 |  |  |

| Severity 2 (Major Incident) |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-13 |  | Status: Final | Version: v2 |
| Description: | Where one or more of the following apply: - Key functionality within a service is unavailable, however the service is up. - Not enough users are able to access the service to ensure completion of critical activities - Resilience has been degraded beyond the agreed tiering level for that service, however the service is still fully available to users. |  |  |
| Abbreviation: | SEV 2 |  |  |

| Severity 3 |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-14 |  | Status: Final | Version: v2 |
| Description: | Where one or more of the following apply: - A user or users are unable to perform certain functions, however it does not prevent critical activities being completed. - EUC equipment faults - A single user has an issue with an application or mobile device |  |  |
| Abbreviation: | SEV 3 |  |  |

| Severity 4 - Information Request (Non-Service Impacting) |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-15 |  | Status: Final | Version: v2 |
| Description: | No Service Impact - A request for advice and guidance on a non-service affecting issue or one which is not related to IM - Someone has forgotten their Password and requires a Password Reset - Someone has put the wrong password in and has locked their account and requires an account unlock |  |  |
| Abbreviation: | SEV 4 |  |  |

| Unplanned Interruption |  |  |  |
| --- | --- | --- | --- |
| Term ID: CHD-TERM-16 |  | Status: Final | Version: v2 |
| Description: |  |  |  |
| Abbreviation: | A disaster, failure or comparable event that results in a reduction in the quality of an IT service |  |  |