Grove Ferry refuge

Location; https://w3w.co/offices.linked.tucked

Reedbed Pool:

• Semicircular, open to existing ditch along flat side.

• Pool bed surface area: Equal to the refuge top area.

• Water depth: Minimum 1.5m at completion.

• Side Slopes: Minimum 35°.

• Reedbed/Pool interface (“back” of pool): series of “fingers” (higher formed, using spoil from lower areas).

Refuge Pool:

• Water depth: Minimum 1.5m at completion at ditch interface.

• Deep channel to extend to refuge base

o 1.5m minimum water depth

o 1m wide

o Bridge to span deep channel for future machinery use

• Shallow area at mound base for future machinery access.

• Slopes gentle for safe access.

Viewing Refuge:

• Top Surface Area: Minimum 50m², levelled and smooth.

• Target height: 3.8 m AOD (raise approx. 1.8 m, contractor to confirm).

• An entrance channel to be added where the refuge meets the refuge pool.

o A bridge to be installed over the entrance channel.

Viewing refuge slopes:

• Minimum 1:3 gradient, much steeper on pool-facing edge.

Access Slope:

• From existing path to mound top.

• Gradient: ≤1:12.

Compartment Culvert:

• Lower area to create 400 mm water depth over 5 m length.

• Spoil placed in mound on far side to deter trespass.