Harrisons refuge

Shallow Marsh zone – extensive as time allows

Access ramp

Culvert + bridge

Deep pool

Dry reedbed zone

Flexi pipe outlet

Overflow rill

Refuge

Harrisons Refuge

Location; https://w3w.co/gracing.gravy.gaps

Linking Ditch Culvert: linking existing ditch to deep pool at bed of pool

• Width: 5 m.

• Pipe: 450 mm straight culvert pipe.

• Culvert slopes (into linking ditch): 20°.

• Length – culvert length only.

Deep Pool:

• Steep sided.

• Flexi pipe to be installed at base of pool, leading to reedbed area at “back” (furthest from existing ditch) of pool.

• Remaining specifications to be as standard beaver refuge specifications.

Beaver refuge: as standard beaver refuge specifications.

Shallow Marsh Zone:

• Surrounding sides and back of refuge but with high, dry reedbed zone (5m) (extending 5m out from refuge base) except deep pool N.B goes round access ramp.

• Water depth: Minimum 500mm at completion.

• Slope: Gentle rise to adjoining areas, minimum 20° (except at refuge).