Reedbed Hide refuge

Refuge

Deep Pool

Shallow marsh fringe

New ditch

culvert

Spoil from ditch & pool

Deep Pool

Ditch to restore

Reedbed Hide refuge

Location https://w3w.co/cookie.contain.leaned

Ditch Restoration:

• Length: ~75 m.

• Water depth: 1.5m (when lake level = 1.3 m AOD).

• Open water zone: ≥1.5 m wide.

• Nearside edge: 5 m wide lowered (retain reed rhizome).

• Spoil formed into linear bunds (max side slope 25°).

Deep Pool:

• Water Depth: ≥1.5 m (lake level = 1.3 m AOD).

• Base area: ≥30 m².

• Slopes: 45° except at refuge junction.

• Spoil used for adjacent mounds and refuge creation.

Beaver refuge: as standard beaver refuge specifications.

Shallow Pool Margin:

• As standard beaver refuge specifications.

• Scrub clearance (cut to ground, stumps remain).

Access Ramps:

• As standard beaver refuge specifications.

New Ditch:

• Length: ~50 m.

• Water depth: ≥1.5 m water (when lake level = 1.3 m AOD).

• Marginal zones: 3 m wide.

• Culvert: 600 mm double-walled pipe for bogmaster access.