Specification:

Enclosed Platform Lift  without a cabin

Principal Data:

Type: Chain Drive Platform Lift

Drive System: Encapsulated Guided Chain drive

Lift Usage: Impaired mobility use

Capacity: 500 Kg (6 persons)

Speed: 0.15 m/s

Travel: 6233mm

Number of stops: 3

Entrances: 3

Configuration: Open front configuration

Space Requirement / floor aperture:

Width: 1280mm (-0 +15mm)

Depth: 1590mm (-0 +15mm)

Pit Depth: 60mm (-0 +15mm)

Headroom: 2230mm Minimum

Lift Enclosure:

Construction: Cladding on 3 sides in white vinyl coated double skinned steel panels filled with EPS polystyrene

Lighting: Automatic LED Cabin lighting

Lift Cabin:

Width: 1120mm

Depth: 1480mm

Height: 2000mm

Handrail: Stainless Steel to one side on the console

Flooring: Grey vinyl (Anti Slip)

Lighting: Controls illuminated (battery backup) LED

Electrical details and Power requirements:

Supply: 230v – 240v, 1ph, 16A, 50Hz

Supply location: Lockable isolator within 2 metres and in view of the electrical cabinet location to be agreed

Emergency lowering: Electrical emergency lowering controlled from electrical cabinet and manual winding handle

Initiation: Soft start

Control Equipment:

Control system: Microprocessor based with service memory facility

Operation: Platform – constant pressure button / landings – one touch

Face plate: Natural anodised aluminium with coloured text

Push buttons: Flush & Tactile

Landings: Call button with halo indication

Alarm: External Audible Sounder

Pit Prop: Manual with electrical interlock

Audible Indication: Voice synthesiser with directional indication on platform

Fire Alarm interface: Volt free contacts for FAS interface

Call and send facility: Initiated from platform

4G GSM type Auto-dialler providing two-way voice communications

Warranty:

12 months comprehensive maintenance and breakdown cover including 4 service visits.

Standard:

The unit type is to be tested and CE marked in accordance with the Revised European Machinery Directive (2006/42/EC).

Documentation:

1 digital copy of (O&M) Operation & Maintenance manual

Requirements:

Essential - Enhanced DBS - preferably with Child Baring.

Essential - The lift and motors need to be rat proof.

Lockable landing entrance protection hoardings erected at all floors.

Existing Lift: To be removed and disposed of the existing lift using your waste carrier’s licence.

Provide provision sum/estimate for any associated building and electrical costs that will be required as part of the installation.

Optional

Repositioning of the control unit from classroom PC0.08 into a corridor or another suitable location.