West Yorkshire Fire & Rescue Service Bradford Fire Station – Dormitory and Shower Refurbishment Pre-Construction Information Project No: C.20733 14 March 2025

Revision control

| Rev | Date | Description of revision | Prepared by | Checked by | Authorised by |
| --- | --- | --- | --- | --- | --- |
| 01 | 14 Mar 2025 | Initial Report | JT | EK | RY |

Distribution list

Controlled copies have been distributed to the following personnel:

| Copy No | Name of holder | Company | Date issued |
| --- | --- | --- | --- |
| 001 |  |  |  |

| Project number: | C.20733 |
| --- | --- |
| Originated by: | Josef Turner |
| Signature: |  |

Contents

Introduction	1

0.1	Notification of Project	1

Project Details	3

1.1	Description of the Project	3

1.2	Site location and general environment	3

1.3	Timescale	9

1.4	The Project Team	9

1.5	Extent and location of existing records and plans	10

Client’s considerations, planning and management requirements:	13

2.1	Client Brief and Safety Goals	13

2.2	Communication	13

2.3	Security of the site	14

2.4	Welfare provision and temporary facilities for Station personnel	14

2.5	Overlap with the Client’s undertaking and other users of the site	15

2.6	Client requirements	15

2.7	Client permit-to-work systems	15

2.8	Fire precautions and emergency procedures	16

Project Health and Safety Hazards	18

3.1	Safety hazards	18

3.2	Boundaries and general access, including temporary access	18

3.3	Restrictions on deliveries, vehicular traffic or waste collection or storage	18

3.4	Adjacent Land Uses	19

3.5	Existing storage of hazardous materials	19

3.6	Location of Existing Services	19

3.7	Ground conditions	19

3.8	Information about existing structures	19

3.9	Hazard considerations	19

3.10	Health hazards	20

The Construction Phase Plan	22

4.1	Generally	22

The Health and Safety File	24

Appendices

Appendix A - Anticipated Health and Safety Standards / Client Rules	30

Appendix B - Construction Phase Plan Criteria	31

Appendix C - Design Risk Assessment	33

Appendix D - Work Involving Particular Risks	34

Introduction

Bradford Fire Station has been identified as having non-compliant washroom and private rest room facilities when gauged against current West Yorkshire Fire & Rescue Service (WYFRS) Design Guide specifications. This document relates to a proposal to renew existing services and provide compliant unisex wash facilities on all floors, along with an amended layout to accommodate individual rest rooms for operational staff. Ground floor wash facilities are to be made accessible directly via the Appliance Bay to prevent operational staff having to travel through the building to wash off when returning from an incident.

The purpose of this document is to advise prospective or appointed Contractors, including the Principal Contractor and others, of key project health and safety information.

The document represents the “Pre-Construction Information” (PCI) as defined by the Construction (Design and Management) Regulations 2015, reference Regulations 4(4) and 11(6)(a) & (b).

The document advises on key descriptive and background information but restricts itself to that information deemed to be significant or unusual omitting reference to those matters any competent contractor could be expected to anticipate.

It contains information from the client about the site for designers and contractors as well as information from the designer about the risks for contractors, maintenance personnel and future users of the building. The PCI collects and passes on information about significant Health and Safety risks which the principal contractor (PC) will have to manage during the construction phase of the project.

The appointed contractor shall assume the duties of the Principal Contractor under Regulations 13 and 14.

Work on site should not commence until the Principal Contractor has prepared a suitable Construction Phase Plan to advise contractors and others of construction site management arrangements and significant safety issues.

The Principal Contractor’s initial Construction Phase Plan is to be forwarded to WYFRS so that it can be assessed for its adequacy prior to any works starting on site. The Principal Contractor is under a legal duty to administer, implement and update their Construction Phase Plan to reflect changes in design or circumstances throughout the construction period.

## Notification of Project

With reference to Regulation 6 it has been assessed that this project will require notification to the Health and Safety Executive as the project will continue for more than 30 working days and/or have more than 20 operatives on site simultaneously while surpassing 500 working person days.

Workplace (Health, Safety and Welfare) Regulations 1992

The properties will be used as a workplace and, therefore, the Workplace (Health, Safety and Welfare) Regulations 1992 is applicable to this project.

Project Details:

# Project Details

## Description of the Project

The works to Bradford Fire Station comprise the refurbishment of existing WC areas (GF-2F) to provide new unisex shower room units, specifically on the GF for operational staff to use when returning from an incident, facilitated by the formation of a new structural opening between the appliance bay and new shower/WC facilities. 1F dormitory layouts will be altered via the removal and subsequent erection of internal partition walls to provide 12No. individual rest rooms. 1F/2F shower and WC facilities will be refreshed and made unisex. Renewal of small power, data, lighting, ventilation, turnout provisions and heating. The contractors work scope will comprise:

- Establishment of site set up comprising office, welfare accommodation, storage and waste disposal.

- Undertaking a comprehensive survey of the existing site to familiarise themselves with the scope of the works, to identify the remedial works and to obtain all necessary measurements and information to facilitate the required design, specification and procurement of materials in a timely manner.

- Manage site separation from live circulation spaces to prevent obstruction of station operation and manage access control effectively to prevent unauthorised personnel onto the site.

- Replace existing tile, vinyl and carpet floor coverings to the identified rooms on plan, including coved skirtings and thresholds.

- Remove existing partition walls, ceilings and doors/frames as indicated on plan. Form new opening between the existing Appliance Bay and new GF operational shower/WC areas and install structural support as specified.

- Fit new suspended ceilings, stud partition walls and door-sets to form new Unisex WCs. Apply tiled and emulsion finishes to plasterwork as indicated.

- Replace indicated doors with new Certifire certified FD30/60 doorsets, including associated frames, skirtings, architraves and ironmongery.

- Supply and install new sanitaryware, including toilets, wash-hand basins, showers and associated waste and supply pipework.

- Supply, install and commission the testing of new lighting, ventilation and other MEP items as specified.

- Firestop penetrations through compartment walls sufficiently with Part B compliant firestopping materials and methods.

- Removal of temporary works and accommodation.

- Sufficiently cooperating, coordinating, communicating and consulting with all relevant parties involved to ensure that all aspects of the project are managed appropriately with respect to Health & Safety.

## Site location and general environment

Site Location: Bradford Fire Station, 540 Leeds Road, Bradford BD3 9SB

General Environment: The site is a wholetime fire station with two appliances situated close to the centre of Bradford city, near several shops and residential streets. The station is accessed directly off Leeds Road with a rear gated car park (including a lower ground floor undercroft with additional staff parking) accessed off Maud Street. Crews occupy the station full-time so will be in close proximity to the works areas at all times during the contract period. Bradford District staff also operate full-time in the 2F office space. Work should be appropriately phased so as not to inhibit the daily operation of the station and its crews during standard operational activities including, but not limited to: Preparing food, resting, training, turning out and returning from an incident (including all associated cleaning protocols). Gated car parking is available to the rear of the station.

Figure 1: Aerial view extracted from Google Maps

Figure 2: Internal floor plans of phased work areas (GF-2F).

| Rear station car park view from Maud Street (image extracted via Google Maps) | Station front entrance view from Bondgate (image extracted via Google Maps) |
| --- | --- |

Figure 3: Bradford Fire Station externals

| GC.03 Corridor | G.01 Community Room |
| --- | --- |
| G.24 Office | G.15 Appliance Bay |
| G.07 Toilets | G.09 Disabled WC |
| G.15 Muster Bay | 1.04 Male Showers |
| 1.11 Female Showers | 1.13 4-bed Dormitory |
| 1.02 Store Room | 1.07 Drying Room |
| 1C.02/03 Dormitory Corridors | 2.01 Office Space |
| 2C.02 Corridor | 2.07 Male Toilets |
| 2.06 Female Toilets | 2.08 Disabled WC |

Figure 4- Internal Work Areas

## Timescale

| Key Event | Date/Weeks |
| --- | --- |
| Minimum mobilisation period for preparation and planning of construction | 2 Weeks |
| Anticipated start date for the construction phase | 19 May 2025 |
| Anticipated duration of the construction phase | 16 weeks |
| Anticipated completion date | 08 September 2025 |

Works sequencing and phasing to be discussed and agreed at the Pre-Commencement Meeting.

## The Project Team

| Project Team Member | Company Name and Address | Contact Details |
| --- | --- | --- |
| Client | West Yorkshire Fire & Rescue Service Oakroyd Hall, Birkenshaw, BD11 2DY | Contact Name: Josef Turner Mobile No: +44 7760 992 135 Email Address: Josef.Turner@westyorksfire.gov.uk |
| Project Manager | West Yorkshire Fire & Rescue Service Oakroyd Hall, Birkenshaw, BD11 2DY | Contact Name: Josef Turner Mobile No: +44 7760 992 135 Email Address: Josef.Turner@westyorksfire.gov.uk |
| Contract Administrator | West Yorkshire Fire & Rescue Service Oakroyd Hall, Birkenshaw, BD11 2DY | Contact Name: Josef Turner Mobile No: +44 7760 992 135 Email Address: Josef.Turner@westyorksfire.gov.uk |
| Principal Designer | West Yorkshire Fire & Rescue Service Oakroyd Hall, Birkenshaw, BD11 2DY | Contact Name: Josef Turner Mobile No: +44 7760 992 135 Email Address: Josef.Turner@westyorksfire.gov.uk |
| Mechanical & Electrical Engineer | Robert Whetham Associates Limited Castle Hill House, 31 Mill Brow, Chadderton, Oldham, OL1 2RT | Contact Name: Robert Whetham Mobile No: +44 7899 958 929 Email Address: rob@robertwhetham.co.uk |
| Structural Engineer | JNP Group Woodvale House, Woodvale Road, Brighouse, HD6 4AB | Contact Name: Ksenija Bicko Mobile No: +44 7848 121 101 Email Address: Neil.Taylor@jnpgroup.co.uk |
| Principal Contractor | TBC | Contact Name: TBC Tel No: TBC Mobile No: TBC Email Address: TBC |

## Extent and location of existing records and plans

The existing plans and drawings relevant to the contract have been provided within the tender package. The Principal Contractor shall not solely rely upon the information contained within these documents but shall visit site and examine the design proposals to satisfy themselves that the information is sufficiently detailed to allow identification of any hazards that may exist, and they must immediately raise any concerns regarding the quality or accuracy of the information supplied.

Asbestos – No Asbestos is anticipated to be present onsite as the building was constructed between 2006-2007. However, if any materials are discovered which are not identified and which are suspected to contain asbestos, then the contractor must immediately cease works within the vicinity to avoid disturbance and inform the Project Manager and/or Inspectas immediately.

All personnel, who by the nature of their works could be exposed to Asbestos, must have undertaken an appropriate level of Asbestos Awareness training at least annually (UKATA or otherwise certified). The Principal Contractor must ensure that procedures are implemented and brought to the attention of all site personnel via the induction process to stop work and report, if suspect materials are detected.

| Information Type | Document Title and Reference | Held by | Comments |
| --- | --- | --- | --- |
| Existing ‘As Built’ Record | 0040-A-00-P-LGF-EXISTING-01 0040-A-00-P-GF.EXISTING-T01 0040-A-00-P-1F.EXISTING-T01 0040-A-00-P-2F.EXISTING-T01 | WYFRS | Refer to Tender Documentation |
| Health & Safety File | Bradford Fire Station H&S File (Paper Copy) | WYFRS | Paper copy available for inspection upon request. |
| Utilities/ Services Location Drawings | RWA Design Package | WYFRS | Refer to Tender Documentation |
| Asbestos Register/ Existing Surveys & Reports | N/A | N/A | Not applicable |
| Ground Investigation Report | N/A | - | Not required |
| Structural Inspection Report | JNP Group Structural Details | JNP Group / WYFRS | Refer to Tender Documentation |
| CCTV Drainage Survey | N/A | - | Not required |
| Environmental Reports | N/A | N/A | - |
| Fire Action Report | N/A | N/A | Not required |
| Traffic/ Pedestrian Management Plan | - | - | To be developed by Principal Contractor |
| Condition Survey | Capita | WYFRS | Available on request |

Client’s Considerations, Planning and Management Requirements:

# Client’s considerations, planning and management requirements:

## Client Brief and Safety Goals

The project health and safety goals of the Client and the project team is to achieve the following:

- No accidents on site or adjacent to the site.

- No occupational ill health arising from the project.

- No environmental damage.

- Minimise disruption to the local community.

- Establish a site set up that excludes unauthorised persons, especially children, from the construction site.

- Provide safe access and egress from places of work.

- Provide workplaces that are free from risks to the health and safety of persons at work, so far as is reasonably practicable.

- Complete the project within the planned contract timescale to the agreed design and specification requirements with due regard to the welfare of the contractor’s workforce and without causing any significant risk to the health, safety and welfare of others.

The Principal Contractor is required to put in place suitable measures to achieve the above in respect to design and construction responsibilities that are under their control.

The Construction site, compound and storage, welfare and messing facilities and working areas must fully comply with UK Government & Health England current and up to date guidelines and procedures. This may be achieved by adopting the Construction Leadership Council – Site Operating Procedures Version 6 – 20 October 2020 & CITB Site Operating Procedures Compliance checklist CC01, Weekly Site Operating procedures checklist CC04 & Five Steps Poster fully displayed and operated on site.

A primary objective of all duty holders is to cooperate, communicate and coordinate and thereby remove and minimise the risk of injury or incident, to ensure the legal standards for safety and health are met and best practice is always prioritised so that all work is undertaken safely.

## Communication

Communication is a key element of any successful project. The Principal Contractor shall ensure that all those working on this project are advised of the contents of this Pre-Construction Information document, the Construction Phase Plan, Site Rules and all other health and safety procedures that apply.

Daily communication with the Station Watch Commander and/or Project Manager is essential to register presence on site and agree the scope and works sequenced for the day ahead and establish any concurrent works within the estate to avoid overlap.

All formal communications, instructions, technical queries etc. are to be routed via the Project Manager.

Designers, including those working for the Principal Contractor, have a duty to ensure the design is co-ordinated for health and safety. Design development details, changes, instructions etc. are to be copied to the Principal Designer for review and when necessary, comment. To facilitate this, the Principal Contractor shall identify appointed designers, provide designers with all necessary information and thereafter issue in good time design information.

All parties to the project are to co-operate and co-ordinate on matters relating to health and safety throughout the project. Future progress and technical meetings will be agreed at the pre-commencement meeting.

The Principal Contractor shall manage, monitor, and review on an ongoing basis health and safety implementation and performance and copy reports to the Principal Designer.

Health and safety is to be an agenda item on all primary construction co-ordination meetings.

## Security of the site

All construction personnel must take cognisance of the environment that is a live, wholetime fire station and take care not to infringe on the operation of the station when entering and exiting the site. As the site lies in close proximity to public areas, particular account shall be taken of unauthorised persons and children straying into work areas while construction activities are taking place.

The Principal Contractor shall be wholly responsible for the security of all areas in their possession.

All work areas are to be secured and safely isolated. Internal vulnerable and high-risk areas are to be isolated completely or by means of safety barriers and secured/ locked off areas. All works locations shall display appropriate warning signage to alert the facilities staff/ non-construction site staff to the dangers of entering a construction area.

## Welfare provision and temporary facilities for Station personnel

The Principal Contractor shall provide and maintain welfare facilities as laid down in the CDM Regulations 2015 Schedule 2. These facilities shall be provided from the start of construction and be retained at an appropriate level until all works are completed. The contractor will be provided with an appropriate area within the fire station car park to set up these facilities.

It is yet to be determined if the contractor will be allowed to make use of the hot water supply and food preparation facilities available in the 1.01 Kitchen as this is dependent on operational staff agreement, however the G.01 Community Room Kitchenette, G.10 Reception Kitchenette and 2.04 Kitchen are available for use if the contractor agrees to maintain a strict level of cleanliness at all times in these areas as well as establishing and maintaining designated contractor-only access routes to each facility. The conditions for using these facilities will be agreed upon at the Pre-Commencement Meeting with station management staff following the award of the contract.

The contractor may also be able to set up their site office in the G.10 Reception, G.11 Foyer and/or the G.01 Community Room (unless certain construction phases would mandate relocation) if these spaces meet the needs of the contractor. If the contractor is not satisfied with this proposal, they will be expected to provide a temporary site cabin located in the rear car park, the location of which to be agreed with station management along with the following provisions:

- Washing Facilities as required.

- Microwave, cups, hand soap and hand/pot towels etc.

Site Establishment – The Contractor shall inform the Project Manager of all requirements and liaise on a suitable area for the contractor’s use. The Principal Contractor will be solely responsible for this area and will ensure that the area is fully segregated and secured at all times.

The Principal Contractor shall provide in their Construction Phase Plan a marked-up drawing showing the extent and location of these facilities, including any changes to the location of facilities during each phase of the project.

The Principal Contractor shall include in all progress reports a statement for the Client confirming the ongoing suitability of the welfare provision.

## Overlap with the Client’s undertaking and other users of the site

No other construction activities are scheduled to take place concurrently within the proposed site boundary, but the contractor must be aware of periods of increased activity on site in relation to crews undertaking training, appliance maintenance, or turning out/returning from an incident.

## Client requirements

The Principal Contractor shall, as soon as possible after their appointment, provide the Principal Designer with contact details for all contractors and designers appointed to work on this contract. Contact details shall include their name, addresses, details of the principal contact, their telephone number and email address.

Normal working hours for the works are described within the tender and contract documentation. Weekend working arrangements must be agreed in advance with the Contract Administrator. Any request by the Contractor to the CA for weekend or late working is not guaranteed, except, where operational requirements and tasks that need to be undertaken out of the normal weekly working hours.

Parking of vehicles, other than that connected with delivery or collection of materials, will be restricted as car parking space is limited and must not impede appliance and crew member access/parking and staff training regimes. The number of parking spaces to be made available to the contractor will be determined at the pre-commencement meeting once the extent and location of welfare units has been agreed.

All plant and materials shall be kept secure when not in use and all ladders shall be removed and securely stored at the end of each working day and during break times. Under no circumstances should any vehicle, plant or materials/waste be parked or be allowed to accumulate in any area that would block an emergency vehicle route.

In undertaking the works, the Principal Contractor shall minimise any disruption to the operation and occupancy of the buildings from noise and dust resulting from the Works. The Contractor shall submit proposals to demonstrate these measures and agree them with the Project Manager.

The responsibility for ensuring compliance with legislation during the Works, to the defined site, for emergency routes, exits etc. rests with the Principal Contractor. Compliance with building regulations does not guarantee compliance with fire safety legislation.

Egress routes shall provide a safe passage for persons to travel from any point in a building to a place of safety and shall include the Client’s requirements for security, good order and safe custody. Please refer to Appendix A for general site safety standards/rules.

## Client permit-to-work systems

The Principal Contractor will be expected to instigate an appropriate permit to work system for the work areas under his control. Permits to work should cover all hot work, confined space entry, working at height, use of cranes and disconnections of services, working with gases and excavations. The permit system will remain the responsibility of the Principal Contractor to administer and control. The above list is not exhaustive but should act as a minimum standard.

This will include and extend to ALL Work at Height, working from ladders/MEWPS/scissor lifts and will include segregation of the work, safe working areas, exclusion zones and protection to the building occupants and the public.

Full method statements and risk assessments shall be issued to the Project Manager and/or Principal Designer a minimum of 7 calendar days in advance of the proposed works.

For the avoidance of doubt, the Principal Contractor is responsible for satisfying themselves that all building services are made safe prior to any work commencing. As required by the scope of the Contract Works and prior to the commencement of any works on site, the Contractor shall isolate, prove dead and lock off all necessary building services, and be satisfied that the services are safe to work on.

## Fire precautions and emergency procedures

The Principal Contractor shall ensure that all necessary fire precautions are implemented and that site personnel are aware of all fire drills, all escape and muster points and positions of all firefighting equipment in the event of a fire.

Contractor is to include fire evacuation route and muster points within the Construction Phase Health and Safety Plan.

In undertaking the works the Contractor shall note that The Regulatory Reform (Fire Safety) Order 2005 applies.

The Principal Contractor will be required to comply with The Joint Code of Practice published by the Building Employers Confederation, the Loss Prevention Council and the National Contractors Group entitled ‘Fire Prevention of Construction Sites’ and take action where the code demands.

HSG 168 Fire Safety in Construction Work gives guidance on control measures to reduce the incidence of fires on construction sites, the Principal Contractor should make themselves aware of its contents and recommendations and introduce procedures to comply especially in such matters as rubbish removal and the storage of flammable materials.

A fire safety plan shall be prepared to include procedures to reduce the risk of fire and for dealing with fires, explosions, and other major incidents.

The fire plan for the works shall address, but is not limited to, the following:

- Ensuring there is a responsible person in charge of fire safety who can assess fire risks, understand fire growth and spread, will prepare and update site evacuation plans as necessary and prepare a salvage operation plan.

- Include procedures to reduce the risk of fire, the spread of Fire and for dealing with fires, explosion and other major incidents.

- Establishing an agreed and suitable Fire Muster Point with the Client’s team.

- Procedures for communicating with neighbours on matters of fire safety/evacuation.

- Maintaining adequate means of escape for all personnel, building occupants, visitors to the site and the public using adjacent car parks and occupied buildings.

- Provision of temporary fire detection and fire extinguishers.

- Identification and maintaining clear access to existing fire hydrants.

- Advising all site staff/operatives on existing building fire alarm systems and procedures.

- Maintaining routes for emergency vehicles.

In addition, the Principal Contractor shall have a formulated emergency procedure for the site and these procedures shall include details of the nearest accident and emergency unit, local police details and a marked-up site plan for use by the emergency services.

There is a no smoking policy for the whole site. Refer also to Appendix A.

The nearest Accident & Emergency Facility is located at:

Bradford General Infirmary

Duckworth Lane,

Bradford,

West Yorkshire,

BD9 6RJ

Tel: 01274 542200

Website:	https://www.bradfordhospitals.nhs.uk/our-hospitals/bradford-royal-infirmary-bri/

# Project Health and Safety Hazards

The following issues have been included as they are deemed to be unusual and/or significant in respect to health and safety. The Principal Contractor is deemed to have visited the site and be fully acquainted with the nature, extent and restrictions relating to the site and its surroundings.

## Safety hazards

Asbestos- No ACMs are deemed to be present on site as the building was constructed between 2006-2007. The Principal Contractor will, however, instigate safe working procedures and be prepared to execute emergency containment protocols in the event that unknown ACMs are unintentionally discovered or disturbed during the course of the works.

The Principal Contractor shall ensure that all operatives and sub-contractors who are liable to disturb materials while carrying out their normal everyday work, or who may influence how work is carried out, have received third-party certified (UKATA or similar) asbestos awareness training. Certification of all contractors and sub-contractors to be evidenced upon appointment of contractor.

It is a requirement that any asbestos removal works required be carried out by a Licensed Asbestos Contractor.

In respect to the site and nature of the works elsewhere, The Principal Contractor should make their own assessment and wear PPE as appropriate. At the time of issue of this PCI document there are no other known significant residual design related safety hazards for the project which a competent and experienced construction contractor would not be expected to identify and be able to manage as part of the construction safety management process.

Any unforeseen eventualities which come to light during the execution of the works and which may result in significant design changes upon which associated safety related hazards must be notified immediately to the Project Manager. Any re-design to be accepted and agreed by all parties prior to the execution of said works.

## Boundaries and general access, including temporary access

Strict demarcation and segregation of the work site locations must be put in place and maintained at all times in order that members of the public or crew members visiting the station are not exposed to construction-based hazards.

The Principal Contractor shall be wholly responsible for the security of all areas in their possession and provide all practicable measures to prevent unauthorised access to all works areas.

All works areas are to be secured and safely isolated. All works locations shall also display appropriate warning signage to alert the facility staff/public to the dangers of entering a construction area.

The contractor must also consider the safe progression to and from the work site locations and these routes must be well maintained and free from hazards which may cause risk of injury to facility staff.

## Restrictions on deliveries, vehicular traffic or waste collection or storage

The access route shall be agreed with the Project Manager. In this regard the Principal Contractor shall prepare a traffic management plan for the project for review by the Project Manager and/or Principal Designer.

General parking shall be restricted to the areas, if any, designated for contractor use.

Removal of Waste

Debris from the works areas will be removed on a daily basis. The works areas must be kept clean and tidy at all times.

## Adjacent Land Uses

The project works will take place at locations indicated within section 2.1. All surrounding areas to the works locations will remain in use periodically by crew members and the general public (outside of the perimeter of the station) for the duration of the works.

## Existing storage of hazardous materials

None noted.

## Location of Existing Services

The following services have been identified as being present on site or adjacent to the site:

- Electrical.

- Water.

- Drainage SW & Foul.

- Communications & Data.

- Mechanical extraction and ventilation.

The Principal Contractor shall on appointment take responsibility for ascertaining the exact location, nature and status of each service and isolation point prior to commencing work, i.e. implement visual inspection, testing, CAT scan, radar survey, hand dig trial pits etc. The Principal Contractor shall not rely on the information provided but shall independently contact and liaise with the Utility Authorities and the Client Estates/Facilities Management team prior to commencing any works.

All services, including those indicated as redundant, shall be treated as being live until proven otherwise.

The Contractor or their employees shall not enter any substation, switch room or similar area without the relevant clearance and authority of the Project Manager. All electrical work and work involving the use of electrical tools and equipment shall be carried out in accordance with the Electricity at Work Regulations 1989. All portable electric tools shall be at 110V and have been recently PAT tested.

## Ground conditions

No hazards identified by Client.

## Information about existing structures

There are no structural concerns with the existing building.

## Hazard considerations

Working at Height

The project works will take place above ground level within rooms for some aspects for the work, namely erection of new walls up to ceiling level, new suspended ceiling installations, high level decoration and various M&E items that pass through and terminate above the ceiling.

The Principal Contractor will be responsible for the design and management of any temporary works and/or access via ladders, towers or stepladders.

The contractor must protect all persons at ground or floor level from the danger of falling debris or falling construction materials (safe working / exclusion zones) and protection from the fall of height of an operative.

Where required, appropriate certification and inspection must be regularly carried out on all scaffolds and temporary works. All means of access to scaffolding should be suitable and fit for purpose.

Live Services

The Works may involve working in close proximity to live services. The contractor shall take due consideration of this and develop and implement safe working practices reflected within their Risk Assessments and Method Statement.

Construction Related Dust & Silica Dust Particles

Various construction related activities may generate dust during the project. All processes which generate dust, including possible silica dust particles, must be point of works risked assessed and appropriate control measures put in place to reduces dust generation during the works. This includes but is not limited to drilling and coring etc. Control measures considered for site operatives must include appropriate PPE.

Furthermore, protection of the general public must also be considered. All operations which may generate dust, including silica dust, must be risked further assessed and adequate control measures put in place to protect the general public. This should include consideration for dust suppression by water/spray, local P3 vacuum fitted to machinery and appropriate forced or mechanical ventilation.

Noise and Vibration

Noise and Vibration consideration must be given to the suppression of noise levels. If requested to stop works, this must take place immediately and reported to the Contract Administrator and Project Manager.

Desk top vibration assessments (using HSE ready reckoner) must be carried out prior to the works commencing and continually monitored throughout the project (if deemed necessary) to ensure that sufficient controls are in place to reduce exposure to vibration from hand tools used by site staff.

## Health hazards

Contaminated land, including results of surveys

No contaminated land has been identified by the Client. Not applicable to scope of works planned.

Existing structures containing hazardous materials

No structures containing hazardous materials have been identified by the Client.

Health risks arising from client’s activities

No risks from Client activities have been identified by the Client.

Hazard management

The Principal Contractor shall independently undertake their own risk assessments in accordance with statutory requirements, and all such risk assessments shall be incorporated within the Construction Phase Plan.

It has been assumed by the project team that standard construction methods shall be adequate for the project.

Reference should also be made to Appendix C.

Significant Hazards

Refer to Appendix D for summary table of significant construction phase hazards.

# The Construction Phase Plan

## Generally

The Construction Phase Plan (CPP) is to be developed by the Principal Contractor throughout the life of the project and should be a specific document which sets out the arrangements for securing Health & Safety of all those carrying out the works and all others who might be affected by it.

When developing the CPP, the Principal Contractor should identify the hazards and assess the risks including but not necessarily limited to those identified in this document. The Principal Contractor should also identify their organisations’ arrangements for managing Health & Safety – see Appendix B for detailed contents.

The CPP needs to be kept up to date and modified in the light of changing circumstances. It is vital that such changes are notified to those working on site that may be affected.

The Health and Safety File:

# The Health and Safety File

Contents

The Principal Contractor shall issue this information and documentation to the Principal Designer not less than ten working days prior to Practical Completion. Receipt of all required information and documentation for the health and safety file is a condition precedent to practical completion being certified.

In order to be useful, the Health and Safety File should be as succinct as possible.  The provision of huge volumes of information is not recommended, because it will not be read.  However, there are core pieces of information, which should be supplied. This section provides guidance on what constitutes this core information.

It is the responsibility of the Principal Contractor to provide sufficient information for the Health and Safety File as required by the Construction (Design and Management) Regulations 2015.

Key Health and Safety File shall include information to be identified relevant to the health and safety of any future construction, maintenance, and repair work or cleaning, including, but not limited to, the following:

| Section | Title | Description | Responsibility |
| --- | --- | --- | --- |
| 1 | Description of the Work | As included in this document. | Principal Designer |
| 2 | Residual Hazards | Residual hazards which remain and how they have been dealt with in the form of an updated Design Risk Register Surveys or other information concerning asbestos; contaminated land; water bearing strata; buried services etc). | Principal Designer, Client, Principal Contractor |
| 3 | Key structural principles | Key Structural Principles (for example, bracing, sources of substantial stored energy – including pre‐ or post‐tensioned members) and safe working loads for floors and roofs, particularly where these may preclude placing scaffolding or heavy machinery. | Structural Engineer, Specialist design contractors, via Principal Contractor |
| 4 | Hazardous Materials | Hazardous Materials used (for example lead paint; pesticides; special coatings which should not be burnt off etc.). | Designers & Principal Contractor |
| 5 | Removal or dismantling information | Information regarding the removal or dismantling of installed plant or equipment. | Designers & Specialist design contractors via Principal Contractor |
| 6 | Cleaning and maintenance equipment | Health and safety information about equipment provided for cleaning or maintaining the structure. | Specialist contractors via Principal Contractor |
| 7 | Significant services | The nature, location and markings of significant services, including underground cables; gas supply equipment; fire‐fighting services etc. | M&E Designers, Specialist contractors via Principal Contractor |
| 8 | As-Built Information and Drawings |  |  |
| 8.1 | SOTER Asset Information | Populate SOTER Asset Data Import Excel spreadsheet template for each newly installed asset to allow WYFRS to update their central asset register. | Principal Contractor via Client |
| 8.2 | As-built construction drawings (Architectural) | (1:50) | Principal Contractor via Architect |
| 8.3 | As-built construction drawings (Structural) | (1:50 plus details) | Principal Contractor via Structural Engineer |
| 8.4 | As-built drawings (Specialist suppliers & contractors) | As built drawings from any specialist contractor or supplier for plant and equipment they have supplied or installed. | Specialist contractors and suppliers via Principal Contractor |
| 8.5 | Construction maintenance manuals | Maintenance manuals from any specialist contractor or supplier, outlining the operating and maintenance procedures for the structure or any plant and equipment they have supplied or installed. | Specialist contractors and suppliers via Principal Contractor & M&E Designers |
| 8.6 | As-built Mechanical services drawings | (1:50 plus details and schematics) | Specialist contractors and suppliers via Principal Contractor & M&E Designers |
| 8.7 | Mechanical services maintenance manuals | A mechanical services maintenance manual detailing maintenance procedures and requirements for any mechanical plant, fittings or equipment. | Specialist contractors and suppliers via Principal Contractor |
| 8.8 | As-built Electrical services drawings | (1:50 plus details and schematics) | Specialist contractors and suppliers via Principal Contractor & M&E Designers |
| 8.9 | Electrical services maintenance manuals | An electrical services maintenance manual including as built drawings detailing maintenance procedures and requirements for the electrical installation and any electrical plant, fittings or equipment. | Specialist contractors and suppliers via Principal Contractor |
| 8.10 | Architectural, Mechanical and Electrical certificates | A copy of mechanical and electrical test certificates for any new or altered services; - Electrical completion certificate. - Electrical emergency lighting completion certificate. - Building regulations completion certificate. - Building regulations fire safety completion certificate. - Gas soundness test certificate. - Fire alarm completion certificate. - Extract systems flow rate testing/completion certificate. - Mains water chlorination certificate. | Specialist contractors and suppliers via Principal Contractor |
| 8.11 | Guarantees and warranties |  | Specialist contractors and suppliers via Principal Contractor |

Health and Safety File information shall be provided by the Principal Contractor independently/separately to completion handover documentation and must be made available and issued to the Principal Designer a minimum of two weeks prior to Practical Completion.

The Health and Safety File shall be prepared in parallel with and, when appropriate, co-ordinated with the Principal Contractor’s completion handover documentation.

Please refer to the detailed requirements of the Contract Preliminaries.

Compilation and presentation of the documentation for and final arrangement & presentation of the health & safety file

The Principal Contractor shall be responsible for ensuring, where relevant, that the following is made available in a timely manner to the Principal Designer for inclusion in / amendment to the health & safety file:

- the obtaining of relevant health and safety information and documentation from all designers;

- the obtaining of relevant health and safety information and documentation from all contractors;

- the updating of any existing health & safety files for the building;

- the incorporation of the relevant health and safety information and documentation for the project into the health & safety file;

- the provision of 1 fully-indexed and editable digital copy of the health & safety file, and O&M Manuals on CD-ROM, in (“.pdf”) format.

- in addition, the provision of all drawings in indexed AutoCAD (“.dwg”) and indexed PDF format (unless otherwise agreed).

- Completed Asset Register as per SOTER mass upload spreadsheet.

- The Mechanical & Electrical Operating & Maintenance Manuals

The Principal Contractor shall test and commission fully all new installations, systems and equipment on completion and shall produce record drawings along with the operations and maintenance (O&M) manuals.

All M&E Operating & Maintenance Manuals must be complete and signed-off by the relevant M&E design consultant prior to submission to the Principal Designer for review and issue to the Client.

The Principal Contractor is directed to ensure the Mechanical and Electrical O & M Manuals are available at Practical Completion.

Appendices

Anticipated Health and Safety Standards / Client Rules

The Principal Contractor shall comply with all statutory health, safety and emergency procedures and this must be acknowledged and referred to in the Construction Phase Plan.

        - Only trained, certified and competent personnel shall be permitted to operate mechanical plant, tools and equipment. Copies of all certificates are to be available on site.

        - Disposal of waste material by burning on site is NOT permitted.

        - Operatives to wear hard hats, safety boots and high visibility vests at all times, and other PPE, as and when required.

        - All personnel (operatives and visitors) are to sign in and out daily in the site register.

        - Eating and drinking shall only be permitted in designated welfare facilities.

        - All operatives (including staff and visitors) shall receive appropriate safety induction from the Principal Contractor.

        - Allow only authorised people onto site.

        - Identify emergency procedures and contact numbers for all contractors, utilities and services and display this information clearly on site and within the Construction Phase Plan.

        - Ensure that all noisy and dusty work is carried out during reasonable hours and that any inconvenience to adjacent neighbouring properties is kept to a minimum. Take all necessary measures to keep noise and dust to a minimum.

        - No personnel, visitors etc. shall be allowed on site under the influence of alcohol or drugs nor allowed to consume these whilst on site.

        - Verify the existence and status of hidden, buried, overhead services prior to commencing works. Liaise with the appropriate authority.

        - The use of foul or abusive language or gestures shall not be tolerated.

        - Racist or sexist behaviour or material shall not be tolerated on site.

        - Fire and emergency procedures are to be set up in conjunction with the Emergency Services. Ensure that all necessary fire precautions are observed and that site personnel are aware of all fire drills, all escape and rally points and positions of all firefighting equipment in the event of a fire.

Construction Phase Plan Criteria

INFORMATION REQUIRED

Section 1 - General Project Information

1.1 Description of the project;

1.2 Programme details;

1.3 Details of Client, PD, Designers, PC and other consultants;

1.4 The management structure and responsibilities

Section 2 – Management Arrangements

2.1 Project health and safety aims and goals

2.2 The site rules

2.3 Arrangements to ensure cooperation between project team members and coordination of their work, e.g. regular site meetings

2.4 Arrangements for involving workers - consultation with workforce

2.5 Site induction / Site Training

2.6 Welfare facilities

2.7 Fire and emergency procedures (Fire Plan)

2.8 Security arrangements

2.9 First aid arrangements

2.10 Accident/incident reporting and investigating, RIDDOR

2.11 Monitor and review health and safety performance

2.12 Site plan/traffic management plan

2.13 Significant safety risks – reference Schedule 3, CDM 2015

2.14 Health and Safety File Information

Construction (Design & Management) Regulations 2015 – L153. Appendix 3

A Construction Phase Plan is a document that must record the:

a)	health and safety arrangements for the construction phase;

b)	site rules; and

c)	where relevant, specific measures concerning work that falls within one or more of the categories listed in Schedule 3 (below).

The plan must record the arrangements for managing the significant health and safety risks associated with the construction phase of a project. It is the basis for communicating these arrangements to all those involved in the construction phase, so it should be easy to understand and as simple as possible.

In considering what information is included, the emphasis is that it:

a) 	is relevant to the project;

b) 	has sufficient detail to clearly set out the arrangements, site rules and special measures needed to manage the construction phase; but

c) 	is still proportionate to the scale and complexity of the project and the risks involved.

The plan should not include documents that get in the way of a clear understanding of what is needed to manage the construction phase, such as generic risk assessments, records of how decisions were reached or detailed safety method statements.

The following list of topics should be considered when drawing up the plan:

a) 	a description of the project such as key dates and details of key members of the project team;

b) 	the management of the work including:

(i) 	the health and safety aims for the project;

(ii) 	the site rules;

(iii) 	arrangements to ensure cooperation between project team members and coordination of their work, e.g. regular site meetings;

(iv) 	arrangements for involving workers;

(v) 	site induction;

(vi) 	welfare facilities; and

(vii) 	fire and emergency procedures;

c)	 the control of any of the specific site risks listed in Schedule 3* where they are relevant to the work involved

If any of the above items are not submitted with the Construction Phase Plan, the Plan is not deemed to be compliant with CDM 2015 Regulation 12 (1) and the Client notified accordingly.

Design Risk Assessment

Refer to Section 5. Drawings & Specifications – Architectural and MEP for each respective designer’s risk register.

Work Involving Particular Risks

Schedule 3 of the Construction (Design and Management) Regulations 2015 lists significant hazards that require specific measures to be taken by the Principal Contractor. See Table 1.  Further project specific hazards are listed in Table 2.

TABLE 1

|  | Activity | Comment / Note |
| --- | --- | --- |
| 1 | Work which puts workers at risk of burial under earthfalls, engulfment in swampland or falling from a height, where the risk is particularly aggravated by the nature of the work or processes used or by the environment at the place of work or site. | Not significant – Working from height |
| 2 | Work which puts workers at risk from chemical or biological substances constituting a particular danger to the health or safety of workers or involving a legal requirement for health monitoring. | Not significant – ‘Red Zone’ contaminant areas |
| 3 | Work with ionizing radiation requiring the designation of controlled or supervised areas under regulation 16 of the Ionising Radiations Regulations 1999(a). | Not applicable |
| 4 | Work near high voltage power lines. | Not applicable |
| 5 | Work exposing workers to the risk of drowning. | Not applicable |
| 6 | Work on wells, underground earthworks and tunnels. | Not applicable |
| 7 | Work carried out by divers having a system of air supply. | Not applicable |
| 8 | Work carried out by workers in caissons with a compressed air atmosphere. | Not applicable |
| 9 | Work involving the use of explosives. | Not applicable |
| 10 | Work involving the assembly or dismantling of heavy prefabricated components. | Not significant |

Table 2

|  | Activity | Comment / Note |
| --- | --- | --- |
| 1. | Work at Height | All work at height must be safely managed by the principal contractor and must fully conform with the Work at Height Regulations 2005. Permit to Work required for all use of ladders on site |
| 2 | Live services, electrical works & Disruption of services in occupied building | The location, nature and status of each service and isolation point must be ascertained prior to commencing work. All services shall be treated as being live until proven otherwise. |
| 3 | Isolation and segregation of works Interface and impact on the public and building users/ occupants | Strict demarcation and security/segregation of the work site location or working area must be put in place using appropriate fencing, screening or barriers. Signage warning of construction hazards must also be put in place. All construction works and vehicle plant operating and working areas must be cordoned off and segregated from building users and public access with secure lockfast Heras fencing or barriers providing safe working zones from the operations and oversailing plant. |
| 4 | Disturbance to hazardous materials and fibre release Asbestos or ACM’s; silica dust. | Where an asbestos-containing material (ACM) has been identified on site, the Principal Contractor must assess if there is a risk that the ACM will be disturbed or otherwise affected by any of the project works. If there is a risk that the project works will disturb or cause a release of asbestos fibre the ACM must be removed as part of the project works package by a licensed specialist contractor. Other Materials – strictly follow COSHH data – PPE & dust extraction as required. |
| 5 | Use of Cleaning Materials, Specialised Coatings, Adhesives, Sealants, Solvents and Fire Stopping materials. | Follow specific COSHH Data with regard to exposure and protection - ventilation of workspaces and PPE. |
| 6 | Manual Handling Materials | All manual handling of materials plant and equipment must be safely managed by the principal contractor and must fully conform with the Manual Handling Operations Regulations 1992. Weather conditions must be fully considered. |
| 7 | Work in occupied areas | Works to be carried out in an operational wholetime fire station. Strict demarcation of work site locations is always required as is the establishment and maintenance of designated contractor access routes throughout the building for contractor personnel and material movements. |

West Yorkshire Fire & Rescue Service

Oakroyd Hall, Birkenshaw, BD11 2DY

T | +44 7760 992135   E | josef.turner@westyorksfire.gov.uk

www.westyorksfire.gov.uk